3GPP TSG-RAN WG3 Meeting #130	R3-258780

Dallas, US, 17 - 21 November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 1604 | Rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of missing LTM Indicator for inter-CU LTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia, LG Electronics, NTT DoCoMo. Ofinno, Jio Platforms, CMCC, Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | A mandatory LTM Indicator IE in LTM Information Setup IE was introduced in the F1AP UE Context Request procedure and UE Context Modification procedure in Rel-18 for intra-CU LTM to explicitly indicate the gNB-DU about the LTM preparation. As the LTM procedures in F1AP introduced for intra-CU LTM in Rel-18 are reused for inter-CU LTM. In principle, the same indicator should be introduced in the Handover Request procedure in XnAP for inter-CU LTM to have a harmonized design with F1AP. However, the LTM Indicator IE is still missing in the HANDOVER REQUEST message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following change is made to the XnAP for inter-CU LTM. - Add LTM Indicator IE in the HANDOVER REQUEST message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment exits between F1AP and XnAP for inter-CU LTM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1, 9.1.1.1, 9.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: editoricals and cover page update. |  |  |  |  |  |  |  |  |

<< Start of Changes >>

### 8.2.1	Handover Preparation

#### 8.2.1.1	General

This procedure is used to establish necessary resources in an NG-RAN node for an incoming handover. If the procedure concerns a conditional handover, parallel transactions are allowed. Possible parallel requests are identified by the target cell ID when the source UE AP IDs are the same.

If the procedure concerns an LTM, parallel transactions are allowed only when using the same source UE AP ID. Possible parallel requests are identified by the target cell ID.

The procedure uses UE-associated signalling.

#### 8.2.1.2	Successful Operation

Figure 8.2.1.2-1: Handover Preparation, successful operation

The source NG-RAN node initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node. When the source NG-RAN node sends the HANDOVER REQUEST message, it shall start the timer TXnRELOCprep.

<skip unchanged part>

If the LTM Handover Information Request IE included in the HANDOVER REQUEST message and the LTM Indicator IE is set to "true", the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the LTM information for the UE in the target NG-RAN node and include the LTM Handover Information Request Acknowledge IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Reference Configuration IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, take it into account for generating the LTM configuration.

If the Request for CSI-RS Resource Configuration for L1 Measurements IE set to “true” is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the CSI-RS Resource Configuration for L1 Measurements IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the Request for CSI-RS Resource Configuration for Early CSI Acquisition IE set to “true” is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the CSI-RS Resource Configuration for Early CSI Acquisition IE in the HANDOVER REQUEST ACKNOWLEDGE message.

If the CSI Resource Configuration IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to generate the LTM CSI reporting configuration included in the LTM Candidate Configuration IE for the requested candidate cell.

If the LTM Configuration ID Mapping List IE is contained in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Proposed LTM No Security Change ID List IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, include the LTM No Security Change ID IE in the HANDOVER REQUEST ACKNOWLEDGE message for the accepted candidate cell, and act as specified in TS 38.300 [9].

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the LTM Handover Information Request Acknowledge IE included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, consider that the LTM candidate configuration is a complete candidate configuration.

If the Target NG-RAN node UE XnAP ID IE is contained in the LTM Handover Information Request IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, consider that the source NG-RAN node requests LTM preparation for an already established UE association.

If the Early Sync Information Request IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, consider that the source NG-RAN node has requested the early synchronization information for the UE in the target NG-RAN node.

If the Early Sync Information Response IE is contained in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, store it and use it as defined in TS 38.300 [9].

If the LTM CFRA Resource Information IE is contained in the LTM Handover Information Request Acknowledge IE included in the HANDOVER REQUEST ACKNOWLEDGE message, the source NG-RAN node shall, if supported, use it for the LTM cell switch command as specified in TS 38.321 [35].

If the Network Slice Area Scope of MDT IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, use it to derive the MDT area scope for MDT measurement collection. Upon reception of the Network Slice Area Scope of MDT IE, the target NG-RAN node shall consider that the area scope for MDT measurement collection is defined only by the Network Slice Area Scope of MDT IE and Area Scope of MDT IE.

If the Geographical Area IE is included in the MDT Configuration-NR IE included in the HANDOVER REQUEST message, and the Geographical Area IE contains the MDT PLMN List IE, the target NG-RAN node shall, if supported, apply the geographical area scope only for UEs served in the listed PLMNs.

<< Next Change >>

#### 9.1.1.1	HANDOVER REQUEST

This message is sent by the source NG-RAN node to the target NG-RAN node to request the preparation of resources for a handover.

Direction: source NG-RAN node  target NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Source NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the source NG-RAN node | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | reject |
| Target Cell Global ID | M |  | 9.2.3.25 | Includes either an E-UTRA CGI or an NR CGI | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Handover Information Request | O |  |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …) |  | – |  |
| >Proposed LTM No Security Change ID List | O |  | 9.2.3.231 |  | – |  |
| >Target NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the target NG-RAN node. | – |  |
| >Reference Configuration | O |  | OCTET STRING | Includes the ltm-ReferenceConfiguration as defined in TS 38.331 [10]. | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.3.221 |  | – |  |
| >CSI Resource Configuration | O |  | 9.2.3.223 |  | – |  |
| >Request for CSI-RS Resource Configuration for L1 Measurements | O |  | ENUMERATED (true, …) |  | – |  |
| >Request for CSI-RS Resource Configuration for Early CSI Acquisition | O |  | ENUMERATED (true, …) |  | – |  |
| Early Sync Information Request | O |  | 9.2.3.217 |  | YES | ignore |

<< Next Change >>

LTMHandoverInformationRequest ::= SEQUENCE {

lTMIndicator							LTMIndicator,

proposedLTM-NoSecurityChangeID-List		LTM-NoSecurityChangeID-List		OPTIONAL,				-- This IE may need to be refined.

targetNG-RANnodeUEXnAPID				NG-RANnodeUEXnAPID				OPTIONAL,

referenceConfiguration					ReferenceConfiguration			OPTIONAL,

lTMConfigurationIDMappingList			LTMConfigurationIDMappingList	OPTIONAL,

cSIResourceConfiguration				CSIResourceConfiguration		OPTIONAL,

requestForCSI-RSResourceConfigForLayer1Measurements		RequestForCSI-RSResourceConfigForLayer1Measurements		OPTIONAL,

requestForCSI-RSResourceConfigForEarlyCSIAcquisition	RequestForCSI-RSResourceConfigForEarlyCSIAcquisition		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { LTMHandoverInformationRequest-ExtIEs} } OPTIONAL,

...

}

<< Next Change >>

LTM-SCG-SecurityConfig-Info-List ::= SEQUENCE (SIZE(1..maxnoofSCGSecurityConfigurations)) OF LTM-SCG-SecurityConfig-Info-Item

LTM-SCG-SecurityConfig-Info-Item ::= SEQUENCE {

s-ng-RANnode-SecurityKey			S-NG-RANnode-SecurityKey,

sk-counter							SK-COUNTER,

iE-Extensions		ProtocolExtensionContainer { { LTM-SCG-SecurityConfig-Info-Item-ExtIEs} } OPTIONAL,

...

}

LTM-SCG-SecurityConfig-Info-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LTMIndicator ::= ENUMERATED {true, ...}

<< End of Changes >>