3GPP TSG-RAN WG3 Meeting #130                                                                R3-258816

Dallas, US, 17 – 21, Nov, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.320 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for propagation of MDT Configuration in stage2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, China Unicom, China Telecom, CMCC, Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core, TEI18 |  |  |  |  |  | Date: |  |  | 2025-11-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | While MDT context handling during Secondary Node Addition is specified in Stage 3, it is absent from Stage 2. The current Stage 2 specification covers only handover and UE context retrieval procedures. This discrepancy between Stage 2 and Stage 3 requires correction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Stage 2 description is added to specify propagation of the MDT configuration during the Secondary Node Addition procedure. Impact Analysis: Impact assessment towards the previous version of the specification (same release): The CR has isolated impact. The CR is backwards compatible from a protocol point of view. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification with risk of inter-operability issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### -------------------------------------------------Start of Change----------------------------------------------

#### 5.1.2.3	MDT context handling during handover and UE context retrieval

The measurements configured in the UE for Immediate MDT should fully comply with the transferring and reconfiguration principles for the current measurements configured in the UE for RRM purpose during handover (including conformance with Rel-8 and Rel-9).

The target node releases the measurements configured in the UE for immediate MDT which are no longer needed based on any MDT trace configuration it receives or does not receive.

In addition, MDT configuration handling during handover and UE context retrieval depends on MDT initiation from OAM defined in clause 5.1.3:

-	The MDT configuration configured by management based trace function will not propagate during handover.

-	For LTE, the MDT configuration received by signalling based trace messages for a specific UE will propagate during intra-PLMN handover, and may propagate during inter-PLMN handover if the Signalling Based MDT PLMN List is available and includes the target PLMN. This behaviour applies also for MDT configuration that includes area scope, regardless of whether the source or target cell is part of the configured area scope. This behaviour applies also for Xn inter-RAT handover.

-	For UMTS, the MDT configuration received by signalling based trace messages for a specific UE will continue during intra-PLMN handover, and may continue during inter-PLMN handover if the Signalling Based MDT PLMN List is available and includes the target PLMN, except for the case of SRNS relocation.

-	For NR, the MDT configuration received by signalling based trace messages for a specific UE will propagate during Secondary Node Addition, intra-PLMN handover and intra-PLMN UE context retrieval, and may propagate during inter-PLMN handover or inter-PLMN UE context retrieval if the Signalling Based MDT PLMN List is available and includes the target PLMN. This behaviour applies also for MDT configuration that includes area scope, regardless of whether the source or target cell is part of the configured area scope. This behaviour applies also for Xn inter-RAT handover.

NOTE:	In the case of SRNS relocation, MDT may be reactivated by the Core Network following a successful relocation.

#### -------------------------------------------------End of Change----------------------------------------------