3GPP TSG-RAN WG3 Meeting #130	R3-258867

Dallas, US, 17-21 Nov, 2025

Title: 	(TP for pCR) RAN-CN procedures and signaling for ISAC

Source: 	Huawei, CATT, Xiaomi, Nokia, Nokia Shanghai Bell, LG Electronics, CMCC, Ericsson, ZTE, NEC, Samsung, China Telecom

Agenda item:	13.3

Document Type:	other

1. Introduction

This contribution provides TP for ISAC pCR according to the agreements made at CB: # 17_ISAC.

Annex – TP to ISAC pCR against R3-258088

CHANGES START

8	RAN-CN procedures and signalling

Editor’s note 1: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

Editor’s Note 2: the following may need further refinement.

The Nx-AP protocol supports the Sensing Information Transfer function.

The following signaling procedures are used for Sensing Information Transfer function:

-	Sensing Initiation: a Class 1 procedure (including Sensing Request, Sensing Response and Sensing Failure);

-	SF-initiated Sensing Abort;

- 	gNB-initiated Sensing Failure indication: a Class 2 procedure.

Editor’s Note 3: FFS on other functions and procedures.

Editor’s Note 4: FFS whether SF-initiated Sensing Abort procedure is class 1 or class 2 procedure.

8.x	Sensing Initiation

Editor’s Note 1: the following may need further refinement.

The SF initiation procedure is illustrated in Figure 8.x.1.

Figure 8.x-1: Message flow for Sensing Initiation

- The SF sends Sensing Request message to the gNB, including the sensing measurement ID, the target sensing area, and the reporting mode (e.g., one time, periodic).

Editor’s Note 2: FFS on other information which can be included in Sensing Request message.

Editor’s Note 3: FFS on the definition of sensing measurement ID and the target sensing area.

2.	The gNB sends Sensing Response message to the SF.

3.	If requested in Step 1, the gNB sends Sensing Report to the SF.

Editor’s Note 4: FFS whether Sensing Report is a signalling procedure.

Editor’s Note 5: FFS whether the sensing results can be included in the Sensing Response message.

8.y	Sensing Abort

Editor’s Note 1: the following may need further refinement.

The SF initiated sensing abort procedure is illustrated in Figure 8.y.1.

Figure 8.y-1: Message flow for Sensing Abort

1.	The SF sends Sensing Abort message to the gNB, which stops the indicated sensing measurement and releases any related resources.

Editor’s Note 2: FFS whether it is a class 1 or class 2 procedure.

8.z	Sensing Failure Indication

The gNB initiated sensing failure indication procedure is illustrated in Figure 8.z-1.

Figure 8.z-1: Message flow for Sensing Failure Indication

1.	The gNB sends Sensing Failure Indication message to the SF to notify that an ongoing sensing measurement can no longer be performed.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>