3GPP TSG-RAN WG3 Meeting #130	R3-258821

Dallas, US, 17-21 Nov, 2025

Title: 	(TP for pCR) RAN-CN procedures and signaling for ISAC

Source: 	Huawei

Agenda item:	13.3

Document Type:	other

1. Introduction

This contribution provides TP for ISAC pCR according to the agreements made at CB: # 17_ISAC.

Annex – TP to ISAC pCR against R3-258088

CHANGES START

8	RAN-CN procedures and signalling

Editor’s note 1: This section is to capture the study outcome of procedures and signaling aspects between RAN and CN for gNB-based monostatic sensing.

Editor’s Note 2: the following may need further refinement.

The Nx-AP protocol supports the Sensing Information Transfer function.

The following procedures are used for Sensing Information Transfer function:

-	Sensing Initiation: a Class 1 procedure (including Sensing Request, Sensing Response and Sensing Failure);

-	SF-initiated Sensing Abort;

- 	gNB-initiated Sensing Failure indication: a Class 2 procedure.

Editor’s Note 3: FFS on other functions and procedures.

Editor’s Note 4: FFS whether SF-initiated Sensing Stop procedure is class 1 or class 2.

8.x	Sensing Initiation

Editor’s Note 1: the following may need further refinement.

The SF initiation procedure is illustrated in Figure 8.x.1.

Figure 8.x-1: Message flow for Sensing Initiation

- The SF sends sensing request to the gNB, including sensing request ID, target sensing area, report mode (e.g., one time, periodic).

Editor’s Note 2: FFS on other information.

Editor’s Note 3: FFS on the definition of sensing area.

2.	The gNB sends sensing response to the SF.

3.	If requested in Step 1, the gNB sends sensing report to the SF.

Editor’s Note 4: FFS whether the Sensing Report message is a signalling procedure.

Editor’s Note 5: FFS whether the sensing report is included in the Sensing Response message.

8.y	Sensing Abort

Editor’s Note 1: the following may need further refinement.

The SF initiated sensing abort procedure is illustrated in Figure 8.y.1.

Figure 8.y-1: Message flow for Sensing Abort

1.	The SF sends the Sensing Abort message to the gNB, which stops the sensing measurement and reporting and releases any resources previously allocated for the same sensing measurement.

Editor’s Note 2: FFS on class 1 or class 2 message.

8.z	Sensing Failure Indication

The gNB initiated sensing failure indication procedure is illustrated in Figure 8.z-1.

Figure 8.z-1: Message flow for sensing failure indication

1.	The gNB sends the Sensing Failure Indication message to the SF.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>