3GPP TSG RAN WG3 Meeting #131	             					R3-260473

Gothenburg, Sweden, Feb 9th –13th, 2026

Agenda Item:	10.3.2

Source:	AT&T

Title:	Discussion on RAN-CN Control Plane Options for 6G

Document for:	Discussion/Decision

# Introduction

This contribution includes description of the principal RAN-CN control plane interface options (focus on main characteristics) [1]. As part of the 6G system study, RAN3 is evaluating protocol stack options for the future RAN-CN control plane interface. The choice represents a foundational architectural decision that will impact flexibility, performance, and evolvability of 6G networks. This contribution analyzes the candidate options currently under consideration: Point-to-Point (P2P) interfaces based on SCTP or QUIC, and Service-Based Interfaces (SBI) using HTTP/2 or HTTP/3 over TCP or QUIC respectively.

# Discussion on RAN-CN Control Plane Options for 6G

## Point-to-Point (P2P) Interface Options

### Option 1a: SCTP-based P2P Interface

This approach would evolve the current 5G NG-AP model by continuing to use SCTP as the transport protocol for elementary procedures between the RAN node and CN entity. The primary advantage lies in continuity and expertise, as this model builds on decades of 3GPP protocol design, debugging, and optimization, offering proven reliability and deterministic performance that is critical for control signaling. However, challenges exist regarding flexibility, as SCTP-based interfaces may be less adaptable to cloud-native, service-based deployments compared to HTTP-based methods. There is also concern about potential stagnation, as this conservative approach might miss an opportunity to align with broader IT trends such as inherent encryption and improved multiplexing capabilities that newer protocols offer.

### Option 1b: QUIC-based P2P Interface

This progressive alternative proposes replacing SCTP with QUIC as the transport layer for elementary procedures. QUIC offers several modern transport advantages, including a low-latency handshake, improved congestion control algorithms, and mandatory encryption. Its native stream multiplexing effectively eliminates head-of-line blocking without requiring separate SCTP streams, and its use of UDP can be more firewall-friendly than SCTP. The significant challenge is the relative immaturity of QUIC within telecom signaling planes, with limited operational experience in core network contexts. Adopting QUIC would also necessitate defining a new 3GPP application protocol on top of it, representing a non-trivial specification effort.

All NG Application Part (NGAP) elementary procedures, including those for UE Context Management, Handover, PDU Session Resource Management, Paging, and NAS Transport, which are currently defined and optimized around SCTP’s service primitives, stream management, and reliability model. Migrating to QUIC would necessitate a comprehensive re-specification of these procedures to align with QUIC’s connection and stream semantics. This is not merely a transport-layer swap; it requires re-evaluating message sequencing, error recovery mechanisms, and state machine interactions in the context of QUIC's features and limitations. The validation burden for ensuring that performance and reliability for critical procedures like handover match or exceed the SCTP baseline would be substantial.

SCTP implementations in both RAN and CN have been heavily optimized over multiple generations for control plane traffic patterns, including small, bursty signaling messages and occasional larger packets. QUIC introduces different congestion control algorithms, loss recovery mechanisms, and head-of-line blocking avoidance at the stream level. While beneficial in the long term, this would require an extensive re-tuning phase to ensure that QUIC’s behavior does not adversely impact the latency or reliability of time-sensitive procedures such as handover cancel or UE context release. Network operators’ existing performance monitoring, troubleshooting, and capacity planning tools, which are built around SCTP metrics, would also need significant adaptation.

During a transition period, networks would need to support dual-stack operation (SCTP and QUIC) to accommodate legacy RAN nodes and CN functions. This introduces significant complexity:

- Network Function Upgrades: Both gNBs and AMFs (or their 6G equivalents) would require software upgrades to support the QUIC stack while maintaining full SCTP backward compatibility.

- Interoperability Testing: The combinatorial matrix for interoperability testing would expand dramatically, requiring validation of interactions between SCTP-based nodes, QUIC-based nodes, and nodes supporting both.

- Node Selection: Core network routing and node selection mechanisms (e.g., based on TNL associations) would need enhancements to manage and prioritize connections over the two distinct transport protocols.

RAN3 should agree to maintain the SCTP-based Point-to-Point interface as the primary control plane transport for 6G core access functions.

## Service-Based Interface (SBI) Options and Deployment Models

Two overarching deployment models are considered for SBIs in the RAN-CN interface. Option 1, or Full SBA, proposes implementing service-based interfaces for all RAN functions, representing a comprehensive architectural shift. Option 2, or Partial SBA, suggests applying service-based interfaces only to new services such as AI and sensing, while maintaining traditional point-to-point interfaces for legacy access functions like mobility and session management, adhering to 5G principles for critical control functions.

RAN3 should agree to a Partial Service-Based Architecture model for 6G, wherein: New 6G Services (e.g., AI Service Management, Sensing Session Management, Dynamic SLA Negotiation) shall be defined using Service-Based Interfaces.

### Option 2a: SBI with TCP + HTTP/2

This option involves adopting a service-based interface where the RAN exposes or consumes RESTful services over TCP with HTTP/2. The key advantages are strong cloud-native alignment, making it a perfect fit for containerized, microservices-based CN functions, and access to vast industry tooling for development, testing, and observability. HTTP/2 has also been proven successful in the 5G core for interfaces such as Nnrf and Namf. The primary challenge is that this represents a major shift for RAN software architecture, and the textual nature and overhead of HTTP-based signaling may be less efficient for certain high-frequency, latency-sensitive control procedures compared to P2P protocols.

### SBI with QUIC + HTTP/3

This option leverages the latest web standards by using QUIC as the transport for HTTP/3. It combines cloud-native SBI benefits with QUIC's transport advantages, including speed, security, and multiplexing, offering a state-of-the-art and potentially future-proof solution.

As RAN3 evaluates control plane protocol options for 6G, we must recognize that the technological landscape is shifting. While SCTP and HTTP/2 over TCP have served 5G reliably, they represent yesterday's standards in a rapidly evolving internet ecosystem. With HTTP/2 and TCP beginning to show their age, and with QUIC/HTTP/3 emerging as the modern, IETF-standardized replacement already seeing widespread adoption, 6G presents a timely opportunity to align with contemporary internet standards.

The development timeline for 6G specifications coincides perfectly with the maturation and widespread deployment of QUIC/HTTP/3 across the global internet. By the time 6G reaches commercialization, QUIC/HTTP/3 will be the established modern standard, while HTTP/2/TCP will represent a legacy stack. Designing new 6G services on QUIC/HTTP/3 from inception ensures these services launch with contemporary capabilities rather than starting with yesterday's technology.

Beginning new service definitions with HTTP/2/TCP would create immediate technical debt, requiring a future migration to QUIC/HTTP/3. By starting with QUIC/HTTP/3, we avoid this transitional complexity and cost for new services, allowing development efforts to focus on service innovation rather than protocol migration.

For new 6G services, RAN3 agrees to considerate QUIC/HTTP/3 from the inception of SBI based RAN-CN control plane.

# Conclusion

In this contribution, we discussed the candidate options currently under consideration for RAN-CN control plane for 6G: P2P interfaces based on SCTP or QUIC, and SBI using HTTP/2 or HTTP/3 over TCP or QUIC respectively. The following proposals are presented:

- RAN3 should agree to maintain the SCTP-based Point-to-Point interface as the primary control plane transport for 6G core access functions.

- RAN3 should agree to a Partial Service-Based Architecture model for 6G, wherein: New 6G Services (e.g., AI Service Management, Sensing Session Management, Dynamic SLA Negotiation) shall be defined using Service-Based Interfaces.

- For new 6G services, RAN3 agrees to considerate QUIC/HTTP/3 from the inception of SBI based RAN-CN control plane.

# References

RAN3_131_agenda_20260107.docx