3GPP TSG-RAN WG3 Meeting #19 	Tdoc R3-010888

Cardiff, WALES, 26 February- 2 March 2001

| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25.413 |  |  |  | CR | 270 |  |  |  rev |  |  |  | - |  |  Current version: |  |  |  |  | 3.4.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  |  |  | Radio Access Network |  |  |  |  |  |  | X |  | Core Network |  | X |
| Title:  |  | Alignment of NAS PDU size with the RRC Specification |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | Lucent Technologies |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | 2001-02-22 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | R99 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | The size of the NAS PDU in the RRC is a bounded Octet string, whereas in RANAP it is unbounded. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | The NAS PDU size is modified to align with the RRC specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | There will be inconsistency between the RANAP and the RRC protocols, which could lead to operational problem. This change is backward compatible. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 9.2.3.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

#### 9.2.3.4	NAS Broadcast Information

Void

#### 9.2.3.5	NAS PDU

This information element contains the CN – UE  or UE – CN message that is transferred without interpretation in the RNC. Typically it contains call control, session management, supplementary services, short message service and mobility management messages.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| NAS PDU | M |  | OCTET STRING (1..4095) |  |

#### 9.2.3.6	LAI

This element is used to uniquely identify a Location Area.

PARTLY OMMITTED

- LAI ::= SEQUENCE {

pLMN-ID				PLMN-ID,

lAC				LAC,

iE-Extensions			ProtocolExtensionContainer { {LAI-ExtIEs} } OPTIONAL

}

- LAI-ExtIEs RANAP-PROTOCOL-EXTENSION ::= {

...

}

L3-Information				::= OCTET STRING

-- M

MaxBitrate				::= INTEGER (1..16000000)

-- Unit is bits per sec

MaxSDU-Size				::= INTEGER (0..32768)

-- MaxSDU-Size

-- Unit is bit

MCC					::= TBCD-STRING (SIZE (2))

-- Reference: 24.008

MNC					::= TBCD-STRING (SIZE (2))

-- Reference: 24.008

-- N

NAS-PDU					::= OCTET STRING (SIZE (1..4095))

NAS-SynchronisationIndicator	::= BIT STRING (SIZE (4))

- NonSearchingIndication ::= ENUMERATED {

non-searching,

searching

}

NumberOfIuInstances			::= INTEGER (1..2)

NumberOfSteps				::= INTEGER (1..16)

-- O

OldBSS-ToNewBSS-Information		::= OCTET STRING

OMC-ID					::= OCTET STRING (SIZE (3..22))

-- Reference: GSM TS 12.20

PARTLY OMMITTED