3GPP TSG-RAN3 Meeting #25 	R3-013534

Makuhari, Japan November 26-30, 2001

| CR-Form-v3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25.423 |  |  |  | CR | 490 |  |  |  rev |  |  |  | 1 |  |  Current version: |  |  |  |  | 3.7.0 |  |  |  |
| For HELP on using this form, see bottom of this page or look at the pop-up text over the  symbols. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects:  |  |  |  |  | (U)SIM |  |  | ME/UE |  |  |  | Radio Access Network |  |  |  |  |  |  | x |  | Core Network |  |  |
| Title:  |  | TDD Transmit Diversity for P-CCPCH and S-CCPCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source:  |  | InterDigital Communications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code:  |  | TEI |  |  |  |  |  |  |  |  |  |  |  |  |  | Date:  |  | November, 2001 |  |  |  |  |  |
| Category:  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  | Release:  |  | R99 |  |  |  |  |  |
|  |  | Use one of the following categories: F (essential correction) A (corresponds to a correction in an earlier release) B (Addition of feature), C (Functional modification of feature) D (Editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: 2 (GSM Phase 2) R96 (Release 1996) R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) REL-4 (Release 4) REL-5 (Release 5) |  |  |  |  |  |  |
| Reason for change:  |  |  | Block STTD Transmit Diversity is no longer applied. It was replaced by Space Code Transmit Diversity (SCTD) and is now applied to S-CCPCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change:  |  |  | The references to Block STTD are replaced by the references to SCTD. Rev 1. Add SCTD in 3.3 Abbreviations. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact with the previous version of the specification (same release) because the signalling remains identical however of course the layer 1 function triggered is different as reflected in the CRs to the physical layer. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  | Misleading information, inconsistencies between specifications. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  | 3.3, 8.3.1.2, 9.1.4.2, 9.2.1.4A, 9.2.1.41D, 9.3.3, 9.3.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |  |  |  | 25.221 CR 64 and CR 65 25.224 CR 67 and CR 68 25.225 CR 35 and CR 36 25.331 CR 1090 and CR 1091 25.433 CR 549 and CR 550 25.423 CR 491 |  |  |  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

How to create CRs using this form:

Comprehensive information and tips about how to create CRs can be found at: http://www.3gpp.org/3G_Specs/CRs.htm.  Below is a brief summary:

1)	Fill out the above form. The symbols above marked  contain pop-up help information about the field that they are closest to.

2)	Obtain the latest version for the release of the specification to which the change is proposed. Use the MS Word "revision marks"  feature (also known as "track changes") when making the changes. All 3GPP specifications can be downloaded from the 3GPP server under ftp://www.3gpp.org/specs/ For the latest version, look for the directory name with the latest date e.g. 2000-09 contains the specifications resulting from the September 2000 TSG meetings.

3)	With "track changes" disabled, paste the entire CR form (use CTRL-A to select it) into the specification just in front of the clause containing the first piece of changed text.  Delete those parts of the specification which are not relevant to the change request.

## 3.3	Abbreviations

For the purposes of the present document, the following abbreviations apply:

ASN.1	Abstract Syntax Notation One

BLER	Block Error Rate

CCCH	Common Control Channel

CCPCH	Common Control Physical Channel

CCTrCH	Coded Composite Transport Channel

CFN	Connection Frame Number

CM		Compressed Mode

CN	Core Network

CPCH	Common Packet Channel

CPICH	Common Pilot Channel

CRNC	Controlling RNC

DCH	Dedicated Channel

DL	Downlink

DPCCH	Dedicated Physical Control Channel

DPCH	Dedicated Physical Channel

DRNC	Drift RNC

DRNS	Drift RNS

D-RNTI	Drift Radio Network Temporary Identifier

DRX	Discontinuous Reception

DSCH	Downlink Shared Channel

EP	Elementary Procedure

FACH	Forward Access Channel

FDD	Frequency Division Duplex

FP	Frame Protocol

IE	Information Element

IMSI	International Mobile Subscriber Identity

ISCP	Interference Signal Code Power

MAC	Medium Access Control

NAS	Non Access Stratum

O&M	Operation and Maintenance

P-CCPCH	Primary CCPCH

PCH	Paging Channel

P-CIPCH	Primary CIPCH

PCPCH	Physical Common Packet Channel

PDU	Protocol Data Unit

PICH	Paging Indication Channel

PRACH	Physical Random Access Channel

RACH	Random Access Channel

RL	Radio Link

RLC	Radio Link Control

RLS	Radio Link Set

RNS	Radio Network Subsystem

RNSAP	Radio Network Subsystem Application Part

RNTI	Radio Network Temporary Identifier

RRC	Radio Resource Control

RSCP	Received Signal Code Power

S-CCPCH	Secondary CCPCH

SCH	Synchronisation Channel

SCTD                  Space Code Transmit Diversity

SDU	Service Data Unit

SFN	System Frame Number

SIR	Signal-to-Interference Ratio

SRNC	Serving RNC

SRNS	Serving RNS

SSDT	Site Selection Diversity Transmission

STTD	Space Time Transmit Diversity

TDD	Time Division Duplex

TFCI	Transport Format Combination Indicator

TFCS	Transport Format Combination Set

TFS	Transport Format Set

ToAWS	Time of Arrival Window Endpoint

TPC	Transmit Power Control

TrCh	Transport Channel

TSTD	Time Switched Transmit Diversity

UARFCN	UTRA Absolute Radio Frequency Channel Number

UE	User Equipment

UL	Uplink

URA	UTRAN Registration Area

USCH	Uplink Shared Channel

UTRA	Universal Terrestrial Radio Access

UTRAN	Universal Terrestrial Radio Access Network

### 8.3.1	Radio Link Setup

#### 8.3.1.1	General

This procedure is used for establishing the necessary resources in the DRNS for one or more radio links.

The connection-oriented service of the signalling bearer shall be established in conjunction with this procedure.

#### 8.3.1.2	Successful Operation

Figure 5: Radio Link Setup procedure: Successful Operation

When the SRNC makes an algorithmic decision to add the first cell or set of cells from a DRNS to the active set of a specific UE-UTRAN connection, the RADIO LINK SETUP REQUEST message is sent to the corresponding DRNC to request establishment of the radio link(s).

The DRNS shall prioritise resource allocation for the RL(s) to be established according to Annex A.

If the RADIO LINK SETUP REQUEST message includes the Allowed Queuing Time IE the DRNS may queue the request the time corresponding to the value of the Allowed Queuing Time IE before starting to execute the request.

If no D-RNTI IE was included in the RADIO LINK SETUP REQUEST message, the DRNC shall assign a new D-RNTI for this UE.

Transport Channels Handling:

DCH(s):

[TDD - If the DCH Information IE is present in RADIO LINK SETUP REQUEST message, the DRNS shall configure the new DCHs according to the parameters given in the message.]

If the RADIO LINK SETUP REQUEST message includes a DCH Information IE with multiple DCH Specific Info IEs then the DRNS shall treat the DCHs in the DCH Information IE as a set of co-ordinated DCHs.

[FDD - For DCHs which do not belong to a set of co-ordinated DCHs with the QE-Selector IE set to "selected", the Transport channel BER from that DCH shall be the base for the QE in the UL data frames. If no Transport channel BER is available for the selected DCH the Physical channel BER shall be used for the QE, ref. [4]. If the QE-Selector is set to "non-selected ", the Physical channel BER shall be used for the QE in the UL data frames, ref. [4].]

For a set of co-ordinated DCHs the Transport channel BER from the DCH with the QE-Selector IE set to "selected" shall be used for the QE in the UL data frames, ref. [4]. [FDD - If no Transport channel BER is available for the selected DCH the Physical channel BER shall be used for the QE, ref. [4]. If all DCHs have QE-Selector IE set to "non-selected" the Physical channel BER shall be used for the QE, ref. [4].]

The DRNS shall use the included UL DCH FP Mode IE for a DCH or a set of co-ordinated DCHs as the DCH FP Mode in the Uplink of the user plane for the DCH or the set of co-ordinated DCHs.

The DRNS shall use the included ToAWS IE for a DCH or a set of co-ordinated DCHs as the Time of Arrival Window Start Point in the user plane for the DCH or the set of co-ordinated DCHs.

The DRNS shall use the included ToAWE IE for a DCH or a set of co-ordinated DCHs as the Time of Arrival Window End Point in the user plane for the DCH or the set of co-ordinated DCHs.

The Frame Handling Priority IE defines the priority level that should be used by the DRNS to prioritise between different frames of the data frames of the DCHs in the downlink on the radio interface in congestion situations once the new RL(s) have been activated.

DSCH(s):

If the DSCH Information IE is included in the RADIO LINK SETUP REQUEST message, the DRNC shall establish the requested DSCHs [FDD - on the RL indicated by the PDSCH RL ID IE]. In addition, the DRNC shall send a valid set of DSCH Scheduling Priority IE and MAC-c/sh SDU Length IE parameters to the SRNC in the message RADIO LINK SETUP RESPONSE message.

[TDD - USCH(s)]:

[TDD – The DRNS shall use the list of RB Identities in the RB Info IE in the USCH information IE to map each RB Identity IE to the corresponding USCH.]

Physical Channels Handling:

[FDD - Compressed Mode]:

[FDD - If the RADIO LINK SETUP REQUEST message includes the Transmission Gap Pattern Sequence Information IE, the DRNS shall store the information about the Transmission Gap Pattern Sequences to be used in the Compressed Mode Configuration. This Compressed Mode Configuration shall be valid in the DRNS until the next Compressed Mode Configuration is configured in the DRNS or last Radio Link is deleted.]

[FDD - If the RADIO LINK SETUP REQUEST message includes the Transmission Gap Pattern Sequence Information IE and the Active Pattern Sequence Information IE, the DRNS shall use the information to activate the indicated Transmission Gap Pattern Sequences(s) in the new RL. The received CM Configuration Change CFN IE refers to latest passed CFN with that value. The DRNS shall treat the received TGCFN IEs as follows:]

-	[FDD - If any received TGCFN IE has the same value as the received CM Configuration Change CFN  IE, the DRNS shall consider the concerning Transmission Gap Pattern Sequence as activated at that CFN.]

-	[FDD - If any received TGCFN IE does not have the same value as the received CM Configuration Change CFN  IE but the first CFN after the CM Configuration Change CFN with a value equal to the TGCFN IE has already passed, the DRNS shall consider the concerning Transmission Gap Pattern Sequence as activated at that CFN.]

-	[FDD - For all other Transmission Gap Pattern Sequences included in the Active Pattern Sequence Information IE, the DRNS shall activate each Transmission Gap Pattern Sequence at the first CFN after the CM Configuration Change CFN with a value equal to the TGCFN IE for the Transmission Gap Pattern Sequence.]

[FDD- If the Downlink Compressed Mode Method IE in one or more Transmission Gap Pattern Sequence is set to 'SF/2' in the RADIO LINK SETUP REQUEST message, the DRNS shall include the Transmission Gap Pattern Sequence Scrambling Code Information IE in the RADIO LINK SETUP RESPONSE message indicating for each DL Channelisation Code whether the alternative scrambling code shall be used or not.]

[FDD - DL Code Information]:

[FDD – When more than one DL DPDCH are assigned per RL, the segmented physical channel shall be mapped on to DL DPDCHs according to [8]. When p number of DL DPDCHs are assigned to each RL, the first pair of DL Scrambling Code and FDD DL Channelisation Code Number corresponds to “PhCH number 1”, the second to “PhCH number 2”, and so on until the pth to “PhCH number p”.]

General:

[FDD - If the Propagation Delay IE is included, the DRNS may use this information to speed up the detection of UL synchronisation on the Uu interface.]

[FDD – If the received Limited Power Increase IE is set to 'Used', the DRNS shall, if supported, use Limited Power Increase according to ref. [10] subclause 5.2.1 for the inner loop DL power control.]

Radio Link Handling:

Diversity Combination Control:

[FDD - The Diversity Control Field IE indicates for each RL except for the first RL whether the DRNS shall combine the RL with any of the other RLs or not on the Iur. If the Diversity Control Field IE is set to "May" (be combined with another RL), then the DRNS shall decide for any of the alternatives. If the Diversity Control Field IE is set to "Must", the DRNS shall combine the RL with one of the other RL. When an RL is to be combined, the DRNS shall choose which RL(s) to combine it with. If the Diversity Control Field IE is set to "Must not", the DRNS shall not combine the RL with any other existing RL.]

[FDD - In the case of combining one or more RLs the DRNC shall indicate in the RADIO LINK SETUP RESPONSE message with the Diversity Indication IE that the RL is combined with another RL. In this case the Reference RL ID IE shall be included to indicate with which RL the combination is performed. The Reference RL ID IE shall be included for all but one of the combined RLs, for which the Transport Layer Address IE and the Binding ID IE shall be included.]

[FDD - In the case of not combining an RL with another RL, the DRNC shall indicate in the RADIO LINK SETUP RESPONSE message with the Diversity Indication IE that no combining is performed. In this case the DRNC shall include both the Transport Layer Address IE and the Binding ID IE for the transport bearer to be established for each DCH and DSCH of the RL in the RADIO LINK SETUP RESPONSE message.]

[TDD - The DRNC shall always include in the RADIO LINK SETUP RESPONSE message both the Transport Layer Address IE and the Binding ID IE for the transport bearer to be established for each DCH, DSCH and USCH of the RL.]

In case of a set of co-ordinated DCHs requiring a new transport bearer on Iur the Binding ID IE and the Transport Layer Address IE shall be included only for one of the DCHs in the set of co-ordinated DCHs.

[FDD-Transmit Diversity]:

[FDD – If the cell in which the RL is being set up is capable to provide Close loop Tx diversity, the DRNC shall include the Closed Loop Timing Adjustment Mode IE in the RADIO LINK SETUP RESPONSE message indicating the configured Closed loop timing adjustment mode of the cell.]

[FDD – When Diversity Mode IE is "STTD", "Closed loop mode1", or "Closed loop mode2", the DRNC shall activate/deactivate the Transmit Diversity to each Radio Link in accordance with Transmit Diversity Indicator IE].

DL Power Control:

[FDD - If both the Initial DL TX Power IE and Uplink SIR Target IE are included in the message, the DRNS shall use the indicated DL TX Power and Uplink SIR Target as initial value. If the value of the Initial DL TX Power IE is outside the configured DL TX power range, the DRNS shall apply these constrains when setting the initial DL TX power. The DRNS shall also include the configured DL TX power range defined by Maximum DL TX Power IE and Minimum DL TX Power IE in the RADIO LINK SETUP RESPONSE message. The DRNS shall not transmit with a higher power than indicated by the Maximum DL TX Power IE or lower than indicated by the Minimum DL TX Power IE on any DL DPCH of the RL except during compressed mode, when the PSIR(k), as described in ref.[10] subclause 5.2.1.3, shall be added to the maximum DL power in slot k.]

[FDD - If both the Initial DL TX Power and the Uplink SIR Target IEs are not included in the RADIO LINK SETUP REQUEST message, then DRNC shall determine the initial Uplink SIR Target and include it in the Uplink SIR Target IE in the RADIO LINK SETUP RESPONSE message.]

[FDD - If the Primary CPICH Ec/No IE is present, the DRNC should use the indicated value when deciding the Initial DL TX Power.]

[TDD - If the Primary CCPCH RSCP IE and/or the DL Time Slot ISCP Info IE are present, the DRNC should use the indicated values when deciding the Initial DL TX Power.]

[FDD – The DRNS shall start the DL transmission using the indicated DL TX power level (if received) or the decided DL TX power level on each DL channelisation code of a RL until UL synchronisation is achieved on the Uu interface for the concerning RLS or a DL POWER CONTROL REQUEST message is received. No inner loop power control or power  balancing shall be performed during this period. The DL power shall then vary according to the inner loop power control (see ref.[10] subclause 5.2.1.2) with DPC_MODE=0 and the power control procedure (see 8.3.7).]

[TDD – The DRNS shall start the DL transmission using the decided DL TX power level on each DL channelisation code and on each Time Slot of a RL until UL synchronisation is achieved on the Uu interface for the concerning RL. No inner loop power control shall be performed during this period. The DL power shall then vary according to the inner loop power control (see ref.[22] subclause 4.2.3.3). ]

[FDD – If the received Inner Loop DL PC Status IE is set to  “Active”, the DRNS shall activate the inner loop DL power control for all RLs. If Inner Loop DL PC Status IE is set to  “Inactive”, the DRNS shall deactivate the inner loop DL power control for all RLs according to ref. [10]]

Neighbouring Cell Handling:

If there are UMTS neighbouring cell(s) to the cell in which a Radio Link was established then:

-	The DRNC shall include the Neighbouring FDD Cell Information IE and/or Neighbouring TDD Cell Information IE in the Neighbouring UMTS Cell Information IE for each neighbouring FDD cell and/or TDD cell respectively. In addition, if the information is available, the DRNC shall include the Frame Offset IE, Primary CPICH Power IE, Cell Individual Offset IE, STTD Support Indicator IE, Closed Loop Mode1 Support Indicator IE and Closed Loop Mode2 Support Indicator IE in the Neighbouring FDD Cell Information IE, and the Frame Offset IE, Cell Individual Offset IE, DPCH Constant Value IE and the PCCPCH Power IE in the Neighbouring TDD Cell Information IE.

-	If a UMTS neighbouring cell is not controlled by the same DRNC, the DRNC shall also include the CN PS Domain Identifier IE and/or CN CS Domain Identifier IE which are the identifiers of the CN nodes connected to the RNC controlling the UMTS neighbouring cell.

For the UMTS neighbouring cells which are controlled by the DRNC, the DRNC shall report in the RADIO LINK SETUP RESPONSE message the restriction state of those cells, otherwise Restriction state indicator IE may be absent. The DRNC shall include the Restriction state indicator IE for the neighbouring cells which are controlled by the DRNC in the Neighbouring FDD Cell Information IE and the Neighbouring TDD Cell Information IE.

If there are GSM neighbouring cells to the cell(s) where a radio link is established, the DRNC shall include the Neighbouring GSM Cell Information IE in the RADIO LINK SETUP RESPONSE message for each of the GSM neighbouring cells. If available the DRNC shall include the Cell Individual Offset IE in the Neighbouring GSM Cell Information IE.

General:

[FDD - If the RADIO LINK SETUP REQUEST message includes the SSDT Cell Identity IE, the DRNS shall activate SSDT, if supported, using the SSDT Cell Identity IE and SSDT Cell Identity Length IE.]

[FDD - If the DRAC Control IE is set to "requested" in the RADIO LINK SETUP REQUEST message for at least one DCH and if the DRNS supports the DRAC, the DRNC shall indicate in the RADIO LINK SETUP RESPONSE message the Secondary CCPCH Info IE for the FACH where the DRAC information is sent, for each Radio Link established in a cell where DRAC is active. If the DRNS does not support DRAC, the DRNC shall not provide these IEs in the RADIO LINK SETUP RESPONSE message.]

If no D-RNTI IE was included in the RADIO LINK SETUP REQUEST message, the DRNC shall include the node identifications of the CN Domain nodes that the RNC is connected to (using LAC and RAC of the current cell), and the D-RNTI IE in the RADIO LINK SETUP RESPONSE message.

[FDD - If the D-RNTI IE was included the RADIO LINK SETUP REQUEST message the DRNC shall include the Primary Scrambling Code IE, the UL UARFCN IE, the DL UARFCN IE, and the Primary CPICH Power IE in the RADIO LINK SETUP RESPONSE message.]

[TDD – If the D-RNTI IE was included in the RADIO LINK SETUP REQUEST message the DRNC shall include the UARFCN IE, the Cell Parameter ID IE, the Sync Case IE, the SCH Time Slot IE, the SCTD Indicator IE, and the PCCPCH Power IE in the RADIO LINK SETUP RESPONSE message.]

[TDD - The DRNC shall include the Secondary CCPCH Info TDD IE in the RADIO LINK SETUP RESPONSE message if at least one DSCH Information Response IE or USCH Information Response IE is included in the message and at least one DCH is configured for the radio link. The DRNC shall also include the Secondary CCPCH Info TDD IE in the RADIO LINK SETUP RESPONSE message if at least one DSCH Information Response IE or USCH Information Response IE is included in the message and the SHCCH messages for this radio link will be transmitted over a different secondary CCPCH than selected by the UE from system information.]

For each Radio Link established in a cell where at least one URA Identity is being broadcast, the DRNC shall include a URA Identity for this cell in the URA ID IE, the Multiple URAs Indicator IE indicating whether or not multiple URA Identities are being broadcast in the cell, and the RNC Identity of all other RNCs that are having at least one cell within the URA in the cell in the URA Information IE in the RADIO LINK SETUP RESPONSE message.

Depending on local configuration in the DRNS, it may include the geographical co-ordinates of the cell and the UTRAN access point position for each of the established RLs in the RADIO LINK SETUP RESPONSE message.

If the Permanent NAS UE Identity IE is included in the RADIO LINK SETUP REQUEST message, the DRNC shall store the information for the considered UE Context for the life-time of the UE Context.

If the RADIO LINK SETUP REQUEST message includes the Permanent NAS UE Identity IE and a C-ID IE corresponding to a cell reserved for operator use, the DRNC shall use this information to determine whether it can set up a Radio Link on this cell or not for the considered UE Context.

[FDD - Radio Link Set Handling]:

[FDD - The First RLS Indicator IE indicates if the concerning RL shall be considered part of the first RLS established towards this UE. The First RLS Indicator IE shall be used by the DRNS to determine the initial TPC pattern in the DL of the concerning RL and all RLs which are part of the same RLS, as described in [10], section 5.1.2.2.1.2.

[FDD – For each RL not having a common generation of the TPC commands in the DL with another RL, the DRNS shall assign the RL Set ID IE included in the RADIO LINK SETUP RESPONSE message a value that uniquely identifies the RL Set within the UE Context.]

[FDD – For all RLs having a common generation of the TPC commands in the DL with another RL, the DRNS shall assign the RL Set ID IE included in the RADIO LINK SETUP RESPONSE message the same value. This value shall uniquely identify the RL Set within the UE context.]

[FDD –The UL Uu synchronisation detection algorithm defined in ref. [10] subclause 4.3 shall for each of the established RL Set(s) use the maximum value of the parameters N_OUTSYNC_IND and T_RLFAILURE, and the minimum value of the parameters N_INSYNC_IND, that are configured in the cells supporting the radio links of the RL Set].

Response Message:

At the reception of the RADIO LINK SETUP REQUEST message, DRNS allocates the requested type of channelisation codes and other physical channel resources for each RL and assigns a binding identifier and a transport layer address for each DCH or set of co-ordinated DCHs and for each DSCH [TDD – and USCH]. This information shall be sent to the SRNC in the message RADIO LINK SETUP RESPONSE when all the RLs have been successfully established.

After sending of the RADIO LINK SETUP RESPONSE message the DRNS shall continuously attempt to obtain UL synchronisation on the Uu interface and start reception on the new RL. [FDD - The DRNS shall start DL transmission on the new RL after synchronisation is achieved in the DL user plane as specified in ref. [4].] [TDD – The DRNS shall start transmission on the new RL immediately as specified in ref. [4].]

### 9.1.4	RADIO LINK SETUP RESPONSE

#### 9.1.4.1	FDD Message

#### 9.1.4.2	TDD Message

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.40 |  | YES | reject |
| Transaction ID | M |  | 9.2.1.59 |  | – |  |
| D-RNTI | O |  | 9.2.1.24 |  | YES | ignore |
| CN PS Domain Identifier | O |  | 9.2.1.12 |  | YES | ignore |
| CN CS Domain Identifier | O |  | 9.2.1.11 |  | YES | ignore |
| RL Information Response |  | 1 |  |  | YES | ignore |
| >RL ID | M |  | 9.2.1.49 |  | – |  |
| >URA Information | O |  | 9.2.1.70B |  | – |  |
| >SAI | M |  | 9.2.1.52 |  | – |  |
| >Cell GAI | O |  | 9.2.1.5A |  | – |  |
| >UTRAN Access Point Position | O |  | 9.2.1.70A |  | – |  |
| >UL Time Slot ISCP Info | M |  | 9.2.3.13D |  | – |  |
| >Maximum Uplink SIR | M |  | Uplink SIR 9.2.1.69 |  | – |  |
| >Minimum Uplink SIR | M |  | Uplink SIR 9.2.1.69 |  | – |  |
| >Maximum Allowed UL Tx Power | M |  | 9.2.1.35 |  | – |  |
| >Maximum DL TX Power | M |  | DL Power 9.2.1.21A |  | – |  |
| >Minimum DL TX Power | M |  | DL Power 9.2.1.21A |  | – |  |
| >UARFCN | O |  | UARFCN 9.2.1.66 | Corresponds to Nt in ref. [7] | – |  |
| >Cell Parameter ID | O |  | 9.2.1.8 |  | – |  |
| >Sync Case | O |  | 9.2.1.54 |  | – |  |
| >SCH Time Slot | C-Case2 |  | 9.2.1.51 |  | – |  |
| > SCTD Indicator | O |  | 9.2.1.x |  | – |  |
| >PCCPCH Power | M |  | 9.2.1.43 |  | – |  |
| >Timing Advance Applied | M |  | 9.2.3.12A |  | – |  |
| >Alpha Value | M |  | 9.2.3.a |  | – |  |
| >UL PhysCH SF Variation | M |  | 9.2.3.13B |  | – |  |
| >Synchronisation Configuration | M |  | 9.2.3.7E |  | – |  |
| >Secondary CCPCH Info TDD | O |  | 9.2.3.7B |  | – |  |
| >UL CCTrCH Information |  | 0..<maxnoofCCTrCHs> |  | For DCH | GLOBAL | ignore |
| >>CCTrCH ID | M |  | 9.2.3.2 |  | – |  |
| >>UL DPCH Information |  | 0..1 |  |  | YES | ignore |
| >>>Repetition Period | M |  | 9.2.3.7 |  | – |  |
| >>>Repetition Length | M |  | 9.2.3.6 |  | – |  |
| >>>TDD DPCH Offset | M |  | 9.2.3.8A |  | – |  |
| >>>UL Timeslot Information | M |  | 9.2.3.13C |  | – |  |
| >DL CCTrCH Information |  | 0..<maxnoofCCTrCHs> |  | For DCH | GLOBAL | ignore |
| >>CCTrCH ID | M |  | 9.2.3.2 |  | – |  |
| >>DL DPCH Information |  | 0..1 |  |  | YES | ignore |
| >>>Repetition Period | M |  | 9.2.3.7 |  | – |  |
| >>>Repetition Length | M |  | 9.2.3.6 |  | – |  |
| >>>TDD DPCH Offset | M |  | 9.2.3.8A |  | – |  |
| >>>DL Timeslot Information | M |  | 9.2.3.2C |  |  |  |
| >DCH Information Response | O |  | 9.2.1.16A |  | YES | ignore |
| >DSCH Information Response |  | 0 .. <Maxnoof DSCHs> |  |  | GLOBAL | ignore |
| >>DSCH ID | M |  | 9.2.1.26A |  | – |  |
| >>DSCH Flow Control Information | M |  | 9.2.1.26B |  | – |  |
| >>Binding ID | O |  | 9.2.1.3 |  | – |  |
| >>Transport Layer Address | O |  | 9.2.1.62 |  | – |  |
| >>Transport Format Management | M |  | 9.2.3.13 |  | – |  |
| >USCH Information Response |  | 0 .. <Maxnoof USCHs> |  |  | GLOBAL | ignore |
| >>USCH ID | M |  | 9.2.3.14 |  | – |  |
| >>Binding ID | O |  | 9.2.1.3 |  | – |  |
| >>Transport Layer Address | O |  | 9.2.1.62 |  | – |  |
| >>Transport Format Management | M |  | 9.2.3.13 |  | – |  |
| >Neighbouring UMTS Cell Information | O |  | 9.2.1.41A |  | – |  |
| >Neighbouring GSM Cell Information | O |  | 9.2.1.41C |  | – |  |
| Uplink SIR Target | M |  | Uplink SIR 9.2.1.69 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.13 |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| Case2 | This IE shall be present if Sync Case IE is "Case2". |

| Range bound | Explanation |
| --- | --- |
| MaxnoofDSCHs | Maximum number of DSCHs for one UE. |
| MaxnoofUSCHs | Maximum number of USCHs for one UE. |
| MaxnoofCCTrCHs | Maximum number of CCTrCH for one UE. |

#### 9.2.1.4A	Block STTD Indicator

- Void.

#### 9.2.1.x	SCTD Indicator

- Indicates if SCTD antenna diversity is applied or not to the PCCPCH and SCCPCH.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SCTD Indicator |  |  | ENUMERATED(active, inactive) |  |

#### 9.2.1.41D	Neighbouring TDD Cell Information

The Neighbouring TDD Cell Information IE provides information for TDD cells that are a neighbouring cells to a cell in the DRNC.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Neighbouring TDD Cell Information |  | 1..<maxnoofTDDneighbours> |  |  | – |  |
| >C-Id | M |  | 9.2.1.6 |  | – |  |
| >UARFCN | M |  | 9.2.1.66 | Corresponds to Nt in ref. [7] | – |  |
| >Frame Offset | O |  | 9.2.1.30 |  | – |  |
| >Cell Parameter ID | M |  | 9.2.1.8 |  | – |  |
| >Sync Case | M |  | 9.2.1.54 |  | – |  |
| >Time Slot | C-Case1 |  | 9.2.1.56 |  | – |  |
| >SCH Time Slot | C-Case2 |  | 9.2.1.51 |  | – |  |
| > SCTD Indicator | M |  | 9.2.1.x |  | – |  |
| >Cell Individual Offset | O |  | 9.2.1.7 |  | – |  |
| >DPCH Constant Value | O |  | 9.2.1.23 |  | – |  |
| >PCCPCH Power | O |  | 9.2.1.43 |  | – |  |
| >Restriction State Indicator | O |  | 9.2.1.48A |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| Case1 | The IE shall be present if Sync Case IE is set to “Case1”. |
| Case2 | The IE shall be present if Sync Case IE is set to “Case2”. |

| Range bound | Explanation |
| --- | --- |
| MaxnoofTDDneighbours | Maximum number of neighbouring TDD cell for one cell. |

### 9.3.3	PDU Definitions

-- **************************************************************

--

-- PDU definitions for RNSAP.

--

-- **************************************************************

RNSAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

umts-Access (20) modules (3) rnsap (1) version1 (1) rnsap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Active-Pattern-Sequence-Information,

AllocationRetentionPriority,

AllowedQueuingTime,

AlphaValue,

BLER,

SCTD-Indicator,

BindingID,

C-ID,

C-RNTI,

CCTrCH-ID,

CFN,

ClosedLoopMode1-SupportIndicator,

ClosedLoopMode2-SupportIndicator,

Closedlooptimingadjustmentmode,

CN-CS-DomainIdentifier,

CN-PS-DomainIdentifier,

CNDomainType,

Cause,

CellParameterID,

ChipOffset,

CriticalityDiagnostics,

D-RNTI,

D-RNTI-ReleaseIndication,

DCH-FDD-Information,

DCH-ID,

DCH-InformationResponse,

DCH-TDD-Information,

DL-DPCH-SlotFormat,

DL-TimeslotISCP,

DL-Power,

DL-ScramblingCode,

DL-Timeslot-Information,

DL-TimeSlot-ISCP-Info,

DPCH-ID,

DRACControl,

DRXCycleLengthCoefficient,

DedicatedMeasurementType,

DedicatedMeasurementValue,

DedicatedMeasurementValueInformation,

DiversityControlField,

DiversityMode,

DSCH-FDD-Information,

DSCH-FDD-InformationResponse,

DSCH-FlowControlInformation,

DSCH-FlowControlItem,

DSCH-TDD-Information,

DSCH-ID,

SchedulingPriorityIndicator,

FACH-FlowControlInformation,

FDD-DCHs-to-Modify,

FDD-DL-ChannelisationCodeNumber,

FDD-DL-CodeInformation,

FDD-S-CCPCH-Offset,

FDD-TPC-DownlinkStepSize,

FirstRLS-Indicator,

FNReportingIndicator,

FrameHandlingPriority,

FrameOffset,

GA-AccessPointPosition,

GA-Cell,

IMSI,

InnerLoopDLPCStatus,

L3-Information,

LimitedPowerIncrease,

MaximumAllowedULTxPower,

MaxNrDLPhysicalchannels,

MaxNrOfUL-DPCHs,

MaxNrTimeslots,

MaxNrULPhysicalchannels,

MeasurementFilterCoefficient,

MeasurementID,

MidambleShiftAndBurstType,

MinimumSpreadingFactor,

MinUL-ChannelisationCodeLength,

MultiplexingPosition,

Neighbouring-GSM-CellInformation,

Neighbouring-UMTS-CellInformation,

NrOfDLchannelisationcodes,

PagingCause,

PagingRecordType,

PDSCHCodeMapping,

PayloadCRC-PresenceIndicator,

PCCPCH-Power,

PC-Preamble,

Permanent-NAS-UE-Identity,

PowerAdjustmentType,

PowerOffset,

PrimaryCCPCH-RSCP,

PrimaryCPICH-EcNo,

PrimaryCPICH-Power,

PrimaryScramblingCode,

PropagationDelay,

PunctureLimit,

QE-Selector,

RANAP-RelocationInformation,

RB-Info,

RL-ID,

RL-Set-ID,

RNC-ID,

RepetitionLength,

RepetitionPeriod,

ReportCharacteristics,

Received-total-wide-band-power,

RxTimingDeviationForTA,

S-FieldLength,

S-RNTI,

SCH-TimeSlot,

SAI,

Secondary-CCPCH-Info,

Secondary-CCPCH-Info-TDD,

SpecialBurstScheduling,

SSDT-CellID,

SSDT-CellID-Length,

SSDT-Indication,

SSDT-SupportIndicator,

STTD-Indicator,

STTD-SupportIndicator,

AdjustmentPeriod,

ScaledAdjustmentRatio,

MaxAdjustmentStep,

SecondaryCCPCH-SlotFormat,

SRB-Delay,

SyncCase,

SynchronisationConfiguration,

TDD-ChannelisationCode,

TDD-DCHs-to-Modify,

TDD-DL-Code-Information,

TDD-DPCHOffset,

TDD-PhysicalChannelOffset,

TDD-TPC-DownlinkStepSize,

TDD-UL-Code-Information,

TFCI-Coding,

TFCI-Presence,

TFCI-SignallingMode,

TimeSlot,

TimingAdvanceApplied,

ToAWE,

ToAWS,

TransmitDiversityIndicator,

TransportBearerID,

TransportBearerRequestIndicator,

TFCS,

Transmission-Gap-Pattern-Sequence-Information,

TransportFormatManagement,

TransportFormatSet,

TransportLayerAddress,

TrCH-SrcStatisticsDescr,

UARFCN,

UC-ID,

UL-DPCCH-SlotFormat,

UL-SIR,

UL-FP-Mode,

UL-PhysCH-SF-Variation,

UL-ScramblingCode,

UL-Timeslot-Information,

UL-TimeSlot-ISCP-Info,

URA-ID,

URA-Information,

USCH-ID,

USCH-Information

-- **************************************************************

--

-- RADIO LINK SETUP RESPONSE TDD

--

-- **************************************************************

RadioLinkSetupResponseTDD ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{RadioLinkSetupResponseTDD-IEs}},

protocolExtensions              ProtocolExtensionContainer {{RadioLinkSetupResponseTDD-Extensions}}                   OPTIONAL,

...

}

RadioLinkSetupResponseTDD-IEs RNSAP-PROTOCOL-IES ::= {

{ ID id-D-RNTI					CRITICALITY ignore	TYPE D-RNTI 					PRESENCE optional	} |

{ ID id-CN-PS-DomainIdentifier			CRITICALITY ignore	TYPE CN-PS-DomainIdentifier			PRESENCE optional	} |

{ ID id-CN-CS-DomainIdentifier			CRITICALITY ignore	TYPE CN-CS-DomainIdentifier			PRESENCE optional	} |

{ ID id-RL-InformationResponse-RL-SetupRspTDD	CRITICALITY ignore	TYPE RL-InformationResponse-RL-SetupRspTDD 	PRESENCE mandatory	} |

{ ID id-UL-SIRTarget				CRITICALITY ignore	TYPE UL-SIR 				PRESENCE mandatory	} |

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

RL-InformationResponse-RL-SetupRspTDD ::= SEQUENCE {

rL-ID						RL-ID,

uRA-Information				URA-Information		OPTIONAL,

sAI							SAI,

gA-Cell						GA-Cell		OPTIONAL,

gA-AccessPointPosition		GA-AccessPointPosition	OPTIONAL,

ul-TimeSlot-ISCP-Info		UL-TimeSlot-ISCP-Info,

maxUL-SIR					UL-SIR,

minUL-SIR					UL-SIR,

maximumAllowedULTxPower		MaximumAllowedULTxPower,

maximumDLTxPower			DL-Power,

minimumDLTxPower			DL-Power,

uARFCNforNt					UARFCN				OPTIONAL,

cellParameterID				CellParameterID		OPTIONAL,

syncCase					SyncCase			OPTIONAL,

sCH-TimeSlot				SCH-TimeSlot		OPTIONAL,

-- This IE shall be present if Sync Case IE is Case2. --

sCTD-Indicator		SCTD-Indicator	OPTIONAL,

pCCPCH-Power				PCCPCH-Power,

timingAdvanceApplied		TimingAdvanceApplied,

alphaValue					AlphaValue,

ul-PhysCH-SF-Variation		UL-PhysCH-SF-Variation,

synchronisationConfiguration		SynchronisationConfiguration,

secondary-CCPCH-Info-TDD			Secondary-CCPCH-Info-TDD	OPTIONAL,

ul-CCTrCHInformation				UL-CCTrCHInformationList-RL-SetupRspTDD		OPTIONAL,

dl-CCTrCHInformation				DL-CCTrCHInformationList-RL-SetupRspTDD		OPTIONAL,

dCH-InformationResponse				DCH-InformationResponseList-RL-SetupRspTDD	OPTIONAL,

dsch-InformationResponse			DSCH-InformationResponse-RL-SetupRspTDD OPTIONAL,

usch-InformationResponse			USCH-InformationResponse-RL-SetupRspTDD OPTIONAL,

neighbouring-UMTS-CellInformation			Neighbouring-UMTS-CellInformation OPTIONAL,

neighbouring-GSM-CellInformation			Neighbouring-GSM-CellInformation OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {RL-InformationResponse-RL-SetupRspTDD-ExtIEs} } OPTIONAL,

...

}

RL-InformationResponse-RL-SetupRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

UL-CCTrCHInformationList-RL-SetupRspTDD ::= ProtocolIE-Single-Container {{UL-CCTrCHInformationListIEs-RL-SetupRspTDD}}

UL-CCTrCHInformationListIEs-RL-SetupRspTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-UL-CCTrCH-InformationListIE-RL-SetupRspTDD   CRITICALITY ignore	TYPE UL-CCTrCHInformationListIE-RL-SetupRspTDD		PRESENCE mandatory }

}

UL-CCTrCHInformationListIE-RL-SetupRspTDD ::= SEQUENCE (SIZE (1..maxNrOfCCTrCHs)) OF UL-CCTrCHInformationItem-RL-SetupRspTDD

UL-CCTrCHInformationItem-RL-SetupRspTDD ::= SEQUENCE {

cCTrCH-ID					CCTrCH-ID,

ul-DPCH-Information				UL-DPCH-InformationList-RL-SetupRspTDD		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {UL-CCTrCHInformationItem-RL-SetupRspTDD-ExtIEs} } OPTIONAL,

...

}

UL-CCTrCHInformationItem-RL-SetupRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

UL-DPCH-InformationList-RL-SetupRspTDD ::= ProtocolIE-Single-Container { {UL-DPCH-InformationListIEs-RL-SetupRspTDD} }

UL-DPCH-InformationListIEs-RL-SetupRspTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-UL-DPCH-InformationItem-RL-SetupRspTDD		CRITICALITY ignore	TYPE UL-DPCH-InformationItem-RL-SetupRspTDD  PRESENCE mandatory	}

}

UL-DPCH-InformationItem-RL-SetupRspTDD ::= SEQUENCE {

repetitionPeriod				RepetitionPeriod,

repetitionLength				RepetitionLength,

tDD-DPCHOffset					TDD-DPCHOffset,

uL-Timeslot-Information			UL-Timeslot-Information,

iE-Extensions					ProtocolExtensionContainer { {UL-DPCH-InformationItem-RL-SetupRspTDD-ExtIEs} } OPTIONAL,

...

}

UL-DPCH-InformationItem-RL-SetupRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

DL-CCTrCHInformationList-RL-SetupRspTDD ::= ProtocolIE-Single-Container {{DL-CCTrCHInformationListIEs-RL-SetupRspTDD}}

DL-CCTrCHInformationListIEs-RL-SetupRspTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-DL-CCTrCH-InformationListIE-RL-SetupRspTDD   CRITICALITY ignore	TYPE DL-CCTrCHInformationListIE-RL-SetupRspTDD  PRESENCE mandatory }

}

DL-CCTrCHInformationListIE-RL-SetupRspTDD ::= SEQUENCE (SIZE (1..maxNrOfCCTrCHs)) OF DL-CCTrCHInformationItem-RL-SetupRspTDD

DL-CCTrCHInformationItem-RL-SetupRspTDD ::= SEQUENCE {

cCTrCH-ID					CCTrCH-ID,

dl-DPCH-Information				DL-DPCH-InformationList-RL-SetupRspTDD		OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { {DL-CCTrCHInformationItem-RL-SetupRspTDD-ExtIEs} } OPTIONAL,

...

}

DL-CCTrCHInformationItem-RL-SetupRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

DL-DPCH-InformationList-RL-SetupRspTDD ::= ProtocolIE-Single-Container { {DL-DPCH-InformationListIEs-RL-SetupRspTDD} }

DL-DPCH-InformationListIEs-RL-SetupRspTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-DL-DPCH-InformationItem-RL-SetupRspTDD		CRITICALITY ignore	TYPE DL-DPCH-InformationItem-RL-SetupRspTDD  PRESENCE mandatory	}

}

DL-DPCH-InformationItem-RL-SetupRspTDD ::= SEQUENCE {

repetitionPeriod				RepetitionPeriod,

repetitionLength				RepetitionLength,

tDD-DPCHOffset					TDD-DPCHOffset,

dL-Timeslot-Information			DL-Timeslot-Information,

iE-Extensions					ProtocolExtensionContainer { {DL-DPCH-InformationItem-RL-SetupRspTDD-ExtIEs} } OPTIONAL,

...

}

DL-DPCH-InformationItem-RL-SetupRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

DCH-InformationResponseList-RL-SetupRspTDD ::= ProtocolIE-Single-Container {{DCH-InformationResponseListIEs-RL-SetupRspTDD}}

DCH-InformationResponseListIEs-RL-SetupRspTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-DCH-InformationResponse   CRITICALITY ignore	TYPE DCH-InformationResponse  PRESENCE mandatory }

}

DSCH-InformationResponse-RL-SetupRspTDD ::= ProtocolIE-Single-Container {{DSCH-InformationList-RL-SetupRspTDD}}

DSCH-InformationList-RL-SetupRspTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-DSCH-InformationListIEs-RL-SetupRspTDD		CRITICALITY ignore	TYPE DSCH-InformationListIEs-RL-SetupRspTDD PRESENCE mandatory }

}

DSCH-InformationListIEs-RL-SetupRspTDD ::= SEQUENCE (SIZE(0..maxNoOfDSCHs)) OF DSCHInformationItem-RL-SetupRspTDD

DSCHInformationItem-RL-SetupRspTDD ::= SEQUENCE {

dsch-ID					DSCH-ID,

dSCH-FlowControlInformation		DSCH-FlowControlInformation,

bindingID				BindingID	OPTIONAL,

transportLayerAddress	TransportLayerAddress	OPTIONAL,

transportFormatManagement	TransportFormatManagement,

iE-Extensions			ProtocolExtensionContainer { {DSCHInformationItem-RL-SetupRspTDD-ExtIEs} } OPTIONAL,

...

}

DSCHInformationItem-RL-SetupRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

USCH-InformationResponse-RL-SetupRspTDD ::= ProtocolIE-Single-Container {{USCH-InformationList-RL-SetupRspTDD}}

USCH-InformationList-RL-SetupRspTDD RNSAP-PROTOCOL-IES ::= {

{ ID id-USCH-InformationListIEs-RL-SetupRspTDD		CRITICALITY ignore	TYPE USCH-InformationListIEs-RL-SetupRspTDD PRESENCE mandatory }

}

USCH-InformationListIEs-RL-SetupRspTDD ::= SEQUENCE (SIZE(0..maxNoOfUSCHs)) OF USCHInformationItem-RL-SetupRspTDD

USCHInformationItem-RL-SetupRspTDD ::= SEQUENCE {

usch-ID						USCH-ID,

bindingID					BindingID	OPTIONAL,

transportLayerAddress		TransportLayerAddress	OPTIONAL,

transportFormatManagement	TransportFormatManagement,

iE-Extensions				ProtocolExtensionContainer { {USCHInformationItem-RL-SetupRspTDD-ExtIEs} } OPTIONAL,

...

}

USCHInformationItem-RL-SetupRspTDD-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

RadioLinkSetupResponseTDD-Extensions RNSAP-PROTOCOL-EXTENSION ::= {

...

}

### 9.3.4	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

RNSAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

umts-Access (20) modules (3) rnsap (1) version1 (1) rnsap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

maxCodeNumComp-1,

maxNrOfFACHs,

maxFACHCountPlus1,

maxIBSEG,

maxNoOfDSCHs,

maxNoOfUSCHs,

maxNoTFCIGroups,

maxNoCodeGroups,

maxNrOfDCHs,

maxNrOfDL-Codes,

maxNrOfDLTs,

maxNrOfDPCHs,

maxNrOfErrors,

maxNrOfFDDNeighboursPerRNC,

maxNrOfMACcshSDU-Length,

maxNrOfNeighbouringRNCs,

maxNrOfTDDNeighboursPerRNC,

maxNrOfTS,

maxNrOfULTs,

maxNrOfGSMNeighboursPerRNC,

maxRateMatching,

maxNrOfPoints,

maxNoOfRB,

maxNrOfTFCs,

maxNrOfTFs,

maxCTFC,

maxRNCinURA-1,

maxNrOfSCCPCHs,

maxTFCI1Combs,

maxTFCI2Combs,

maxTFCI2Combs-1,

maxTGPS,

maxTTI-Count,

id-Neighbouring-GSM-CellInformation,

id-Neighbouring-UMTS-CellInformationItem,

maxNrOfLevels,

id-MessageStructure,

id-RestrictionStateIndicator,

id-TypeOfError

FROM RNSAP-Constants

Criticality,

ProcedureID,

ProtocolIE-ID,

TransactionID,

TriggeringMessage

FROM RNSAP-CommonDataTypes

ProtocolIE-Single-Container{},

ProtocolExtensionContainer{},

RNSAP-PROTOCOL-IES,

RNSAP-PROTOCOL-EXTENSION

FROM RNSAP-Containers;

-- A

Active-Pattern-Sequence-Information ::= SEQUENCE {

cMConfigurationChangeCFN		CFN,

transmission-Gap-Pattern-Sequence-Status	Transmission-Gap-Pattern-Sequence-Status-List		OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {Active-Pattern-Sequence-Information-ExtIEs} } OPTIONAL,

...

}

Active-Pattern-Sequence-Information-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

AdjustmentPeriod			::= INTEGER(1..256)

-- Unit Frame

AllocationRetentionPriority ::= SEQUENCE {

priorityLevel				PriorityLevel,

pre-emptionCapability		Pre-emptionCapability,

pre-emptionVulnerability	Pre-emptionVulnerability,

iE-Extensions			ProtocolExtensionContainer { {AllocationRetentionPriority-ExtIEs} } OPTIONAL,

...

}

AllocationRetentionPriority-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

AllowedQueuingTime			::= INTEGER (1..60)

-- seconds

AlphaValue					::=	INTEGER (0..8)

-- Actual value = Alpha / 8

-- B

Band-Indicator ::= ENUMERATED {

dcs1800Band,

pcs1900Band,

...

}

BCC ::= BIT STRING (SIZE (3))

BCCH-ARFCN ::= INTEGER (0..1023)

BetaCD ::= INTEGER (0..15)

BindingID				::= OCTET STRING (SIZE (1..4,...))

BLER					::= INTEGER (-63..0)

-- Step 0.1 (Range -6.3..0). It is the Log10 of the BLER

SCTD-Indicator	::= ENUMERATED {

active,

inactive

}

BSIC ::= SEQUENCE {

nCC			NCC,

bCC			BCC

}

-- N

NCC ::= BIT STRING (SIZE (3))

Neighbouring-UMTS-CellInformation ::= SEQUENCE (SIZE (1..maxNrOfNeighbouringRNCs)) OF ProtocolIE-Single-Container {{ Neighbouring-UMTS-CellInformationItemIE }}

Neighbouring-UMTS-CellInformationItemIE RNSAP-PROTOCOL-IES ::= {

{ ID id-Neighbouring-UMTS-CellInformationItem	CRITICALITY ignore	TYPE	Neighbouring-UMTS-CellInformationItem	PRESENCE	mandatory }

}

Neighbouring-UMTS-CellInformationItem ::= SEQUENCE {

rNC-ID									RNC-ID,

cN-PS-DomainIdentifier					CN-PS-DomainIdentifier		OPTIONAL,

cN-CS-DomainIdentifier					CN-CS-DomainIdentifier		OPTIONAL,

neighbouring-FDD-CellInformation		Neighbouring-FDD-CellInformation	OPTIONAL,

neighbouring-TDD-CellInformation		Neighbouring-TDD-CellInformation	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { {Neighbouring-UMTS-CellInformationItem-ExtIEs} } OPTIONAL,

...

}

Neighbouring-UMTS-CellInformationItem-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

Neighbouring-FDD-CellInformation ::= SEQUENCE ( SIZE (1..maxNrOfFDDNeighboursPerRNC,...)) OF Neighbouring-FDD-CellInformationItem

Neighbouring-FDD-CellInformationItem ::= SEQUENCE {

c-ID								C-ID,

uARFCNforNu							UARFCN,

uARFCNforNd							UARFCN,

frameOffset							FrameOffset			OPTIONAL,

primaryScramblingCode				PrimaryScramblingCode,

primaryCPICH-Power					PrimaryCPICH-Power		OPTIONAL,

cellIndividualOffset				CellIndividualOffset	OPTIONAL,

txDiversityIndicator 				TxDiversityIndicator,

sTTD-SupportIndicator				STTD-SupportIndicator	OPTIONAL,

closedLoopMode1-SupportIndicator	ClosedLoopMode1-SupportIndicator	OPTIONAL,

closedLoopMode2-SupportIndicator	ClosedLoopMode2-SupportIndicator	OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { Neighbouring-FDD-CellInformationItem-ExtIEs} } OPTIONAL,

...

}

Neighbouring-FDD-CellInformationItem-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

{ ID id-RestrictionStateIndicator				CRITICALITY ignore			EXTENSION RestrictionStateIndicator		PRESENCE optional	},

...

}

Neighbouring-GSM-CellInformation ::= ProtocolIE-Single-Container {{ Neighbouring-GSM-CellInformationIE }}

Neighbouring-GSM-CellInformationIE RNSAP-PROTOCOL-IES ::= {

{ ID id-Neighbouring-GSM-CellInformation	CRITICALITY ignore	TYPE	Neighbouring-GSM-CellInformationIEs	PRESENCE	mandatory }

}

Neighbouring-GSM-CellInformationIEs ::= SEQUENCE ( SIZE (1..maxNrOfGSMNeighboursPerRNC,...)) OF Neighbouring-GSM-CellInformationItem

Neighbouring-GSM-CellInformationItem ::= SEQUENCE {

cGI									CGI,

cellIndividualOffset				CellIndividualOffset	OPTIONAL,

bSIC								BSIC,

band-Indicator 						Band-Indicator,

bCCH-ARFCN							BCCH-ARFCN,

iE-Extensions						ProtocolExtensionContainer { { Neighbouring-GSM-CellInformationItem-ExtIEs} } OPTIONAL,

...

}

Neighbouring-GSM-CellInformationItem-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

...

}

Neighbouring-TDD-CellInformation ::= SEQUENCE ( SIZE (1..maxNrOfTDDNeighboursPerRNC,...)) OF Neighbouring-TDD-CellInformationItem

Neighbouring-TDD-CellInformationItem ::= SEQUENCE {

c-ID							C-ID,

uARFCNforNt						UARFCN,

frameOffset						FrameOffset			OPTIONAL,

cellParameterID					CellParameterID,

syncCase						SyncCase,

timeSlot						TimeSlot			OPTIONAL

-- This IE shall be present if Sync Case = Case1 -- ,

sCH-TimeSlot					SCH-TimeSlot			OPTIONAL

-- This IE shall be present if Sync Case = Case2 -- ,

sCTD-Indicator			SCTD-Indicator,

cellIndividualOffset			CellIndividualOffset	OPTIONAL,

dPCHConstantValue				DPCHConstantValue	OPTIONAL,

pCCPCH-Power					PCCPCH-Power			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { Neighbouring-TDD-CellInformationItem-ExtIEs} } OPTIONAL,

...

}

Neighbouring-TDD-CellInformationItem-ExtIEs RNSAP-PROTOCOL-EXTENSION ::= {

{ ID id-RestrictionStateIndicator				CRITICALITY ignore			EXTENSION RestrictionStateIndicator		PRESENCE optional	},

...

}

NrOfDLchannelisationcodes	::= INTEGER (1..8)

NrOfTransportBlocks			::= INTEGER (0..512)