3GPP TSG-RAN WG3 67	R3-100731

San Francisco, USA, February 22-26, 2010

Agenda item:	14

Source:	Kineto, Orange, Genband, NEC, AT&T, ZTE

Title:	Clarification to Iu UP Handling during Intra HNB-GW mobility

Document for:	Discussion and Approval

# Introduction

Intra-HNB-GW inter-HNB mobility has already been specified in Release 9. One of the main motivations of the specified mechanism was to ensure that intra HNB-GW mobility is transparent to the CN. However a critical component of this transparent solution i.e. the Iu User Plane for intra HNB-GW mobility was overlooked. This was identified in [1] and discussed during RAN3 #66 Bis, Valencia, and three possible solutions were proposed. It was also estimated in [1] that intra-HNB-GW mobility could not be supported without one of the proposed solutions.

This paper shows that the issue is already addressable by HNB-GW product implementation and clarified via a minor Stage 2 update [3].

# Discussion

The key concern noted in [1] was that while intra-HNB-GW mobility should be transparent to the CN, this would not be possible to accomplish in practice since the Iu UP termination points were restricted to the HNB and the CN. Therefore, special handling of the Iu UP is needed to ensure the desired transparency.

## Solution

The HNB-GW terminates the Iuh interface and presents itself as an RNC to the CN using the existing Iu interface (see Section 4.1.3 of [2]). Further, due to the widespread use of ATM transport for the core network and IP for consumer/enterprise broadband, the HNB-GW needs to provide IP-ATM interworking functionality. Therefore, the HNB-GW is already in a position to create and manage HNBHNB-GW and HNB-GWCN legs for Iu UP. In summary, the HNB-GW is also a legitimate termination point for Iu-UP, and allowing this functionality addresses the problem via a minor Stage 2 clarification. To reiterate, this was already proposed as a viable Solution 2 in [1]. Furthermore the source companies view this functionality as of minimal and incremental impact, if any, on the HNB-GW. The requisite Stage 2 clarification is specified in the accompanying CR [3].

# Proposal

This contribution clarifies that the issue identified in [1] is addressable via product implementations and a minor Stage 2 update. The source companies propose that the Stage 2 update in [3] is accepted as a simple solution with minimal impact on the HNB-GW.

# References

[1] R3-100079, Iu UP Handling during Intra HNB-GW mobility, RAN3 #66 Bis, Jan. 2010.

[2] TS 25.467 UTRAN architecture for 3G Home NodeB Stage-2.

[3] R3-100732, Iu UP Handling during Intra HNB-GW mobility CR 25.467