3GPP TSG-RAN3 Meeting #69 	R3-102497

# Madrid, Spain, 23rd  - 27th  August 2010

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.469 | CR | 058 | rev | 1 | Current version: | 9.2.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of HNB GW transparent HNB to HNB mobility in HNBAP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Siemens Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | HNB-HENB_mob_enh |  |  |  |  |  |  | Date: |  | 12/08/2010 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change: |  | To support HNB to HNB HO signalling for Cell DCH and Cell FACH mobility, in a way transparent to the HNB GW, additional messages are needed in HNBAP. These messages are very similar to the procedures already used in RNSAP and RANAP |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance the HNB Registration procedure and add procedure for Context Retrieve, Relocation, Relocation Commit and Relocation Complete. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Enhanced intra HNB GW mobility between HNBs will not occur in a transparent way to the HNB GW. If an alternative solution aimed at terminating RANAP in the HNB GW is chosen there will be a high number of duplicated functions between HNB GW and CN due to the fact HNB GW needs to terminate Iu-UP and RANAP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1, 8.2, 8.x, 9.1.3, 9.1.4, 9.1.x, 9.2.1, 9.2.15, 9.2.x, 9.3.2, 9.3.3, 9.3.4, 9.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

/* start of modification*/

# Contents

Foreword	6

1	Scope	7

2	References	7

3	Definitions and abbreviations	8

3.1	Definitions	8

3.2	Abbreviations	8

4	General	8

4.1	Procedure Specification Principles	8

4.2	Forwards and Backwards Compatibility	9

4.3	Specification Notations	9

5	HNBAP Services	9

6	Services expected from the Transport layer	9

7	Functions of HNBAP	10

8	HNBAP Procedures	10

8.1	Elementary Procedures	10

8.2	HNB Registration Procedure	10

8.2.1	General	10

8.2.2	Successful Operation	11

8.2.3	Unsuccessful Operation	11

8.2.4	Abnormal Conditions	12

8.3	HNB De-registration Procedure	12

8.3.1	Successful Operation (HNB Originated)	12

8.3.2	Successful Operation (HNB-GW Originated)	12

8.3.3	Abnormal Conditions	13

8.4	UE Registration	13

8.4.1	General	13

8.4.2	Successful Operation	13

8.4.3	Unsuccessful Operation	14

8.4.4	Abnormal Conditions	14

8.5	UE De-Registration	14

8.5.1	General	14

8.5.2	Successful Operation (HNB Originated)	14

8.5.3	Successful Operation (HNB-GW Originated)	15

8.5.4	Abnormal Conditions	15

8.6	Error Indication	15

8.6.1	General	15

8.6.2	Successful Operation	16

8.7	CSG Membership Update Procedure	16

8.7.1	Successful Operation	16

8.7.2	Abnormal Conditions	16

9	Elements for HNBAP Communication	17

9.1	Message Functional Definition and Content	17

9.1.1	General	17

9.1.2	Message Contents	17

9.1.2.1	Presence	17

9.1.2.2	Criticality	17

9.1.2.3	Range	17

9.1.2.4	Assigned Criticality	17

9.1.3	HNB REGISTER REQUEST	18

9.1.4	HNB REGISTER ACCEPT	18

9.1.5	HNB REGISTER REJECT	18

9.1.6	UE REGISTER REQUEST	18

9.1.7	UE REGISTER ACCEPT	19

9.1.8	UE REGISTER REJECT	19

9.1.9	HNB DE-REGISTER	19

9.1.10	UE DE-REGISTER	19

9.1.11	ERROR INDICATION	20

9.1.12	CSG MEMBERSHIP UPDATE	20

9.2	Information Element Definitions	20

9.2.0	General	20

9.2.1	Message Type	21

9.2.2	HNB Identity	21

9.2.3	HNB Location Information	21

9.2.4	Geographical Location	22

9.2.5	Geographical Coordinates	22

9.2.6	Altitude and Direction	22

9.2.7	Macro Coverage Information	22

9.2.8	IP Address	23

9.2.9	Context-ID	23

9.2.10	IMSI	23

9.2.11	LAC	24

9.2.12	RAC	24

9.2.13	SAC	24

9.2.14	PLMN-ID	24

9.2.15	Cause	25

9.2.16	Criticality Diagnostics	27

9.2.17	UE Identity	28

9.2.18	IMEI	28

9.2.19	TMSI (GSM-MAP)	28

9.2.20	LAI	28

9.2.21	Registration Cause	29

9.2.22	P-TMSI (GSM-MAP)	29

9.2.23	Routing Area Identification	29

9.2.24	UE Capabilities	29

9.2.25	Cell-ID	29

9.2.26	RNC-ID	30

9.2.27	CSG-ID	30

9.2.28	Backoff Timer	30

9.2.29	Mux Port Number	30

9.2.30	CSG Membership Status	30

9.2.31	HNB Cell Access Mode	30

9.3	Message and Information Element Abstract Syntax (with ASN.1)	31

9.3.0	General	31

9.3.1	Usage of private message mechanism for non-standard use	31

9.3.2	Elementary Procedure Definitions	32

9.3.3	PDU Definitions	35

9.3.4	Information Element Definitions	42

9.3.5	Common Definitions	49

9.3.6	Constant Definitions	50

9.3.7	Container Definitions	52

9.4	Message Transfer Syntax	56

10	Handling of unknown, unforeseen, and erroneous protocol data	56

10.1	General	56

10.2	Transfer Syntax Error	56

10.3	Abstract Syntax Error	56

10.3.1	General	56

10.3.2	Criticality Information	57

10.3.3	Presence Information	57

10.3.4	Not comprehended IE/IE group	58

10.3.4.1	Procedure Code	58

10.3.4.1A	Type of Message	58

10.3.4.2	IEs other than the Procedure Code and Type of Message	58

10.3.5	Missing IE or IE group	59

10.3.6	IEs or IE groups received in wrong order or with too many occurrences or erroneously present	60

10.4	Logical Error	61

10.5	Exceptions	61

Annex A (informative):	Change History	62

# Foreword

This Technical Specification (TS) has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the Home Node B Application Part (HNBAP) between the Home Node B (HNB)  and the Home Node B Gateway (HNB-GW). It fulfils the HNB- HNB-GW communication requirements specified in [3] and is defined over the Iuh – reference point. It provides control and management procedures between HNB and HNB-GW.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

	For a specific reference, subsequent revisions do not apply.

	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 25.401: "UTRAN Overall Description".

[2]	3GPP TS 25.413: “RANAP”

[3]	3GPP TS 25.467: “UTRAN architecture for 3G Home NodeB”

[4]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[5]	3GPP TR 25.921: "Guidelines and Principles for Protocol Description and Error Handling".

[6]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[7]	ITU-T Recommendation X.691 (07/2002): "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)".

[8]	ITU-T Recommendation X.680 (07/2002): "Information technology - Abstract Syntax Notation One (ASN.1): Specification of basic notation".

[9]	ITU-T Recommendation X.681 (07/2002): "Information technology - Abstract Syntax Notation One (ASN.1): Information object specification".

[10]	3GPP TS 25.331: “Radio Resource Control (RRC) Protocol Specification”

[11]	IETF RFC 4960 (09/2007): "Stream Control Transmission Protocol".

[12]	3GPP TS 23.003: "Numbering, addressing and identification".

[13]	IETF RFC 4282 (12/2005): "The Network Access Identifier".

[14]	Broadband Forum TR-069 Amendment 2, CPE WAN Management Protocol, Broadband Forum Technical Report, 2007.

[15]	3GPP TS 22.220: "Service requirements for Home NodeBs and Home eNodeBs".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

Elementary Procedure: HNBAP consists of Elementary Procedures (EPs). An Elementary Procedure is a unit of interaction between the HNB and HNB-GW. These EPs are defined separately and are intended to be used to build up complete sequences in a flexible manner. If the independence between some EPs is restricted, it is described under the relevant EP description. Unless otherwise stated by the restrictions, the EPs may be invoked independently of each other as stand alone procedures, which can be active in parallel.

An EP consists of an initiating message and possibly a response message. Two kinds of EPs are used:

-	Class 1: Elementary Procedures with response (success or failure).

-	Class 2: Elementary Procedures without response.

For Class 1 EPs, the types of responses can be as follows:

# Successful

-	A signalling message explicitly indicates that the elementary procedure successfully completed with the receipt of the response.

# Unsuccessful

-	A signalling message explicitly indicates that the EP failed.

-	On time supervision expiry (i.e. absence of expected response).

Class 2 EPs are considered always successful.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

EP	Elementary Procedure

ESN	Electronic Serial Number

HNB	Home Node B

HNB-GW	Home Node B Gateway

HNBAP	HNB Application Part

PDU	Protocol Data Unit

PER	Packed Encoding Rules

SAC	Service Area Code

SCTP				Stream Control Transmission Protocol
U-RNTI			UTRAN Radio Network Temporary Identifier

# 4	General

The protocol described in the present document is the protocol between HNB-GW and HNB.

## 4.1	Procedure Specification Principles

The principle for specifying the procedure logic is to specify the functional behaviour of the HNB & HNB-GW exactly and completely..

The following specification principles have been applied for the procedure text in clause 8:

-	The procedure text discriminates between:

1)	Functionality which "shall" be executed:

-	The procedure text indicates that the receiving node "shall" perform a certain function Y under a certain condition. If the receiving node supports procedure X but cannot perform functionality Y requested in the REQUEST message of a Class 1 EP, the receiving node shall respond with the message used to report unsuccessful outcome for this procedure, containing an appropriate cause value.

2)	Functionality which "shall, if supported" be executed:

-	The procedure text indicates that the receiving node "shall, if supported," perform a certain function Y under a certain condition. If the receiving node supports procedure X, but does not support functionality Y, the receiving node shall proceed with the execution of the EP, possibly informing the requesting node about the not supported functionality.

-	Any required inclusion of an optional IE in a response message is explicitly indicated in the procedure text. If the procedure text does not explicitly indicate that an optional IE shall be included in a response message, the optional IE shall not be included.

## 4.2	Forwards and Backwards Compatibility

The forwards and backwards compatibility of the protocol is assured by mechanism where all current and future messages, and IEs or groups of related IEs, include Id and criticality fields that are coded in a standard format that will not be changed in the future. These parts can always be decoded regardless of the standard version.

## 4.3	Specification Notations

For the purposes of the present document, the following notations apply:

Procedure	When referring to an elementary procedure in the specification the Procedure Name is written with the first letters in each word in upper case characters followed by the word  "procedure", e.g. HNB Registration procedure.

Message	When referring to a message in the specification the MESSAGE NAME is written with all letters in upper case characters followed by the word  "message", e.g. HNB REGISTRATION REQUEST message.

IE	When referring to an information element (IE) in the specification the Information Element Name is written with the first letters in each word in upper case characters and all letters in Italic font followed by the abbreviation "IE", e.g. HNB Identity IE.

Value of an IE	When referring to the value of an information element (IE) in the specification the "Value" is written as it is specified in subclause 9.2 enclosed by quotation marks, e.g. "Abstract Syntax Error (Reject)" or "Background ".

# 5	HNBAP Services

HNBAP provides the signalling service between the HNB and the HNB-GW that is required to fulfil the HNBAP functions in Clause 7.

# 6	Services expected from the Transport layer

Following service is expected from the transport layer:

-	reliable and in sequence delivery of Signalling data using SCTP[11].

# 7	Functions of HNBAP

The HNBAP has the following functions:

-	Registration

-	UE Registration

-	Error Handling. This function allows the reporting of general error situations, for which function specific error messages have not been defined.

These functions are implemented by one or several HNBAP elementary procedures described in the following clauses.

# 8	HNBAP Procedures

## 8.1	Elementary Procedures

In the following tables, all EPs are divided into Class 1 and Class 2 Procedures.

Table 1: Class 1

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| HNB Registration | HNB REGISTER REQUEST | HNB REGISTER ACCEPT | HNB REGISTER REJECT |
| UE Registration | UE REGISTER REQUEST | UE REGISTER ACCEPT | UE REGISTER REJECT |
| Relocation | RELOCATION REQUEST | RELOCATION RESPONSE | RELOCATION FAILURE |

Table 2: Class 2

| Elementary Procedure | Message |
| --- | --- |
| HNB De-Registration | HNB DE-REGISTER |
| UE De-Registration | UE DE-REGISTER |
| Error Indication | ERROR INDICATION |
| CSG Membership Update | CSG MEMBERSHIP UPDATE |
| Relocation Commit | RELOCATION COMMIT |
| Context Retrieve | CONTEXT RETRIEVE |
| Relocation Complete | Relocation Complete |
| Relocation Cancel | Relocation Cancel |

## 8.2	HNB Registration Procedure

### 8.2.1	General

The purpose of the HNB Registration Procedure is to register the HNB with the HNB-GW to enable the HNB-GW to provide service and core network connectivity for the HNB. This procedure shall be the first HNBAP procedure triggered after the SCTP association has become operational.

### 8.2.2	Successful Operation

Figure 1: HNB Register Procedure: Successful Operation

The HNB shall initiate this procedure by sending a HNB REGISTER REQUEST message whenever it needs to commence operations and requiring service from the HNB-GW.

If the HNB is CSG capable and operates in a Closed access mode as defined in [15], then it shall provide the CSG-ID IE and may provide the HNB Cell Access Mode IE within the HNB REGISTER REQUEST message.

If the HNB is supporting ETWS it shall provide the Service Area For Broadcast IE within the HNB REGISTER REQUEST message.

If the HNB operates in a Hybrid access mode as defined in [15], then it shall provide the CSG-ID IE and HNB Cell Access Mode IE within the HNB REGISTER REQUEST message.

If the HNB operates in an Open access mode as defined in [15], then it shall provide the HNB Cell Access Mode IE within the HNB REGISTER REQUEST message.

If the HNB supports the HNBAP Relocation procedures it shall provide the Enhanced Mobility Indicator IE to the HNB GW. The HNB shall report the Cell-ID IE for each of the HNB’s configured neighbour cells served by the same HNB GW.

If the registration is successful, the HNB-GW will respond with a HNB REGISTER ACCEPT message indicating acceptance and registration.

If the HNB REGISTER REQUEST includes the Enhanced Mobility Indicator IE and the Cell-ID IE for each of the HNB configured neighbour cells under the same HNB GW, the HNB GW shall respond with a HNB REGISTER ACCEPT including PLMN ID IE, Cell ID IE, Enhanced Mobility Indicator IE and CSG ID IE for each neighbour cell indicated in the HNB REGISTER REQUEST.

If the HNB-GW is capable of de-multiplexing, then the MuxPortNumber IE may be included in the HNB REGISTER ACCEPT message.

### 8.2.3	Unsuccessful Operation

Figure 2: HNB Register Procedure: Un-Successful Operation

If the HNB-GW cannot register the HNB, the HNB-GW will respond with a HNB REGISTER REJECT message.

Typical cause values are:

Radio Network Layer Cause:

-	Unauthorised Location

-	Unauthorised HNB

-	Overload

-	HNB Parameter Mismatch

-	Unspecified

If the HNB receives the “overload” cause code in the HNB REGISTER REJECT message, the HNB shall not retry registration to the same HNB-GW for at least the duration indicated in the Backoff Timer IE.

### 8.2.4	Abnormal Conditions

If the HNB-GW receives a duplicate HNB REGISTER REQUEST (i.e. for an already registered HNB identified by the unique HNB identity), then the new HNB REGISTER REQUEST shall override the existing registration and the handling of the new HNB REGISTER REQUEST is according to section 8.2.

## 8.3	HNB De-registration Procedure

### 8.3.1	Successful Operation (HNB Originated)

Figure 3: HNB De-register Procedure: Successful Operation

The HNB will initiate this procedure whenever it needs to terminate operations.

The HNB-GW shall clear all related resources associated with the HNB.

Typical cause values are:

Radio Network Layer Cause:

-	Normal

-	Unspecified

### 8.3.2	Successful Operation (HNB-GW Originated)

Figure 4: HNB De-register Procedure: Successful Operation

The HNB-GW will initiate this procedure whenever it needs to terminate operations with a HNB.

The HNB-GW shall clear all related resources associated with the HNB.

Typical cause values are:

Radio Network Layer Cause:

-	Overload

-	Unspecified

If the HNB receives the “overload” cause code in the HNB DE-REGISTER message, the HNB shall not retry registration to the same HNB-GW for at least the duration indicated in the Backoff Timer IE.

### 8.3.3	Abnormal Conditions

-

## 8.4	UE Registration

### 8.4.1	General

The UE Registration procedure provides means for the HNB to convey UE identification data to the HNB-GW in order to perform access control for the UE in the HNB-GW.  The UE Registration also establishes a UE specific context identifier to be used between HNB and HNB-GW. The procedure triggered when the UE attempts to access the  HNB via an initial NAS message and there is no context in the HNB allocated for that UE.

### 8.4.2	Successful Operation

Figure 5: UE Register Procedure: Successful Operation

The HNB shall initiate the procedure by sending a UE REGISTER REQUEST message to the HNB-GW including UE specific data.  The HNB shall always include the UE permanent identity (i.e. IMSI) if it does not serve a CSG cell or for registration of UEs without CSG capability except if the Registration Cause indicates “emergency call”.

The HNB-GW shall perform access control or membership verification in case of non-CSG UEs or non-CSG HNBs. If the HNB does not operate in closed access mode or access control is successful, the HNB shall respond with a UE REGISTER ACCEPT message including the Context-ID IE.

The UE REGISTER ACCEPT message may include the CSG Membership Status IE for the UE registering to the cell.

The HNB-GW shall not perform access control if the Registration Cause indicates “emergency call”.

### 8.4.3	Unsuccessful Operation

Figure 6: UE Register Procedure: Unsuccessful Operation

If the HNB-GW is unable to accept the UE registration it shall reject the procedure with a UE REGISTER REJECT message.

Typical Cause values:

Radio Network Layer Cause:

-	Invalid UE identity

-	UE not allowed on this HNB

-	HNB not registered

-	Unspecified

### 8.4.4	Abnormal Conditions

-

## 8.5	UE De-Registration

### 8.5.1	General

The purpose of the UE De-Registration Procedure is to disconnect a UE context in the HNB-GW or the HNB.

### 8.5.2	Successful Operation (HNB Originated)

Figure 7 UE De-Register Procedure: HNB Originated Successful Operation

The HNB shall initiate the procedure by sending a UE DE-REGISTER message. When receiving the UE DE-REGISTER message the HNB-GW shall release the resources associated with that UE including the Context-ID.

Typical Cause values:

Radio Network Layer Cause:

-	Connection with UE lost

-	UE RRC Release

-	Unspecified

-	UE relocated

### 8.5.3	Successful Operation (HNB-GW Originated)

Figure 8 UE De-Register Procedure: HNB-GW Originated Successful Operation

The HNB-GW shall initiate the procedure by sending a UE DE-REGISTER message. When receiving the UE DE-REGISTER message the HNB shall release the included Context-ID.

Typical Cause values:

Radio Network Layer Cause:

-	UE Registered in another HNB

-	Unspecified

-	UE not allowed on this HNB

### 8.5.4	Abnormal Conditions

--

## 8.6	Error Indication

### 8.6.1	General

The Error Indication procedure is initiated by either HNB or HNB-GW to report detected errors in one incoming message, provided they cannot be reported by an appropriate failure message.

### 8.6.2	Successful Operation

Figure 9 Error Indication HNB Originated, Successful Operation

Figure 10 Error Indication HNB-GW Originated, Successful Operation

## 8.7	CSG Membership Update Procedure

### 8.7.1	Successful Operation

Figure 11: CSG Membership Update: Successful Operation

The HNB-GW shall initiate this procedure whenever it needs to indicate a change of a UE's CSG membership status to the HNB.

### 8.7.2	Abnormal Conditions

-

## 8.8	Relocation Request Procedure

### 8.8.1	Successful Operation (HNB Originated)

Figure 11: Relocation Request Procedure: Successful Operation

The  Source HNB will initiate this procedure whenever it needs to initiate a handover operation towards a neighbour HNB cell for which the Enhanced Mobility Indicator IE is set to “supported”. It is FFS whether the RELOCATION REQUEST message can be used to initiate relocation procedures towards cells supporting a CSG ID different from the one supported by the source cell.

When receiving the RELOCATION REQUEST message the HNB GW shall check that the target HNB Enhanced Mobility Indicator IE is set to “supported”.

### 8.8.2	Successful Operation (HNB-GW Originated)

Figure 12: HNB De-register Procedure: Successful Operation

The HNB-GW will initiate this procedure towards the target HNB whenever it receives a successful RELOCATION REQUEST message from a source HNB and after verifying that the relocation to the target is allowed. This procedure shall trigger the establishment of a new UE-associated Signalling Connection between HNB-GW and HNB, which is directly mapped to the Iu Signalling Connection the RANAP message refers to.

In cases of UEs involved in Cell FACH mobility the U-RNTI IE shall be included in the RELOCATION REQUEST message.

## 8.9	Relocation Response Procedure

### 8.9.1	Successful Operation (HNB Originated)

Figure 13: Relocation Request Procedure: Successful Operation

The Target HNB will initiate this procedure after receipt of a successful Relocation Request message and when it can proceed with Handover operation.

### 8.9.2	Successful Operation (HNB-GW Originated)

Figure 14: HNB De-register Procedure: Successful Operation

The HNB GW will initiate this procedure after receipt of a successful Relocation Response message from the target HNB and when it can proceed with Handover operation.

## 8.10	Relocation Failure

## 8.10.1	Successful Operation (HNB GW Originated)

This is procedure is used to notify the source HNB of the failure of an already initiated Relocation procedure.

Figure 15: RELOCATION FAILURE procedure (HNB GW originated)

This procedure shall be triggered if the Enhanced Mobility Indicator IE stored at the HNB GW for the target HNB is set to “not-supported”.

It is FFS whether the procedure shall be triggered for cases of relocation to target cells supporting a CSG ID different from the CSG ID of the source cell.

### 8.10.2	Successful Operation (HNB Originated)

This is procedure is used to notify the HNB GW of the failure of an already initiated Relocation procedure.

Figure 16: RELOCATION FAILURE procedure (HNB originated)

It is FFS whether the procedure shall be triggered for cases of relocation to target cells supporting a CSG ID different from the CSG ID of the source cell.

## 8.11	Relocation Commit

### 8.11.1	Successful Operation (HNB Originated)

Figure 17: Relocation Commit Procedure - HNB Originated

## The Relocation Commit procedure is used by source HNB to execute the Relocation. The source HNB sends the RELOCATION COMMIT message to the HNB GW to request the HNB GW to inform the target HNB of proceeding with the Relocation.

The RELOCATION COMMIT message includes information enabling the target to resume Iu-UP procedures without the need of notify the CN (Frame Sequence Numbers, List of RFCIs, Data Volume Reports)

### 8.11.2	Successful Operation (HNB GW Originated)

Figure 18: Relocation Commit Procedure - HNB GW Originated

Upon reception of the RELOCATION COMMIT message from source HNB the HNB GW shall forward the RELOCATION COMMIT message to the target HNB to request the target HNB of proceeding with the Relocation.

The RELOCATION COMMIT message includes information enabling the target to resume Iu-UP procedures without the need of notify the CN (Frame Sequence Numbers, List of RFCIs, Data Volume Reports)

## 8.12	Relocation Complete

### 8.12.1	General

The purpose of the Relocation Complete procedure is to inform the HNB GW that the relocation at terget HNB has been completed.  The message indicates the RABs established at the target.

The procedure uses connection oriented signalling.

### 8.12.2	Successful Operation

# Figure 19: Enhanced Relocation Complete procedure. Successful operation.

## The HNB initiates the procedure by sending the RELOCATION COMPLETE REQUEST message to the HNB GW. The puprpose of this procedure is to inform the HNB GW of the successful completion of relocation at target HNB and of  relocated bearers.

## 8.13	Relocation Cancel

This procedure is used to cancel an ongoing enhanced relocation or an already prepared relocation.  The source HNB initiates this message towards target-HNB /HNB-GW to abort the ongoing relocation procedure and restore the resource with source-HNB.

### 8.13.1  Successful Operation (HNB Originated)

If the handover  procedure with UE fails source HNB shall initiate the RELOCATION CANCEL procedure towards HNB-GW to abort the ongoing relocation. The HNB shall indicate the reason for cancelling the relocation by means of an appropriate cause value.

Figure 21: Relocation Cancel Procedure - HNB Originated

### 8.13.2  Successful Operation (HNB-GW Originated)

When HNB-GW knows that the handover procedure with source-HNB failed, or after reception of RELOCATION TRANSFER from source HNB, it initiates the relocation cancel message to target-HNB.

At the reception of the RELOCATION CANCEL message, the target HNB shall remove any reference to, and release any resources previously reserved to the concerned UE context.

Figure 22: Relocation Cancel Procedure – HNB GW Originated

# 9	Elements for HNBAP Communication

## 9.1	Message Functional Definition and Content

### 9.1.1	General

Section 9.1 presents the contents of HNBAP messages in tabular format. The corresponding ASN.1 definition is presented in section 9.3. In case there is contradiction between the tabular format in section 9.1 and the ASN.1 definition, the ASN.1 shall take precedence, except for the definition of conditions for the presence of conditional IEs, where the tabular format shall take precedence.

NOTE:	The messages have been defined in accordance to the guidelines specified in [5].

For each message there is, a table listing the signalling elements in their order of appearance in the transmitted message.

### 9.1.2	Message Contents

#### 9.1.2.1	Presence

All information elements in the message descriptions below are marked mandatory, optional or conditional according to table 3

Table 3: Meaning of abbreviations used in HNBAP messages

| Abbreviation | Meaning |
| --- | --- |
| M | IE's marked as Mandatory (M) will always be included in the message. |
| O | IE's marked as Optional (O) may or may not be included in the message. |
| C | IE's marked as Conditional (C) will be included in a message only if the condition is satisfied. Otherwise the IE is not included. |

#### 9.1.2.2	Criticality

Each Information Element or Group of Information Elements may have criticality information applied to it.
Following cases are possible.

Table 4: Meaning of content within "Criticality" column

| Abbreviation | Meaning |
| --- | --- |
| – | No criticality information is applied explicitly. |
| YES | Criticality information is applied. This is usable only for non-repeatable IEs |
| GLOBAL | The IE and all its repetitions together have one common criticality information. This is usable only for repeatable IEs. |
| EACH | Each repetition of the IE has its own criticality information. It is not allowed to assign different criticality values to the repetitions. This is usable only for repeatable IEs. |

#### 9.1.2.3	Range

The Range column indicates the allowed number of copies of repetitive IEs/IE groups.

#### 9.1.2.4	Assigned Criticality

This column provides the actual criticality information as defined in subclause 10.3.2, if applicable.

### 9.1.3	HNB REGISTER REQUEST

This message is sent by the HNB to the HNB-GW to register the HNB at the HNB-GW.

Direction: 	HNB  HNB-GW

| PARAMETER | PRESENCE | RANGE | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1 |  | YES | reject |
| HNB Identity | M |  | 9.2.2 |  | YES | Reject |
| HNB Location Information | M |  | 9.2.3 |  | YES | Reject |
| PLMN-ID | M |  | 9.2.14 |  | YES | Reject |
| Cell-ID | M |  | 9.2.25 |  | YES | Reject |
| LAC | M |  | 9.2.11 |  | YES | Reject |
| RAC | M |  | 9.2.12 |  | YES | Reject |
| SAC | M |  | 9.2.13 |  | YES | Reject |
| CSG-ID | O |  | 9.2.27 |  | YES | Reject |
| Service Area For Broadcast | O |  | SAC 9.2.13 |  | YES | Ignore |
| HNB Cell Access Mode | O |  | 9.2.31 |  | YES | reject |
| Enhanced Mobility Indicator | O |  | 9.2.45 |  | YES | reject |
| Neighbour Cell List | O | 0 to <maxnoofNeighbours> |  |  | YES | reject |
| >PLMN-ID | M |  | 9.2.14 | PLMN ID of each HNB’s configured neighbour cell | YES | reject |
| >Cell-ID | M |  | 9.2.25 | Cell ID of each HNB’s configured neighbour cell | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofNeighbours | Maximum no. of neighbour cells associated to a given HNB. Value is 32. |

### 9.1.4	HNB REGISTER ACCEPT

This message is sent by the HNB-GW to the HNB as a successful response to a HNB REGISTER REQUEST message.

Direction: 	HNB-GW  HNB

| PARAMETER | PRESENCE | RANGE |  | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  |  | 9.2.1 |  | YES | reject |
| RNC-ID | M |  |  | 9.2.26 |  | YES | reject |
| Mux Port Number | O |  | 9.2.29 |  | The mux port number at which HNB-GW expects to receive multiplexed packets. | YES | Ignore |
| Neighbour Information List | O | 0 to maxnoofNeighbours |  |  |  | YES | reject |
| PLMN-ID | M |  | 9.2.14 |  | PLMN ID of HNB neighbour cell | YES | reject |
| Cell-ID | M |  | 9.2.25 |  | Cell ID of HNB neighbour cell | YES | reject |
| Enhanced Mobility Indicator | M |  | 9.2.45 |  | Enhanced mobility Indicator of configured neighbour cell | YES | reject |
| CSG-ID | M |  | 9.2.27 |  | CSG-ID of each configured neighbour cell | YES | reject |

### 9.1.5	HNB REGISTER REJECT

This message is sent by the HNB-GW to the HNB as unsuccessful response to a HNB REGISTER REQUEST message.

Direction: 	HNB-GW  HNB

| PARAMETER | PRESENCE | RANGE | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1 |  | YES | reject |
| Cause | M |  | 9.2.15 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.16 |  | YES | ignore |
| Backoff Timer | C-ifOverload |  | 9.2.28 |  | YES | reject |

| Condition | Explanation |
| --- | --- |
| IfOverload | This IE shall be present if the Cause IE is set to "Overload". |

### 9.1.6	UE REGISTER REQUEST

This message is sent by the HNB to the HNB-GW to register a UE at HNB-GW for service.

Direction: 	HNB  HNB-GW

| PARAMETER | PRESENCE | RANGE | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1 |  | YES | reject |
| UE Identity | M |  | 9.2.17 |  | YES | reject |
| Registration Cause | M |  | 9.2.21 |  | YES | ignore |
| UE Capabilities | M |  | 9.2.24 |  | YES | reject |

### 9.1.7	UE REGISTER ACCEPT

This message is sent by the HNB-GW to the HNB as a successful response to a UE REGISTER REQUEST message.

Direction: 	HNB-GW  HNB

| PARAMETER | PRESENCE | RANGE | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1 |  | YES | reject |
| UE Identity | M |  | 9.2.17 |  | YES | reject |
| Context-ID | M |  | 9.2.9 |  | YES | reject |
| CSG Membership Status | O |  | 9.2.30 |  | YES | reject |

### 9.1.8	UE REGISTER REJECT

This message is sent by the HNB-GW to the HNB as unsuccessful response to a UE REGISTER REQUEST message.

Direction: 	HNB-GW  HNB

| PARAMETER | PRESENCE | RANGE | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1 |  | YES | reject |
| UE Identity | M |  | 9.2.17 |  | YES | reject |
| Cause | M |  | 9.2.15 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.16 |  | YES | ignore |

### 9.1.9	HNB DE-REGISTER

This message is sent by the HNB to the HNB-GW or HNB-GW to HNB to deregister the HNB,

Direction: 	HNB  HNB-GW or HNB-GW  HNB

| PARAMETER | PRESENCE | RANGE | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1 |  | YES | reject |
| Cause | M |  | 9.2.15 |  | YES | ignore |
| Backoff Timer | C-ifOverload |  | 9.2.28 |  | YES | reject |

| Condition | Explanation |
| --- | --- |
| IfOverload | This IE shall be present if the Cause IE is set to "Overload". |

### 9.1.10	UE DE-REGISTER

This message is sent by the HNB to the HNB-GW or HNB-GW to HNB to de-register a UE.

Direction: 	HNB  HNB-GW, HNB-GW  HNB

| PARAMETER | PRESENCE | RANGE | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1 |  | YES | Reject |
| Context-ID | M |  | 9.2.9 |  | YES | Reject |
| Cause | M |  | 9.2.15 |  | YES | Ignore |

### 9.1.11	ERROR INDICATION

This message is sent by the HNB to HNB-GW or HNB-GW to HNB and is used to indicate that some errors have been detected.

Direction: 	HNB  HNB-GW, HNB-GW  HNB

| PARAMETER | PRESENCE | RANGE | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1 |  | YES | ignore |
| Cause | M |  | 9.2.15 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.16 |  | YES | ignore |

### 9.1.12	CSG MEMBERSHIP UPDATE

This message is sent by the HNB-GW to HNB to indicate CSG Membership changes to the HNB.

Direction: 	HNB-GW  HNB

| PARAMETER | PRESENCE | RANGE | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1 |  | YES | reject |  |
| Context-ID | M |  | 9.2.9 |  | YES | reject |
| CSG Membership Status | M |  | 9.2.30 |  | YES | reject |

### 9.1.13	RELOCATION REQUEST

This message is sent by the HNB-GW to HNB or from HNB to HNB-GW to initiate HNB handover.

Direction: 	HNB-GW  HNB, HNB HNB-GW

| PARAMETER | PRESENCE | RANGE | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1 |  | YES | reject |
| Context-ID | M |  | 9.2.9 |  | YES | reject |
| Source Cell Identity | M |  |  |  |  |  |
| >PLMN-ID | M |  | 9.2.14 | PLMN ID of source cell | YES | reject |
| >Cell-ID | M |  | 9.2.25 | Cell ID of source cell | YES | reject |
| Target Cell Identity | M |  |  |  |  |  |
| >PLMN-ID | M |  | 9.2.14 | PLMN ID of target cell | YES | reject |
| >Cell-ID | M |  | 9.2.25 | Cell ID of target cell | YES | reject |
| URNTI | O |  | 9.2.46 |  | YES | reject |
| Cause | M |  | 9.2.15 |  | YES | reject |
| Relocation Information Request | M |  | 9.2.32 |  | YES | Reject |
| RAB-TNL-parameters | M |  | 9.2.35d | UL RAB TNL information |  |  |
| Relocation parameters | O |  | 9.2.35a |  | YES | Ignore |

### 9.1.14	RELOCATION RESPONSE

This message is sent by the HNB-GW to HNB or from HNB to HNB-GW to acknowledge HNB handover.

Direction: 	HNB-GW  HNB, HNB-GW  HNB

| PARAMETER | PRESENCE | RANGE | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1 |  | YES | reject |  |
| Context-ID | M |  | 9.2.9 |  | YES | reject |
| Source Cell Identity | M |  |  |  |  |  |
| >PLMN-ID | M |  | 9.2.14 | PLMN ID of source cell | YES | reject |
| >Cell-ID | M |  | 9.2.25 | Cell ID of source cell | YES | reject |
| Target Cell Identity | M |  |  |  |  |  |
| >PLMN-ID | M |  | 9.2.14 | PLMN ID of target cell | YES | reject |
| >Cell-ID | M |  | 9.2.25 | Cell ID of target cell | YES | reject |
| RAB-TNL-parameters | M |  | 9.2.35d | UL RAB TNL information | YES | reject |
| Relocation Information Response | M |  | 9.2.33 |  | YES | reject |

### 9.1.15	RELOCATION FAILURE

This message is sent by the Target HNB to HNB-GW or from the HNB-GW to the Source HNB as unsuccessful response to a RELOCATION  REQUEST message.

Direction: 	HNB-GW  HNB, HNB-GW  HNB

| PARAMETER | PRESENCE | RANGE | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1 |  | YES | reject |
| Context-ID | M |  | 9.2.9 |  | YES | reject |
| Source Cell Identity | M |  |  |  |  |  |
| >PLMN-ID | M |  | 9.2.14 | PLMN ID of source cell | YES | reject |
| >Cell-ID | M |  | 9.2.25 | Cell ID of source cell | YES | reject |
| Target Cell Identity | M |  |  |  |  |  |
| >PLMN-ID | M |  | 9.2.14 | PLMN ID of target cell | YES | reject |
| >Cell-ID | M |  | 9.2.25 | Cell ID of target cell | YES | reject |
| Cause | M |  | 9.2.15 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.16 |  | YES | ignore |

### 9.1.16	RELOCATION COMMIT

This message is sent by the Source HNB to HNB-GW and by the HNB-GW to the Target HNB to execute the relocation,

Direction: 	HNB-GW  HNB, HNB-GW  HNB

| PARAMETER | PRESENCE | RANGE | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1 |  | YES | reject |
| Context-ID | M |  | 9.2.9 |  | YES | reject |
| Source Cell Identity | M |  |  |  |  |  |
| >PLMN-ID | M |  | 9.2.14 | PLMN ID of source cell | YES | Reject |
| >Cell-ID | M |  | 9.2.25 | Cell ID of source cell | YES | Reject |
| Target Cell Identity | M |  |  |  |  |  |
| >PLMN-ID | M |  | 9.2.14 | PLMN ID of target cell | YES | Reject |
| >Cell-ID | M |  | 9.2.25 | Cell ID of target cell | YES | Reject |
| Relocation parameters | M |  | 9.2.35a |  | YES | Ignore |
| Relocation Information | O |  | 9.2.34 |  | YES | reject |

### 9.1.17	RELOCATION COMPLETE

This message is sent by the HNB to inform the HNB that a relocation is completed.

Direction: HNB  HNB GW

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1 |  | YES | reject |
| Context-ID | M |  | 9.2.9 |  | YES | reject |
| Cell Identity | M |  |  |  |  |  |
| >PLMN-ID | M |  | 9.2.14 | PLMN ID of serving cell | YES | reject |
| >Cell-ID | M |  | 9.2.25 | Cell ID of serving cell | YES | reject |
| RABs Setup List | O |  |  |  | YES | reject |
| >RABs Setup Item IEs |  | 1 to <maxnoofRABs> |  |  | EACH | reject |
| >>RAB ID | M |  | 9.2.42 |  | - |  |
| >>Transport Layer Address | O |  | 9.2.43 | IPv6 or IPv4 address. | - |  |
| >>Iu Transport Association | O |  | 9.2.44 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRABs | Maximum no. of RABs for one UE. Value is 256. |

### 9.1.29 	RELOCATION CANCEL

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1 |  | YES | ignore |
| Transaction ID | M |  | 9.2.47 |  | – |  |
| Cause | M |  | 9.2.15 |  | YES | ignore |

### 9.1.21	CONTEXT RETRIEVE

This message is sent by the Target HNB to HNB-GW and by the HNB-GW to the Source HNB to initiate Cell FACH mobility procedures,

Direction: 	HNB-GW  HNB, HNB-GW  HNB

| PARAMETER | PRESENCE | RANGE | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1 |  | YES | reject |
| URNTI | M |  | 9.2.46 |  | YES | reject |
| Target Cell Identity | M |  |  |  |  |  |
| >PLMN-ID | M |  | 9.2.14 | PLMN ID of target cell | YES | reject |
| >Cell-ID | M |  | 9.2.25 | Cell ID of target cell | YES | reject |

## 9.2	Information Element Definitions

### 9.2.0	General

Section 9.2 presents the HNBAP IE definitions in tabular format. The corresponding ASN.1 definition is presented in section 9.3. In case there is contradiction between the tabular format in section 9.2 and the ASN.1 definition, the ASN.1 shall take precedence, except for the definition of conditions for the presence of conditional elements, where the tabular format shall take precedence.

When specifying information elements which are to be represented by bit strings, if not otherwise specifically stated in the semantics description of the concerned IE or elsewhere, the following principle applies with regards to the ordering of bits:

-	The first bit (leftmost bit) contains the most significant bit (MSB);

-	The last bit (rightmost bit) contains the least significant bit (LSB);

-	When importing bit strings from other specifications, the first bit of the bit string contains the first bit of the concerned information;

### 9.2.1	Message Type

Message Type IE uniquely identifies the message being sent. It is mandatory for all messages.

| IE/GROUP NAME | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Message Type |  |  |  |  |
| >Procedure Code | M |  | ENUMERATED ( HNB register, UE Register, UE De-Register, HNB De-Register Error Indication ,… , CSG Membership Update, Relocation Request, Relocation Response, Relocation Failure, Relocation Commit, Relocation Complete, Relocation Cancel, Context Retrieve) |  |
| >Type of Message | M |  | ENUMERATED (Initiating Message, Successful Outcome, Unsuccessful Outcome, Outcome) |  |

### 9.2.2	HNB Identity

HNB Identity IE  is sent from the HNB to the HNB-GW and identities the HNB.

| IE/GROUP NAME | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| HNB Identity |  |  | OCTET STRING (SIZE(1..255)) | See note below. |

Note:

The octet string shall take form of an Network Access Identifier(NAI) as defined in IETF RFC4282 [13]. The format of the HNB-Identity will be:

0<IMSI>@<realm>

Or

1<OUI>-<SerialNumber>@<realm>

Where <IMSI> is a 16 digit number coded as specified in 23.003[12] ;

and <OUI> and <SerialNumber>  are coded as specified in TR-069[14].

### 9.2.3	HNB Location Information

HNB Location Information IE is sent from the HNB to HNB-GW to provide information on the location of the HNB.

| IE/GROUP NAME | PRESENCE | RANGE | IE Type and Reference | Semantics Description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| HNB Location Information |  |  |  |  | - | - |
| > Macro Coverage Information | O |  | 9.2.7 |  | - | - |
| > Geographic Location | O |  | 9.2.4 |  | - | - |
| > HNB Internet Information | O |  | IP Address 9.2.8 |  | YES | reject |

### 9.2.4	Geographical Location

This IE identifies the Ellipsoid point with altitude as in ref [4].

| IE/GROUP NAME | PRESENCE | RANGE | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Geographical Coordinates | M |  | 9.2.5 |  |
| Altitude and Direction | M |  | 9.2.6 |  |

### 9.2.5	Geographical Coordinates

This IE contains the geographical coordinates of an ellipsoid point.

| IE/GROUP NAME | PRESENCE | RANGE | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Latitude Sign | M |  | ENUMERATED (North, South) |  |
| Degrees Of Latitude | M |  | INTEGER ( 0..223-1) | The IE value (N) is derived by this formula: N223 X /90  N+1 X being the latitude in degree (0°.. 90°) |
| Degrees Of Longitude | M |  | INTEGER ( -223..223-1) | The IE value (N) is derived by this formula: N224 X /360  N+1 X being the longitude in degree (-180°..+180°) |

### 9.2.6	Altitude and Direction

This IE contains the altitude and direction of an ellipsoid point.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Altitude and direction |  |  |  |  |
| >Direction of Altitude | M |  | ENUMERATED (Height, Depth) |  |
| >Altitude | M |  | INTEGER ( 0..215-1) | The relation between the value (N) and the altitude (a) in meters it describes is N a <N+1, except for N=215-1 for which the range is extended to include all grater values of (a). |

### 9.2.7	Macro Coverage Information

The Macro Coverage Information IE identifies the macro cell used by the HNB for location. This may be e.g. a GERAN Cell ID or a UTRAN Cell ID .

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Choice Cell Identifier |  |  |  |  |
| >UTRAN Cell ID Information |  |  |  |  |
| >>LAC | M |  | 9.2.11 |  |
| >>RAC | M |  | 9.2.12 |  |
| >>PLMN-ID | M |  | 9.2.14 |  |
| >>Cell-ID | M |  | 9.2.25 |  |
| >GERAN Cell ID Information |  |  |  |  |
| >>PLMN-ID | M |  | 9.2.14 |  |
| >>LAC | M |  | 9.2.11 | 0000 and FFFE not allowed. |
| >>CI | M |  | OCTET STRING (2) |  |

### 9.2.8	IP Address

This  IE defines a IP address.

| IE/GROUP NAME | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE IP Address |  |  |  |  |
| >Internet Address ipv4 | M |  | OCTET STRING(4) |  |
| >Internet Address ipv6 | M |  | OCTET STRING(16) |  |

### 9.2.9	Context-ID

Context ID IE uniquely identifies a particular UE in the HNB and HNB-GW. This unique Context ID is used for PS and CS domain.

| IE/GROUP NAME | PRESENCE | RANGE | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Context-ID |  |  | BIT STRING(24) |  |

### 9.2.10	IMSI

The IMSI is used to uniquely identify a UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IMSI |  |  | OCTET STRING (SIZE (3..8)) | - digits 0 to 9, encoded 0000 to 1001, - 1111 used as filler digit, two digits per octet, - bit 4 to 1 of octet n encoding digit 2n-1 - bit 8 to 5 of octet n encoding digit 2n -Number of decimal digits shall be from 6 to 15 starting with the digits from the PLMN identity. When the IMSI is made of an odd number of digits, the filler digit shall be added at the end to make an even number of digits of length 2N. The filler digit shall then be consequently encoded as bit 8 to 5 of octet N. |

### 9.2.11	LAC

This element is used to identify a Location Area.

| IE/GROUP NAME | PRESENCE | RANGE | IE Type and Reference | Semantics Description |  |
| --- | --- | --- | --- | --- | --- |
| LAC |  |  | OCTET STRING(2) |  |

### 9.2.12	RAC

This element is used to identify a Routing Area within a Location Area. It is used for PS services.

| IE/GROUP NAME | Presence | Range | IE Type and Reference | Semantics Description |  |
| --- | --- | --- | --- | --- | --- |
| RAC |  |  | OCTET STRING(1) |  |

### 9.2.13	SAC

The SAC identifies the Service Area.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SAC |  |  | OCTET STRING (SIZE(2)) |  |

### 9.2.14	PLMN-ID

The PLMN-ID identifies a Public Land Mobile Network.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN-ID |  |  | OCTET STRING (SIZE (3)) | - digits 0 to 9, encoded 0000 to 1001, - 1111 used as filler digit, two digits per octet, - bits 4 to 1 of octet n encoding digit 2n-1- bits 8 to 5 of octet n encoding digit 2n -The PLMN identity consists of 3 digits from MCC followed by either - a filler digit plus 2 digits from MNC (in case of 2 digit MNC) or - 3 digits from MNC (in case of a 3 digit MNC). |

### 9.2.15	Cause

Cause IE indicates the reason for a particular error event for the HNBAP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group |  |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED ( overload, unauthorised-Location, unauthorised-HNB, HNB Parameter mismatch, Invalid UE identity, UE not allowed on this HNB, UE unauthorised, Connection with UE lost, UE RRC Release, HNB not registered, unspecified, Normal, UE relocated, UE Registered in another HNB, ..., ) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Transport Resource Unavailable, Unspecified, ...) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Unspecified, Abstract Syntax Error (Falsely Constructed Message), HNBAP Relocation not allowed, ...) |  |
| >Misc |  |  |  |  |
| >>Misc Cause | M |  | ENUMERATED (Processing Overload, Hardware Failure, O&M Intervention, Unspecified, ...) |  |

The meaning of the different cause values is described in the following table. Cause values for information ‘not valid’ indicates that the information is not valid in the context that it was received.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Overload | The HNB-GW cannot handle the HNB due to overload. |
| Unauthorised-Location | The HNB-GW cannot register the HNB because the location information provided is not valid. |
| Unauthorised-HNB | The HNB-GW cannot register the HNB because its supplied information is not considered valid |
| HNB Parameter Mismatch | The HNB-GW cannot register the HNB because of mismatch in parameters between HNB and HNB-GW |
| Invalid UE identity | The UE ID supplied for UE registration is not valid |
| UE not allowed on this HNB | The UE is identified as not being allowed to use services on this HNB. (temporary rejection) |
| UE not authorised | The UE is identified as not being allowed to use services on a HNB (permanent rejection). |
| Connection with UE lost | The connection to a Registered UE has been lost |
| UE RRC release | The Registered UE’s RRC is released (e.g. due to IMSI detach NAS procedure) |
| HNB not registered | The HNB is not registered on this gateway |
| Unspecified | Sent when none of the above cause values applies but still the cause is Radio Network layer related. |
| UE relocated | The UE has been relocated to a neighbouring cell |
| UE Registered in another HNB | The UE has already registered in another HNB on same HNB-GW. |
| HNBAP Relocation not allowed | The HNBAP relocation towards the target HNB is either not supported or not allowed |
| Normal | No error has occurred |

### 9.2.16	Criticality Diagnostics

The Criticality Diagnostics IE is sent by the RNC or the CN when parts of a received message have not been comprehended or were missing, or if the message contained logical errors. When applicable, it contains information about which IEs were not comprehended or were missing.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Criticality Diagnostics |  |  |  |  |
| >Procedure Code | O |  | INTEGER (0..255) | Procedure Code is to be used if Criticality Diagnostics is part of Error Indication procedure, and not within the response message of the same procedure that caused the error |
| >Triggering Message | O |  | ENUMERATED (initiating message, successful outcome, unsuccessful outcome) | The Triggering Message is used only if the Criticality Diagnostics is part of Error Indication procedure. |
| >Procedure Criticality | O |  | ENUMERATED(reject, ignore, notify) | This Procedure Criticality is used for reporting the Criticality of the Triggering message (Procedure). |
| Information Element Criticality Diagnostics |  | 0 to <maxnoof errors> |  |  |
| >IE Criticality | M |  | ENUMERATED(reject, ignore, notify) | The IE Criticality is used for reporting the criticality of the triggering IE. The value ’ignore’ shall not be used. |
| >IE ID | M |  | INTEGER (0..65535) | The IE Id of the not understood or missing IE |
| >Type of Error | M |  | ENUMERATED(not understood, missing, …) |  |

| Range bound | Explanation |
| --- | --- |
| Maxnooferrors | Maximum no. of IE errors allowed to be reported with a single message. The value for maxnooferrors is 256. |

### 9.2.17	UE Identity

This is a unique identifier for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE UE Identity |  |  |  |  |
| > IMSI |  |  | 9.2.10 |  |
| > TMSI and LAI (GSM-MAP) |  |  |  |  |
| >>TMSI | M |  | 9.2.19 |  |
| >>LAI | M |  | 9.2.20 |  |
| >P- TMSI and RAI (GSM-MAP) |  |  |  |  |
| >>P-TMSI | M |  | 9.2.22 |  |
| >>RAI (GSM-MAP) | M |  | 9.2.23 |  |
| > IMEI |  |  | 9.2.18 |  |
| > ESN (DS-41) |  |  | BIT STRING (SIZE (32)) |  |
| > IMSI (DS-41) |  |  | OCTET STRING (SIZE (5..7)) |  |
| >IMSI and ESN (DS-41) |  |  |  |  |
| >> ESN (DS-41) | M |  | BIT STRING(SIZE (32)) |  |
| >> IMSI (DS-41) | M |  | OCTET STRING (SIZE (5..7)) |  |
| >TMSI (DS-41) |  |  | OCTET STRING (SIZE (2..17)) |  |

### 9.2.18	IMEI

This IE contains an International Mobile Equipment Identity.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IMEI |  |  | BIT STRING(60) |  |

### 9.2.19	TMSI (GSM-MAP)

This IE contains a Temporary Mobile Subscriber Identity, used towards a GSM-MAP type of PLMN.

| Information Element/Group name | Presence | Range | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| TMSI (GSM-MAP) |  |  | BIT STRING (32) | The first/leftmost bit of the bit string contains the most significant bit of the TMSI. |

### 9.2.20	LAI

Location Area Identification identifies uniquely a location area for a GSM-MAP type of PLMN.

| Information Element/Group name | Presence | Range | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PLMN-ID | M |  | 9.2.14 |  |
| LAC | M |  | 9.2.11 |  |

### 9.2.21	Registration Cause

Indicates if a UE registration is for an emergency call.

| Information Element/Group name | Presence | Range | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Registration cause |  |  | Enumerated { emergency call, normal, ...} |  |

### 9.2.22	P-TMSI (GSM-MAP)

This IE contains a Packet Temporary Mobile Subscriber Identity (P-TMSI), used towards a GSM-MAP type of PLMN.

| Information Element/Group name | Presence | Range | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| P-TMSI |  |  | BIT STRING (32) | The first/leftmost bit of the bit string contains the most significant bit of the P-TMSI. |

### 9.2.23	Routing Area Identification

Identifies uniquely a routing area for a GSM-MAP type of PLMN.

| Information Element/Group name | Presence | Range | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LAI | M |  | 9.2.20 |  |
| RAC | M |  | 9.2.12 |  |

### 9.2.24	UE Capabilities

Identifies UE capabilities and release.

| Information Element/Group name | Presence | Range | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Capabilities |  |  |  |  |
| >Access Stratum Release Indicator | M |  | Enumerated { R99, Rel-4, Rel-5, Rel-6, Rel-7, Rel-8-and-beyond, …} | Values as defined in [10] |
| >CSG Capability | M |  | Enumerated {CSG capable, Not CSG capable, …} | Indicates a CSG capable UE. |

### 9.2.25	Cell-ID

Identifies uniquely a cell within a PLMN.

| Information Element/Group name | Presence | Range | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell-ID |  |  | INTEGER (0..268435455) | This information element identifies a cell uniquely within UTRAN. |

### 9.2.26	RNC-ID

Uniquely identifies the HNB-GW towards the CN on a particular Iu interface.

| Information Element/Group name | Presence | Range | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RNC-ID |  |  | INTEGER (0..65535) | Values greater than 4095 are extended (16bit) RNC Ids. |

### 9.2.27	CSG-ID

Indicates the CSG-ID of a particular HNB, as defined in [12]

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSG-ID |  |  | BIT STRING (SIZE(27)) |  |

### 9.2.28	Backoff Timer

The Backoff Timer IE indicates in seconds the minimum duration for which the HNB registration shall not be retried.

| Information Element/Group name | Need | Multi | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Backoff Timer |  |  | INTEGER (0..3600) | Value ‘0’ indicates no specified time. |

### 9.2.29	Mux Port Number

The mux port number on which the HNB-GW expects the multiplexed packets from the HNB

| Information Element/Group name | Presence | Range | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Mux port number |  |  | INTEGER (1024..65535) |  |

### 9.2.30	CSG Membership Status

This element indicates the Membership status of the UE to a particular CSG.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSG Membership Status | M |  | ENUMERATED (member, not-member,…) |  |

#### 9.2.31	HNB Cell Access Mode

This information element indicates whether the cell of the HNB operates in a Closed, Hybrid, or Open Access mode as defined in [15].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| HNB Cell Access Mode | M |  | ENUMERATED(closed, hybrid, open, …) |  |

#### 9.2.32	Relocation Information Request

This IE is part of the HNBAP Relocation Request message and is sent transparently to the HNB GW between source and target HNB during HNBAP relocation. The parameter contains information for the Relocation procedure as defined in [2].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Relocation Information Request |  |  | BIT STRING | The content is defined in ref. [2], 9.1.81 |

#### 9.2.33	Relocation Information Response

#### This IE is part of the HNBAP Relocation Response message and is sent transparently to the HNB GW between target and source HNB during HNBAP relocation.  The parameter contains information for the Relocation procedure as defined in [2].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Relocation Information Response |  |  | BIT STRING | The content is defined in ref. [2], 9.1.82 |

#### 9.2.34	Relocation Information

This IE is sent by source HNB to the target HNB transparently to the HNB GW during Relocation procedures.  The parameter contains information for the Enhanced Relocation procedure as defined in [2].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Relocation Information |  |  | BIT STRING | The content is defined in ref. [2], 9.1.43 |

### 9.2.35a	Relocation Parameters

This IE provides additional information for relocation not included in the Relocation Information

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Relocation Parameters List |  | 1 to <maxnoofRABs> |  |  |
| >RAB ID | M |  | 9.2.42 |  |
| >RAB Data Volume Report | O |  | 9.2.21 |  |
| >UP Information | O |  | 9.2.23 |  |
| Location Reporting | O |  | 9.2.26 |  |
| Trace Information | O |  | 9.2.29 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRABs | Maximum no. of RABs for one UE. Value is 256. |

### 9.2.35b	RAB Data Volume Report

This information element indicates the data volume (octets) and the time when it was counted that was unsuccessfully transmitted over the radio interface in the DL direction for a RAB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAB Data Volume Report |  |  |  |  |
| >RAB Data Volume Report Item IEs |  | 1 to <maxnoofVol> |  |  |
| >>Unsuccessfully Transmitted DL Data Volume | M |  | INTEGER (0..232-1) | Unit is Octet. |
| >>Data Volume Reference | O |  | INTEGER (0..255) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofVol | Maximum no. of reported data volume for one RAB. Value is 2. |

### 9.2.35c	UP Information

Provides information to enable the UP operation to be continued from the target HNB.

| Information Element/Group name | Presence | Range | Type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Frame Sequence Number UL | M |  | Frame Sequence Number 9.2.36 |  |
| Frame Sequence Number DL | M |  | Frame Sequence Number 9.2.36 |  |
| PDU Type 14 Frame Sequence Number UL | M |  | PDU Type 14 Frame Sequence Number 9.2.38 |  |
| PDU Type 14 Frame Sequence Number DL | M |  | PDU Type 14 Frame Sequence Number 9.2.38 |  |
| PDU Type | M |  | 9.2.35 |  |
| Rate Control RFCIs | O |  | Bit String (1..62) | Bit 1 indicates RFCI0, Bit 2 indicates RFCI1 etc. |
| RFCI List |  | 1 to <maxnoofRFCIs> |  |  |
| >RFCI | M |  | Integer(0..63) |  |
| >CHOICE LI |  |  |  |  |
| >>Length8 |  |  |  |  |
| >>>SubFlow List8 |  | 1 to <maxnoofSubFlows> |  |  |
| >>>>Subflow Length8 | M |  | Integer (0..255) |  |
| >>Length16 |  |  |  |  |
| >>>SubFlow List16 |  | 1 to <maxnoofSubFlows> |  |  |
| >>>>Subflow Length16 | M |  | Integer (0..65635) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRFCIs | Maximum no. of RFCIs for one RAB. Value is 63. |
| maxnoofSubFlows | Maximum no of Subflows for one RFCI. Value is 7 |

### 9.2.35d	RAB TNL Parameters

This IE provides additional information for relocation not included in the Relocation Information

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| TNL Parameter List |  | 1 to <maxnoofRABs> |  |  |
| >CN Domain Indicator | M |  | 9.2.48 |  |
| >RAB ID | M |  | 9.2.42 |  |
| > Transport Layer Address | M |  | 9.2.35e |  |
| > Iu Transport association | M |  | 9.2.35f |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofRABs | Maximum no. of RABs for one UE. Value is 256. |

#### 9.2.35e	Transport Layer Address

.This element provides the transport IP address of the TNL association. Only IP Address format shall be used in HNBAP messages.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transport Layer Address | M |  | BIT STRING (1..160, …) |  |

#### 9.2.35f	Iu Transport Association

This element is used to associate the RAB and the corresponding transport bearer.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Choice Iu Transport Association |  |  |  |  |
| >GTP TEID |  |  | OCTET STRING (4) |  |
| >Binding ID |  |  | OCTET STRING (4) | If the Binding ID includes an UDP port, the UDP port is included in octet 1 and 2. The first octet of the UDP port field shall be included in the first octet of the Binding ID. |

#### 9.2.35	PDU Type

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU Type | M |  | ENUMERATED(PDU Type 0, PDU Type 1…) |  |

#### 9.2.36	Frame sequence number

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Frame sequence number UL | M |  | INTEGER (0..15) |  |

### 9.2.38	PDU type 14 frame sequence number

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PDU type 14 frame sequence number | M |  | INTEGER (0..3) |  |

### 9.2.42	RAB ID

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| RAB ID | M |  | BIT STRING (8) |  |

### 9.2.43	Transport Layer Address

This information element is an IP address to be used for the user plane transport.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Transport Layer Address | M |  | BIT STRING (1..160, …) |  |

### 9.2.44	Iu Transport Association

This element is used to associate the RAB and the corresponding transport bearer. For the CS domain this information element is either the Binding ID to be used in Transport Network Control Plane signalling during set up of the transport bearer or it contains the UDP port in order to allow transport bearer establishment without ALCAP. In PS domain this information element is the GTP Tunnel Endpoint Identifier.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Choice Transport Association |  |  |  |  |
| >GTP TEID |  |  | OCTET STRING (4) |  |
| >Binding ID |  |  | OCTET STRING (4) | If the Binding ID includes an UDP port, the UDP port is included in octet 1 and 2. The first octet of the UDP port field shall be included in the first octet of the Binding ID. |

### 9.2.45	Enhanced Mobility Indicator

### This information element specified if enhanced HNBAP mobility is supported by the HNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Enhanced Mobility Indicator | M |  | ENUMERATED(supported, not-supported, …) |  |

### 9.2.46	U-RNTI

U-RNTI (UTRAN RNTI) is the UE identifier allocated by the HNB or HNB GW to be used over the radio interface. It is unique in the HNB GW domain. One UE Context has one unique U-RNTI value allocated in the HNB GW.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| U-RNTI |  |  | INTEGER(0.. 4294967295) |  |

#### 9.2.47	Transaction ID

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Transaction ID Value | M |  | INTEGER (0..127) |  |

#### 9.2.48	CN Domain Indicator

Indicates the CN domain from which the message originates or to which the message is sent.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CN Domain Indicator | M |  | ENUMERATED (CS domain, PS domain) |  |

/* end of modification */

ASN.1 code to be provided after final agreement

/* end of change */