3GPP TSG-RAN WG3 Meeting #74   															R3-112898

San Francisco, USA, November 14 - 18, 2011

Title: 	Discussion on the potential problem and optimization of mobile relay

Source: 	ZTE

Agenda item:	16.3

Document for:	Discussion

# Introduction

In RAN3 #73bis meeting, the mobile relay deployment scenarios of interest are discussed. For example, the spectrum model, multi-RAT, in-band or out-band, etc. The potential characteristics of the scenario are also listed, such as high speed, known trajectory, UE at pedestrian speed, etc. Based on these scenarios, we discuss the potential problems and optimizations of mobile relay.

# Discussion

The basic idea of mobile relay is to install the relay node (RN) in high speed vehicles to reduce handover and TAU overhead, as shown in Fig. 1. In this section, we discuss the potential problems of the R10 relay to support the mobile relay. The optimizations based on the mobile relay characteristics are also discussed.

Fig. 1 Scenarios for mobile relay

## The identities of network entities

For the mobile relay scenario, the RN cells’ ECGI, PCI and TAC naming rule of R10 may have problems. The details are as follows.

- RN cell’s ECGI and PCI

According to R10 specification [1], the RN cell’s ECGIs are configured by RN OAM which consists of the eNB ID and the cell identity parts. The eNB ID part of RN is the same as its serving DeNB. In the mobile relay scenario, when the RN handovers between DeNBs, the eNB ID part of the ECGIs of RN’s cells changes with the serving DeNBs. The cell identity part may also need to be changed to avoid collision with the target DeNB’s cells. Similarly, the PCI of the RN cell may collide with the neighboring eNBs along the RN movement trace. To avoid the potential interference, the PCI of RN cell may also need change.

However, when the PCI and ECGI of RN cells change, a group of active UEs within the high speed vehicle need to perform re-establishment so as to re-connect to the RN cells. For the active UEs, the simultaneous re-establishments of UEs may lead to the random access resource congestion. The higher random access delay may consequently influence the UE’s service quality.

To solve this problem, one possible approach is network planning. For example, the PCIs and ECGIs specially reserved for the mobile relay are assigned to the RN cells. The RN keep this PCI and ECGI unchanged when handover between DeNBs. However, the eNB ID of RN becomes different from the serving DeNB. It may impact the current R10 procedures where the DeNB decides whether to forward the S1/X2 signaling based on the ECGI. Another approach is to allow the RN OAM configure new PCIs and ECGIs whenever the RN handovers. The RN may inform the served UE about the change before the new PCIs and ECGIs are applied. The UE can adapt to this change without performing the re-establishment. However, this approach requires the air-interface change.

- RN’s TAC

When the RN moves in high speed, it is possible that the RN handover to a target DeNB which belongs to different TA. In this case, if the downlink data for the idle UE camped on the RN arrives, the UE MME may not send the paging message to DeNB according to the supported TAs info of DeNB. Hence, the RN could not receive the paging message forwarded by the DeNB and fails to page the camped idle UE’s.

Similar to the RN cell’s PCI and ECGI change, it seems that this problem could be solved by network planning, for example, all the DeNBs along the moving trace of RN are configured with the same TAs with RN. However, the moving trace may be too long for network planning. The famous high speed rail in China, Beijing-Shanghai rail for example, is up to 1318 kilometers and across seven provinces. It brings the complexity and inflexibility for the network planning.

## Relay architecture

The RN’s S-GW and P-GW are located in the serving DeNB in R10 relay architecture. In the mobile relay scenario, the serving DeNB of RN changes with the RN’s handover. As we can see, the change of serving DeNBs leads to the RN’s P-GW and S-GW change. However, the R10 specification does not support the P-GW change during the handover procedure. On the other hand, suppose the RN’s P-GW and S-GW is unchanged and continuously located in the initial serving DeNB1 of RN, as shown in Fig. 2, the DeNB1 has to keep the RN and UE context until the RN leaves the active mode. In this case, when RN handovers to DeNB2, the data packets for UE need to be forwarded between the old and new serving DeNBs, which leads to extra forwarding delay.

Fig. 2 RN local GW unchanged scenario

This problem can be resolved by allowing the change of P-GW during the handover of RN. How to support this mechanism needs to be discussed together with the SA. Besides, in [2], four different RN architectures have been proposed and discussed. It is recommended to re-evaluate them and discuss their capabilities of supporting mobile relay.

## Signaling procedure

One of the benefits of mobile relay is to replace the frequent handovers of a group of UEs by the individual handover of RN. However, to support the group mobility of served UEs, the handover procedure for the RN needs to be tailored for this purpose. The potential problems are listed as follows:

- When RN completes the handover from source DeNB to target DeNB, new signaling is necessary for the target DeNB to notify the UE MME/GW so that the UE’s S-GW can correctly forward the data packet. As shown in Fig. 3, when RN handover from DeNB 1 to DeNB2, the UE’s MME/GW should be notified to switch the downlink path to the DeNB2 and then forward the packets correspondingly.

Fig. 3 Downlink path switch for UEs

- Since the DeNB act as the S1 signaling proxy and is responsible for the mapping of S1AP IDs. When the serving DeNB of RN changes, the new serving DeNB and the MME of UE should update the S1AP IDs correspondingly for the following S1 signaling exchange.

- Both X2-based and S1-based handover procedure may be used for normal UE. When UE handover, the X2-based handover is used if the X2 connectivity is available between the source and target eNBs and the MME is unchanged. The S1-based handover procedure is used when the X2-based handover cannot be used. When it comes to RN handover, the RN may perform X2-based or S1-based handover. Meanwhile, some UEs served by the RN meet the requirement for X2-based handover while others need to perform the S1-based handover. Under this combined handover scenarios, there is no existing signaling procedures for reference.

Proposal 1: The network identity, relay architecture and signaling procedure of R10 are hard to meet the requirements of mobile relay. The problems should be identified and solved to support the mobile relay.

## Potential Optimization

- System information:  In R10 relay, the SIB4~SIB8 is broadcasted by the RN to assist the UE cell re-selection.  Since the UE moves rapidly together with RN, the idle UE is hardly to re-select other neighboring cells other than the RN cell. It can be observed that the SIB4~SIB8 is almost useless to the UE and consume the limited power of UE. So it is recommended to reduce or even eliminate the SIB4~SIB8 broadcast by the RN.

- Measurement: in mobile relay, the measurement could be divided into two categories, UE measurement, RN measurement. For the RN measurement, since the RN may only handover to the DeNB in the forwarding direction, it is better to configure the measurement of DeNB cells according to the RN’s movement trace. For the UE measurement, since the UE and RN are relatively stationary, it seems that the UE measurement can be reduced for power efficiency. However, when the trains arrive at a station and stay for a while, the UE may handover to macro eNB for better service quality. Based on this observation, it is suggested to configure different measurement pattern for usage scenario, such as enable the measurement when the train is stop, and vice versa.

- Neighbor cell relation: In mobile relay scenario, the passing-by eNB does not need to keep the info of moving RN. However, the moving RN needs to keep the neighbor DeNB info for handover purpose. As we can see, only the DeNB in the forwarding direction of RN is needed by the moving RN. Based on this observation, the DeNB may filter the neighbor cell info and forward only the neighboring DeNB cells in the forwarding direction to the RN. It is also suggested to prohibit the moving RN to send the eNB configuration update message to reduce the signaling overhead.

Proposal 2: Based on the mobile relay characteristic, the potential optimizations should be considered to improve the efficiency of mobile relay.

# Conclusion

In this contribution, the problems of R10’s relay to support mobile relay, such as network identities, architecture and signalling procedure are presented. The potential optimizations based on the mobile relay characteristics are also discussed. Several proposals have been outlined below, providing guidance to the future mobile relay study.

Proposal 1: The network identity, relay architecture and signaling procedure of R10 are hard to meet the requirements of mobile relay. The problems should be identified and solved to support the mobile relay.

Proposal 2: Based on the mobile relay characteristic, the potential optimizations should be considered to improve the efficiency of mobile relay.

# Reference

- 3GPP TS 36.300 V10.4.0, 3GPP Technical Specification Group Radio Access Network E-UTRA and E-UTRAN overall description Stage 2

- 3GPP TR 36.806 V9.0.0, 3GPP Technical Specification Group Radio Access Network E-UTRA Relay architectures for E-UTRA