3GPP TSG RAN3 Meeting #85bis	R3-142352

Shanghai, China, 06 – 10 Oct, 2014

Agenda item:       11

Source:	NEC

Title:	CSI Exchange in Inter-eNB CoMP

Document for:		Discussion and Decision

1. Introduction

In RAN3#85, a discussion on exchange of CSI was initiated based on [1].

In the following we provide our views on CSI and eRNTP exchange, as well as text proposals containing the required message structures.

2. Discussion

2.1	CSI exchange

One eNB can send CSI report pertaining to one or more of its users (UEs) to a neighboring eNB. For each UE the CSI that the eNB sends can comprise of:

- CQI: up-to 2 CQIs, each including a wideband component and possible sub-band differential components

- RI: one wideband component

We note that the PMI was excluded from the CSI exchange report [1]. The justification for this exclusion was to minimize the overhead and the fact that PMI can depend on fast changing channel information, thus reducing its utility over non ideal backhaul with a higher latency. However, in the absence of PMI the use of RI is limited. Indeed, any rank greater than 1 will convey only 2 CQI(s), one for each of the two codewords. No further information about the (average) spatial directions seen by that user can be deduced by the eNB receiving the report. As a result, reporting the RI should be made optional. Moreover, the eNB requesting the CSI report should be able to specify whether or not it would like to receive RI reports. Similarly, the eNB requesting the CSI reports should be able to specify whether or not it requires subband specific CQI reports. This can be achieved by setting a bit (in the measurement request) to be 0 if rank is not requested and 1 otherwise. Another bit can be set to 0 if subband CQIs are not requested and 1 otherwise.

One of the key goals of CSI exchange was to facilitate centralized RRM [2]. In a scenario with centralized RRM, the central node receiving the CSI reports should be able to keep track of the CSI information received for each particular UE, over all the received CSI reports. This can be achieved by including a UE identifier in each CSI report for each UE whose CSI is conveyed in that report.

Moreover, for each CSI in the report, the CSI process configuration information should be included in order to convey the conditions under which the CSI was measured by the UE. This configuration information includes non-zero power CSI-RS information and IMR   information (comprising of the subframe indices and zero-power CSI-RS information). Since this configuration is anyway informed to the UE via higher layer signaling, the same signaling can be reused to convey the configuration to the neighboring eNB. Another way of conveying this configuration information is through a look-up-table. A look-up-table mapping an index to each distinct applied CSI process configuration can be constructed for each eNB. Here, by an applied CSI process we mean a process that is used by at-least one served UE to measure its CSI. Such a table can be exchanged among neighbors first and from then on the configuration information can be exchanged via indices.

2.2	eRNTP exchange

Our view on eRNTP exchange is captured in a corresponding text proposal.

We note that the RNTP (i.e., downlink (DL) power restriction) for the first subframe is always conveyed. If no information about the DL power restriction on any subsequent subframe is conveyed, then the one conveyed for the first subframe can be assumed to remain static (i.e., applicable over subsequent subframes).

3. Conclusion

In this contribution we discussed the necessary X2 message to support CSI and eRNTP exchange for  inter-eNB CoMP and presented corresponding text proposals.

References

[1] RP-141032, ``New Work Item on Enhanced Signaling for Inter-eNB CoMP”.

[2] R1-141206, “Signaling Considerations for Inter-eNB CoMP”, NEC

Text Proposal:

9.1.2.1	LOAD INFORMATION

This message is sent by an eNB to neighbouring eNBs to transfer load and interference co-ordination information.

Direction: eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| Cell Information | M |  |  |  | YES | ignore |
| >Cell Information Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 | Id of the source cell | – | – |
| >>UL Interference Overload Indication | O |  | 9.2.17 |  | – | – |
| >>UL High Interference Information |  | 0 .. <maxCellineNB> |  |  | – | – |
| >>>Target Cell ID | M |  | ECGI 9.2.14 | Id of the cell for which the HII is meant | – | – |
| >>>UL High Interference Indication | M |  | 9.2.18 |  | – | – |
| >>Relative Narrowband Tx Power (RNTP) | O |  | 9.2.19 |  | – | – |
| >>ABS Information | O |  | 9.2.54 |  | YES | ignore |
| >>Invoke Indication | O |  | 9.2.55 |  | YES | ignore |
| >>Intended UL-DL Configuration | O |  | ENUMERATED(sa0, sa1, sa2, sa3, sa4, sa5, sa6,…) | One of the UL-DL configurations defined in TS 36.211 [10]. The UL subframe(s) in the indicated configuration is subset of those in SIB1 UL-DL configuration. This IE applies to TDD only. | YES | ignore |
| >>Extended UL Interference Overload Info | O |  | 9.2.67 | This IE applies to TDD only. | YES | ignore |
| >>Enhanced Relative Narrowband Tx Power (eRNTP) | O |  | 9.2.x2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

#### 9.1.2.11	RESOURCE STATUS REQUEST

This message is sent by an eNB1 to a neighbouring eNB2 to initiate the requested measurement according to the parameters given in the message.

Direction: eNB1  eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| eNB1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | ignore |
| Registration Request | M |  | ENUMERATED(start, stop, …) | A value set to “stop”, indicates a request to stop all cells measurements. | YES | reject |
| Report Characteristics | O |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates measurement object the eNB2 is requested to report. First Bit = PRB Periodic, Second Bit = TNL load Ind Periodic, Third Bit = HW Load Ind Periodic, Fourth Bit = Composite Available Capacity Periodic, this bit should be set to 1 if at least one of the First, Second or Third bits is set to 1, Fifth Bit = ABS Status Periodic, Xth Bit = UE-CSI Periodic. Other bits shall be ignored by the eNB2. | YES | reject |
| Cell To Report |  | 1 |  | Cell ID list for which measurement is needed | YES | ignore |
| >Cell To Report Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 |  | – | – |
| Reporting Periodicity | O |  | ENUMERATED(1000ms, 2000ms, 5000ms,10000ms, …) |  | YES | ignore |
| Partial Success Indicator | O |  | ENUMERATED(partial success allowed, ...) | Included if partial success is allowed | YES | ignore |
| CSI Measurement Report type | O |  | BITSTRING (SIZE(2)) | Each position in the bitmap indicates the type of CSI measurement to report. First bit=Rank, Second bit=subband CQI. | YES | ignore |
| ((Reporting Periodicity of CSI Measurement Report | O |  | ENUMERATED(5ms, 10ms, 20ms,40ms, 80ms, aperiodic, ...) | Periodicity for CSI Measurement Report Periodic | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value “stop”. |

#### 9.1.2.14	RESOURCE STATUS UPDATE

This message is sent by eNB2 to neighbouring eNB1 to report the results of the requested measurements.

Direction: eNB2  eNB1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | ignore |
| eNB1 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB1 | YES | reject |
| eNB2 Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by eNB2 | YES | reject |
| Cell Measurement Result |  | 1 |  |  | YES | ignore |
| >Cell Measurement Result Item |  | 1 .. <maxCellineNB> |  |  | EACH | ignore |
| >>Cell ID | M |  | ECGI 9.2.14 |  |  |  |
| >>Hardware Load Indicator | O |  | 9.2.34 |  |  |  |
| >>S1 TNL Load Indicator | O |  | 9.2.35 |  |  |  |
| >>Radio Resource Status | O |  | 9.2.37 |  |  |  |
| >>Composite Available Capacity Group | O |  | 9.2.44 |  | YES | ignore |
| >>ABS Status | O |  | 9.2.58 |  | YES | ignore |
| >> UE-CSI Report | O |  | 9.2.x1 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

### 9.2.x1	UE-CSI Report

This IE provides UE-CSI information for a set of UEs served by eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE subset CSI Report |  | 1 .. <maxUEsubsetCSIReport> |  |  |
| >C-RNTI | M |  | BIT STRING (SIZE (16)) | ID of the UE served by the cell in eNB2. Defined in TS 36.331. |
| >UE-CSI process information |  | 1 .. <maxUE-CSIprocess> |  |  |
| >>Rank Indicator | O |  | BIT STRING (SIZE (3)) | The rank indicator is present only if it is requested in the associated request. Cf. TS 36.213 [7.2.3]. |
| >>Wideband CQI For Codeword 0 | M |  | BIT STRING (SIZE (4)) | Cf. TS 36.213 [7.2.3]. |
| >>Wideband CQI For Codeword 1 | O |  | BIT STRING (SIZE (4)) | Cf. TS 36.213 [7.2.3]. |
| >>Subband CQI List |  | 0..<maxCQISubbands > |  | 0 indicates no subband CQI, which is always chosen if associated request does not want subband CQI |
| >>>Subband CQI for codeword 0 | O |  | BIT STRING (SIZE (2)) | Cf. TS 36.213 [7.2.3]. |
| >>>Subband CQI for codeword 1 | O |  | BIT STRING (SIZE (2)) | Cf. TS 36.213 [7.2.3]. |
| >>UE-CSI process Configuration information | M |  | FFS | CSI process configuration information. |

| Range bound | Explanation |
| --- | --- |
| maxUEsubsetCSIReport | Maximum UE subset size for which UE-CSI can be reported. The value is 32. |
| maxUE-CSIProcess | Maximum number of CSI processes per-UE. The value is 4. |
| maxCQISubbands | Maximum number of subbands for UE CQI reporting. The value is 28. |

### 9.2.19	Relative Narrowband Tx Power (RNTP)

This IE provides an indication on DL power restriction per PRB in a cell and other information needed by a neighbour eNB for interference aware scheduling.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| RNTP Per PRB | M |  | BIT STRING (6..110, …) | Each position in the bitmap represents a nPRB value (i.e. first bit=PRB 0 and so on), for which the bit value represents RNTP (nPRB), defined in TS 36.213 [11]. Value 0 indicates "Tx not exceeding RNTP threshold". Value 1 indicates "no promise on the Tx power is given". This IE is used to indicate DL power restriction per PRB for the first subframe. In case the DL power restriction is static, the indicated DL power restriction is maintained over the subsequent subframes. | – | – |
| RNTP Threshold | M |  | ENUMERATED (-∞, -11, -10, -9, -8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, …) | RNTPthreshold is defined in TS 36.213 [11]. | – | – |
| Number Of Cell-specific Antenna Ports | M |  | ENUMERATED (1, 2, 4, …) | P (number of antenna ports for cell-specific reference signals) defined in TS 36.211 [10] | – | – |
| P_B | M |  | INTEGER (0..3, …) | PB is defined in TS 36.213 [11]. | – | – |
| PDCCH Interference Impact | M |  | INTEGER (0..4, …) | Measured by Predicted Number Of Occupied PDCCH OFDM Symbols (see TS 36.211 [10]). Value 0 means "no prediction is available". | – | – |
| RNTP List | O | 2 .. <maxnoofSubframes> |  | The first item in the list corresponds to the second subframe, the second to the third subframe, and so on. The DL power restrictions conveyed for the first subframe and the ones conveyed for the subsequent subframes in the list, are together applied repeatedly. |  |  |
| >RNTP Per PRB Subframe-Specific | M |  | BIT STRING (6..110, …) | Each position in the bitmap represents a nPRB value (i.e. first bit=PRB 0 and so on), for which the bit value represents RNTP (nPRB), defined in TS 36.213 [11]. Value 0 indicates "Tx not exceeding RNTP threshold". Value 1 indicates "no promise on the Tx power is given". This IE is used to indicate DL power restriction per PRB for the corresponding subframe. |  |  |
| >Starting SFN | M |  | INTEGER (0..1023, …) | Number of the first system frame from which the RNTP Per PRB Per Subframe IE is valid. |  |  |
| >Starting Subframe Index | M |  | INTEGER (0..9, …) | Index of the first subframe from which the RNTP Per PRB Per Subframe IE is valid. |  |  |

| Range bound | Explanation |
| --- | --- |
| maxSubframe | Maximum number of subframes. Value is 40. |