3GPP TSG-RAN WG3 Meeting #88	R3-150992

Fukuoka, Japan, 25 – 29 May, 2015

Agenda item:       11.2.1

Source:	NEC

Title:	On the subband definition in CSI Signalling

Document for:		Discussion and Decision

1. Introduction

In RAN3#87bis, after fruitful discussions on the exchange of CSI, four options for subband indexing have been shortlisted in [1].  In this document we discuss some issues with these candidates and then present our preference in a text proposal.

2. Discussion

2.1	Conveying both Periodic and Aperiodic reports

A desirable feature that should be supported by a CSI signaling scheme is the exchange of both periodic and aperiodic CSI reports in the same X2 message, possibly in a combined (or merged) form. In the absence of this feature, i.e., when the sending eNB is forced to choose either the periodic or the aperiodic CSI report obtained under a CSI process of some user, the sending eNB will have to drop available CSI reports. This would be unfortunate given that precious over-the-air signaling resources have already been spent in acquiring these reports and that these reports can together convey more CSI that either individual one.

Then, a key problem that needs to be overcome in order to send both aperiodic and periodic subband reports together in the same X2 message, is described next. In particular, the associated reference wideband reports that are used to compute the constituent subband parts of the aperiodic and periodic CSI reports, respectively, can be different. Indeed, the wideband rank indicators (RIs) that are determined by the UE under the configured aperiodic mode and the configured periodic mode can be different. Similarly, the wideband CQIs determined by the UE under the configured periodic mode and the configured aperiodic mode are also more likely to be different. Furthermore, each subband CQI determined by the UE under the configured mode can be reported by it as a differential value with respect to a corresponding reference CQI. For example, suppose aperiodic feedback mode 2-2 and periodic feedback mode 2-1 are configured. Then, under the aperiodic mode 2-2 the  UE will report (over PUSCH) one absolute wideband CQI (per codeword) as well as one subband CQI (per codeword) for the selected best-M feedback as a differential value (using 2 bits) with respect to the wideband CQI corresponding to that codeword.   On the other hand, under the periodic 2-1 mode the UE will report (over PUCCH) one wideband CQI (per codeword), with the wideband CQI of the second codeword being reported as a differential value with respect to the wideband CQI of the first one. In addition the UE will report one subband CQI for the first codeword  and the second codeword subband CQI as a differential value with respect to the subband CQI of the first one.

Therefore it becomes clear that we have two alternatives to overcome this issue:

Alternative-1: Provision to include two separate sets of RIs and wideband CQIs in the X2 message whenever that message contains both aperiodic and periodic subband reports. This will allow simple forwarding of both aperiodic and periodic reports in the same X2 message. In this context, we note that the structure in option-A does not provide for including two separate sets of wideband components, which forces the sending eNB to merge the wideband components and use a common reference for both aperiodic and periodic subband information. This defeats the purpose of forwarding separate aperiodic and periodic subband reports in the same X2 message. Another issue in including the report type IE (specifying periodic or aperiodic report) within the list of subbands, is that ambiguity can be introduced when certain aperiodic and periodic reports are combined.

Alternative-2: Always merge separate sets of wideband components into one wideband component that will also be used as a common reference. In this case it is logical to merge the respective subband information as well, and upon doing so there is no need to indicate the type of the subband CSI report. This view is aligned to options B, C and D. However, it prevents simple forwarding of both aperiodic and periodic reports in the same X2 message.

In order to obtain the merits of both the aforementioned alternatives we propose a simple extension of option-D. This extension is presented in two versions with the second one being a more bit-efficient version of the first.

The benefits of this proposal are as follows:

- It allows for simple forwarding of both aperiodic and periodic reports in the same X2 message. In fact it allows for forwarding of multiple aggregated aperiodic or periodic CSI reports (from a UE under a CSI process) or their combinations in the same X2 message.

- The parameter NRBDL together with the conveyed subband size IE defines the subband partition, which corresponds to one of those defined in TS36.213.

- It also allows for merging an aperiodic and a periodic report (or merging combinations of multiple periodic and/or multiple aperiodic CSI reports) without introducing any new subband definitions. This is because for a given total number of RBs (or PRBs), NRBDL, the subband size in aperiodic mode 2-* is exactly half of that of the aperiodic mode 3-* as well as periodic mode 2-*.  Thus, in order to combine such reports we can use the subbands defined by the smaller subband size and convey (possibly processed) CQIs for them.

- Other implementation based processing of the short-term CSI is also supported.

The first version is relatively straightforward. Some of its features are however worth pointing out:

- For each CSI process we can convey up-to maxReferenceType reports. The Reference Type IE can be ENUMERATED  for instance as periodic or aperiodic. Alternatively, the Reference Type IE can simply be dropped.

- The subband CQIs are conveyed sequentially in the increasing order of subband indices.  Then, in case the subband CQI IE for a subband is not conveyed, the receiving eNB must use the CQI conveyed for the last preceding subband. The CQI for the first subband must always be included. This feature can significantly save overhead by avoiding redundancy. Note that when a UE is configured in the aperiodic mode 2-*, it selects and reports indices for M out of N subbands. However, only one CQI (per codeword) is reported by it for all the M selected subbands. Therefore, it is beneficial that redundancy is avoided in reporting such CQIs.

- The Subband Index IE is optional. In case this IE is not included then the subband CQI information for each one of the total number of subbands is conveyed.  Recall that the parameter NRBDL together with the conveyed subband size IE defines the subband partition, thereby conveying the total number of subbands N.

We now consider a more bit efficient second version in which the sub band selection is conveyed by means of a combinatorial index, which was briefly introduced in [2].

Here, under each Reference Type IE, the parameter NRBDL together with the conveyed subband size IE defines the subband partition, thereby conveying the total number of subbands N. Also the number of subbands for which subband CQI is conveyed, M, is determined by the size of the Subband CQI List IE.

The combinatorial index, r, is defined based on TS36.213 (section 7.2.1) as follows:

  - The positions of the M selected subbands is conveyed using a combinatorial index r defined as

where N denotes the total number of subbands and the set , () contains the M sorted subband indices and  is the extended binomial coefficient, resulting in unique label .

To illustrate, consider first the case when NRBDL=110 and the Reference Type IE is set to be aperiodic. Then, we have two possibilities for subband selection. The first one is when the configured mode is 2-* in which case the subband size is 4 so that N=28 and here M=6. On the other hand, for aperiodic mode 3-* the subband size is 8 so that N=14 and here M=14. Similar argument applies to all other modes as well. It is also apparent that there is significant flexibility in aggregating several different reports under a Reference Type, as long as the subband partition is a valid one, i.e., corresponds to a one defined in TS36.213. Since the maximum value of N=28 (when NRBDL=110   and subband size is 4) we represent the combinatorial index using a bit string of length 26. This allows us to convey any possible selection choice of subbands from the maximum of 28 subbands.

3. Conclusion

In this contribution we discussed the necessary X2 message to support CSI exchange for inter-eNB CoMP and presented our views on subband indexing along with corresponding text proposals.

References

[1] R1-150839, “Summary of offline discussion on subband definition”, Nokia Networks

[2] R1-150604, “CSI Exchange for Inter-eNB CoMP”, NEC

#### 9.1.2.14	RESOURCE STATUS UPDATE

This message is sent by eNB2 to neighbouring eNB1 to report the results of the requested measurements.

Direction: eNB2 → eNB1.

### 9.2.aa	UE-CSI Report (Version-1)

This IE provides UE-CSI information for a set of UEs served by eNB2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSI Report per UE |  | 1 .. <maxUEReport> |  |  |
| >UE ID | M |  | BIT STRING (SIZE(16)) | ID of the UE served by the cell in eNB2. |
| >CSI Report per CSI Process |  | 1 .. <maxCSIProcess> |  |  |
| >> UE-CSI process Configuration index | M | FFS | FFS | CSI process configuration information. |
| >> Reference type per CSI process |  | 1.. <maxReferenceTypes> |  |  |
| >>>Reference Type | M |  | ENUMERATED |  |
| >>>RI | M |  | INTEGER (1..8, …) | Defined in TS 36.213 [11]. |
| >>>Wideband CQI | M |  | 9.2.bb |  |
| >>> Subband Size | M |  | ENUMERATED (2, 3, 4, 6, 8, …) | Corresponds to a value of subband size k defined in TS 36.213 [11] for the system bandwidth. Ignored if the Subband Index corresponds to the highest frequency subband. |
| >>>Subband CQI List |  | 1 .. <maxSubband> |  |  |
| >>>>Subband CQI | O |  | 9.2.cc | If this IE is not present, the CQI is identical to the one provided for the last preceding subband. This IE is always present for the first subband in the list. |
| >>>>Subband Index | O |  | INTEGER (0..27, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxUEReport | Maximum number of UE measurement reports. Value is 128. |
| maxCSIProcess | Maximum number of CSI processes. The value is 4. |
| maxReferenceTypes | Maximum types of CSI reports. The value is FFS. |
| maxSubband | Maximum number of subbands. The value is 28 |

### UE-CSI Report (Version-2)

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CSI Report per UE |  | 1 .. <maxUEReport> |  |  |
| >UE ID | M |  | BIT STRING (SIZE(16)) | ID of the UE served by the cell in eNB2. |
| >CSI Report per CSI Process |  | 1 .. <maxCSIProcess> |  |  |
| >> UE-CSI process Configuration index | M | FFS | FFS | CSI process configuration information. |
| >> Reference type per CSI process |  | 1.. <maxReferenceTypes> |  |  |
| >>>Reference Type | M |  | ENUMERATED |  |
| >>>RI | M |  | INTEGER (1..8, …) | Defined in TS 36.213 [11]. |
| >>>Wideband CQI | M |  | 9.2.bb |  |
| >>> Subband Size | M |  | ENUMERATED (2, 3, 4, 6, 8, …) | Corresponds to a value of subband size k defined in TS 36.213 [11] for the system bandwidth. Ignored if the Subband Index corresponds to the highest frequency subband. |
| >>> combinatorial index | M |  | BITSTRING (SIZE(26)) | As defined in TS36.213 (section 7.2.1). The indices of the subbands in the list are indicated by this combinatorial index. Subband CQIs are sorted in the order of increasing frequency (increasing subband indices). |
| >>>Subband CQI List |  | 1 .. <maxSubband> |  |  |
| >>>>Subband CQI | O |  | 9.2.cc | If this IE is not present, the CQI is identical to the one provided for the last preceding subband. This IE is always present for the first subband in the list. |

| Range bound | Explanation |
| --- | --- |
| maxUEReport | Maximum number of UE measurement reports. Value is 128. |
| maxCSIProcess | Maximum number of CSI processes. The value is 4. |
| maxReferenceTypes | Maximum types of CSI reports. The value is FFS. |
| maxSubband | Maximum number of subbands. The value is 28 |