3GPP TSG RAN WG3#89	R3-151459

Beijing, China, August 24th - 28th, 2015

Agenda Item:	23.1

Source:	CATT

Title:	Discussion on the coordination of discovery resource

Document for:	Discussion and Approval

1	Introduction

During RAN2#89bis meeting, some agreements on D2D discovery for inter-carrier and inter-PLMN are reached [1] which may have impact on RAN3. In this contribution, we make some analysis on this topic based on the conclusions of RAN2 and give our proposals accordingly.

2	Discussion

About intra-PLMN scenario, the agreement reached in RAN2#89bis meeting is as follows:

The option to configure a UE via RRC signalling to transmit discovery in another carrier can be allowed.  The RRC signalling can be used to configure either Type 1 or Type 2 discovery configuration for non-primary carrier.

Also, for coordinated inter-PLMN scenario, the agreement is as follows:

If the network has inter-PLMN information then the network should have the option to configure the UE similar to the intra-PLMN case.

For both intra-PLMN and inter-PLMN scenarios, if the serving cell of the UE and the non-primary carrier are in the same eNB, then the serving cell could acquire the discovery configuration on non-primary carrier easily, it has no impact on RAN3.However, if the serving cell and the non-primary carrier belong to two neighbor eNBs respectively, discovery resource coordination between the two eNBs is needed. Considering the discovery configuration difference between Type1 and Type2B, they will be analyzed individually.

Type1 discovery resource allocation:

For Type 1 discovery resource allocation, two alternatives could be used to inform the discovery resource pool of non-primary cell to the serving cell:

- Alternative 1: through OAM configuration

- Alternative 2: through X2 interface

For the above two options, it is more flexible for the non-primary carrier to inform its Type1 discovery configuration to its neighbor cells through X2 interface. However, considering the discovery configuration will not change frequently, alternative 1 is simpler and has less impact on specification. So, alternative 1 is preferred.

Proposal 1: For Type1 discovery, OAM should configure the discovery resource pool of neighbor cells to support the scenario of discovery transmission in non-serving cell.

Type 2B discovery resource allocation:

Type 2B is UE-specific resource allocation, thus it is more complex for the serving cell to configure the discovery configuration of non-primary carrier. Two options are feasible.

- Option1: Serving eNB requests the dedicated discovery transmission resource towards its neighbor cell every time, and then configures it to its served connected-mode UE via dedicated signaling, as depicted in Figure 1.

Figure1.  Option 1 for Type 2B

In this option, a new X2 procedure (i.e.Sidelink Resource Request/Response) is introduced to fetch the UE dedicated Type2B discovery resource. Since the inter-eNB neighbour cell doesn’t have any context of the UE, the serving cell needs to include UE context in the Sidelink discovery resource request procedure. If there is no X2 interface between the serving cell and the neighbor cell, this information needs to be transferred by MME through S1 signalling.

- Option 2: Serving eNB acquires the discovery resources pool pre-allocated by its neighbor cells and the serving cell is responsible for the resource allocation within the pool for active UEs.

Figure2.  Option 2 for Type 2B

In this option, a prose function supported cell should coordinate with its neighbor cells the allocated discovery resource pool dedicated to the receive node. Similar to type 1 discovery configuration, there are two alternatives to transfer the resource pool. One is through OAM configuration and the other is through X2/S1 interface.

For the above two options, the main difference is that the Type2B discovery resources are allocated by the non-primary cell which support prose function in option 1 and it is allocated by the serving cell within the preconfigured discovery resource pool in option 2. More comparisons could be seen in the following table.

Table1.   Comparison of Option1 and Option2

|  | Option1 | Option2 |
| --- | --- | --- |
| Resource usage | High No waste of discovery resource. | Medium When prose function supported cell allocate a discovery resource pool for its neighbor cell, it could not estimate how many UEs would request for D2D resources and the assigned resources pool may be bigger than it needs. |
| latency | High X2 backhaul latency is introduced | low |
| UE context needs to be transferred to non-primary cell | Yes The non-primary cell needs to acquire the UE context to assign the discovery resource | No |

According to the analysis, Option 2 may degrade the resource usage, it is simpler and there is no need for the non-primary cell to fetch UE context. Hence, we prefer Option 2. Similar with type1 discovery configuration, OAM solution is preferred.

Proposal 2: For Type2B discovery, the non-primary carrier should inform one resource pool used for Type2B discovery to its neighbor cells through OAM.

3	Conclusion

Based on the above analysis, we have the following proposals:

Proposal 1: For Type1 discovery, OAM should configure the discovery resource pool of neighbor cells to support the scenario of discovery transmission in non-serving cell.

Proposal 2: For Type2B discovery, the non-primary carrier should inform one resource pool used for Type2B discovery to its neighbor cells through OAM.

4        Reference

[1] ProSe_notes_RAN2_89bis_Bratislava_R2-15xxxx_23-04_20-15