3GPP TSG-RAN WG3 #90	                                              R5-152837

Anaheim, USA, 16th – 20th November 2015

Agenda item:	20.1.3

Source: 	Samsung

Title :	Text proposal for data fowarding

Document for:	Approval

- Introduction

During the meeting, whether to capture the data forwarding in case of bearer type change was discussed. This document provides a compromised text proposal for data forwarding description based on the baseline CR [1] and the reference in [2][3][4].

Note: only sentence marked by yellow is the text proposal to baseline CR.

- Text Proposal

Start of the 1st modification

10.1.2.8.x1	eNB to MeNB change

The eNB to MeNB change procedure is used to transfer context data from a source eNB to a target MeNB that adds an SeNB during the handover.

Figure 10.1.2.8.x1-1: eNB to MeNB change

Figure 10.1.2.8.x1-1 shows an example signaling flow for eNB to MeNB change:

1.	The source eNB starts the handover procedure by initiating the X2 Handover Preparation procedure.

2.	The target MeNB sends SeNB Addition Request to the target SeNB.

3.	The target SeNB replies with SeNB Addition Request Acknowledge. If data forwarding is needed, the target SeNB provides forwarding addresses to the target MeNB.

4.	The target MeNB includes within the Handover Request Acknowledge message a transparent container to be sent to the UE as an RRC message to perform the handover which also includes the SCG configuration, and may also provide forwarding addresses to the source eNB. Either direct data forwarding or indirect data forwarding is used for SCG bearer. Only indirect data forwarding is used for split bearer.

5.	The source eNB triggers the UE to apply the new configuration.

6/7.	The UE synchronizes to the target MeNB and replies with RRCConnectionReconfigurationComplete message.

8.	The UE synchronizes to the target SeNB

9.	If the RRC connection reconfiguration procedure was successful, the target MeNB informs the target SeNB.

10/11.	Data forwarding from the source eNB takes place.

12-15.	The target MeNB initiates the S1 Path Switch procedure.

NOTE:	If new UL TEIDs of the S-GW are included, the target MeNB performs MeNB initiated SeNB Modification procedure to provide them to the target SeNB.

16.	The target MeNB initiates the UE Context Release procedure towards the source eNB.

10.1.2.8.x2	Inter-MeNB handover without SeNB change

Inter-MeNB handover without SeNB change is used to transfer context data from a source MeNB to a target MeNB while the context at the SeNB is kept.

Figure 10.1.2.8.x2-1: Inter-MeNB handover without SeNB change

Figure 10.1.2.8.x2-1 shows an example signaling flow for inter-MeNB handover without SeNB change:

1.	The source MeNB starts the handover procedure by initiating the X2 Handover Preparation procedure. The source MeNB includes the SCG configuration in the HandoverPreparationInformation. The source MeNB includes the SeNB UE X2AP ID and SeNB ID as a reference to the UE context in the SeNB that was established by the source MeNB in the Handover Request message.

Editor’s note:	Whether the SeNB ID needs to be introduced in X2AP or not is FFS.

2.	If the target MeNB decides to keep the SeNB, the target MeNB sends SeNB Addition Request to the SeNB including the SeNB UE X2AP ID as a reference to the UE context in the SeNB that was established by the source MeNB.

3.	The SeNB replies with SeNB Addition Request Acknowledge.

Editor’s note:	Whether SeNB Addition Request is rejected when the information provided by target MeNB does not match the existing UE context (e.g. X2AP ID, SCG configuration) or an indicator as the UE context kept in SeNB is introduced in SeNB Addition Request Acknowledge is FFS.

4.	The target MeNB includes within the Handover Request Acknowledge message a transparent container to be sent to the UE as an RRC message to perform the handover which also includes the SCG configuration, and may also provide forwarding addresses to the source MeNB. The target MeNB indicates to the source MeNB that the UE context in the SeNB is kept if the target MeNB and the SeNB decided to keep the UE context in the SeNB in step 2 and step 3.

5.	The source MeNB sends SeNB Release Request to the SeNB. The source MeNB indicates to the SeNB that the UE context in SeNB is kept. If the indication as the UE context kept in SeNB is included, the SeNB keeps the UE context.

6.	The source MeNB triggers the UE to apply the new configuration.

7/8.	The UE synchronizes to the target MeNB and replies with RRCConnectionReconfigurationComplete message.

9.	The UE synchronizes to the SeNB.

10.	If the RRC connection reconfiguration procedure was successful, the target MeNB informs the SeNB.

11/12.	Data forwarding from the source MeNB takes place. Data forwarding can be omitted for SCG bearers. Direct data forwarding from the source MeNB to the SeNB is not possible for split bearers.

NOTE: Direct data forwarding may occur only for bearer type change.

13-16.	The target MeNB initiates the S1 Path Switch procedure.

NOTE:	If new UL TEIDs of the S-GW are included, the target MeNB performs MeNB initiated SeNB Modification procedure to provide them to the SeNB.

17.	The target MeNB initiates the UE Context Release procedure towards the source MeNB.

18.	Upon reception of the UE Context Release message, the SeNB can release C-plane related resource associated to the UE context towards the source MeNB. Any ongoing data forwarding may continue. The SeNB shall not release the UE context associated with the target MeNB if the indication was included in the SeNB Release Request in step 5.

End of the 1st modification

- Reference

[1] R3-152396 Introduction of handover enhancement for Dual Connectivity, Nokia Network

[2] R3-152462 Discussion the open issues in mobility enhancement, Samsung

[3] R3-152476 Consideration on remaining issues for HO enhancement, CATT

[4] R3-152618 Discussion on open issues in DC handover enhancement, ZTE Corporation