3GPP TSG-RAN WG3 #91bis	R3-161005

Bangalore, India, 11th – 15th April 2016

Agenda Item:	10.1.2

Source:	Ericsson

Title:	5G architecture scenarios

Document for:	Discussion, Decision

# Introduction

Based on discussions carried out during RAN3-91bis and basd on the text proposal presented in R3-160842, this paper presents a text proposal for inclusion in TR38.801

# Text proposal

----------------------------------------------First Change----------------------------------------------

## Deployment Scenarios

The following example scenarios should be considered for support by the NR radio network architecture

## x.1	Stand-alone Deployment

In this scenario the NR base station (NR BS) is deployed stand-alone e.g. in a macro deployment or indoor hotspot environment (could be public or enterprise). The NR BS can be connected to “any” transport. It is assumed that the full protocol stack is supported at the Standalone NR BS node.

## x.2	Co-sited deployment with LTE

In this scenario the NR functionality is co-sited with LTE functionality either as part of the same base station or as multiple base stations at the same site. Co-sited deployment can be applicable in all NR deployment scenarios e.g. Urban Macro. In this scenario it is desirable to fully utilise all spectrum resources assigned to both RATs by means of load balancing or connectivity via multiple RATs (e.g. utilising lower frequencies as coverage layer for users on cell edge).

## x.3	Centralized base-band deployments (high performance transport)

NR should support centralized based band deployments using remote radio units connected over high performance transport, e.g. optical networks, to a centralised base band unit. This will enable advanced CoMP schemes and scheduling optimisation, which could be useful in high capacity scenarios, or scenarios where cross cell coordination is beneficial. Different protocol split options between Central Unit and lower layer nodes may be possible.

Both standalone deployment and co-sited deployment with LTE could be considered for this scenario.

Editor’s Note: The title and text in this section may be subject to rewarding

## x.4	Centralized deployment with low performance transport

NR should support centralisation of the higher protocol layers of the NR radio stacks. These protocol layers require lower performances on the transport layer in terms of bandwidth, delay, synchronization and jitter.

Both standalone deployment and co-sited deployment with LTE could be considered for this scenario.

Editor’s Note: The text in this section may be subject to rewarding

## x.5	Shared RAN deployments

Similar to LTE, NR should support shared RAN deployments.