RAN WG3 Meeting #93	R3-161752

Gothenburg, Sweden, 22 - 26 Aug 2016

Title:	RAN configuration for network slices

Source: 	Huawei

Work item:	New Radio

Agenda item:	10.2.2

Document for:	Approval

Introduction

Network slicing is a network management tool that allows a Mobile Network Operator to efficiently allocate network resources in order to meet the service requirements of a customer or an application. In NextGen CN systems, MNOs will have the ability to configure and manage network slices through a set of configuration and orchestration functions provided by the management plane. In addition, to ensure efficient utilisation of network resources while meeting service performance goals, network slicing solutions will allow independent scalability of network functions and the flexible placement of physical and virtualised functions within the network to enable both centralised and geographically distributed deployments.

This paper presents a framework for configuring the RAN to provide appropriate treatment to traffic associated with each of the active network slice instances.

Discussion

Each MNO may create a customised set of network slices to meet their business and service needs. In some instances, a network slice may be pre-configured; in other instances, a network slice may be dynamically commissioned or re-configured to meet traffic demands or to meet changing service requirements.

Observation 1	Network slices may be dynamically commissioned or re-configured to meet traffic demand or may be pre-configured.

In addition to a customised set of network functions, each slice may have a distinct set of performance requirements that must be met to ensure that traffic associated with the slice satisfies those business and service needs. In some business scenarios, radio resources may be dedicated to a particular network slice in order to meet specific business or performance goals. However, in many instances, pre-allocation of radio resources to a network slice ("hard slicing") is not a viable solution for meeting service requirements due to the unpredictable distribution of UEs throughout the service area and the unpredictable demand for a particular service within a given area. In this case, the RAN must rely on scheduling to efficiently multiplex the traffic from multiple UEs in multiple network slices onto the available shared radio resources.

Observation 2	Pre-allocation of radio resources to a network slice ("hard slicing") may be viable in some business scenarios but may not be a viable solution in other scenarios.

Observation 3	Without pre-allocation of radio resources to a network slice, the RAN must rely on scheduling to multiplex UEs operating in different slices onto the available shared radio resources.

To meet network slice performance requirements, the RAN needs per-UE traffic management parameters.

Per-UE traffic management

Per-UE traffic management parameters are used to ensure that traffic to and from a particular UE meets the terms of the service agreement between the user and the MNO. These parameters may be further classified as:

per-UE, per-slice management parameters that govern traffic associated with a particular network slice.

per-UE aggregate traffic management parameters that govern the amalgamated traffic across all slices that a UE may be associated with.

Observation 4	The RAN requires per-UE traffic management parameters to manage per-slice and aggregate UE traffic.

Per-UE traffic management parameters may be pre-configured into the RAN or may be conveyed to the RAN by the CN when a UE is attached to a network slice [3]. Traffic management parameters that are pre-configured into the RAN may be changed by network management procedures that are outside the scope of this document. Traffic management parameters conveyed to the RAN by the CN during UE association with a slice override any parameters that may have been pre-configured into the RAN.

Over the radio link, pre-allocation of radio resources to a network slice may not always be viable due to the unpredictable distribution of UEs throughout the service area and the unpredictable demand for a particular service within a given area. When radio resources are not dedicated to a particular network slice, the RAN must use the per-UE traffic management parameters to govern assignment of radio resources to a UE and to manage the multiplexing of multiple UEs in multiple slices onto a radio interface.

Application of policies

The application of traffic management parameters is usually sufficient to provide isolation between UEs operating in different slices. However, in any shared medium used for bursty traffic, resource demands may produce temporary conflicts that cannot be resolved through normal scheduling and allocation of resources. When a conflict amongst UE resource requirements is detected, the RAN uses policies associated with the UE and, if necessary, policies associated with the network slice to resolve the conflict. Per-UE priority and pre-emption policies are associated with a particular UE within a particular network slice. If the application of per-UE policies does not resolve the conflict, the RAN uses priority and pre-emption policies associated with a particular slice within the PLMN.

Observation 5	The RAN requires policies to resolve scheduling conflicts within a slice and across slices.

Control of unscheduled uplink transmissions

Un-scheduled transmissions over an uplink common channel (e.g. a contention-based channel) need to be controlled by the RAN to ensure that performance metrics associated with a network slice can be met. In particular, congestion experienced by traffic in one network slice should not adversely impact other network slices. In some cases, this may require allocation and segregation of common uplink radio resources on a per-slice basis. Configuration of these resources may be tailored to meet the specific requirements of the network slice in terms of maximum latency between transmission opportunities, target probability of collision, collision backoff procedures, etc.

Observation 6	The RAN must be able to segregate and configure common (e.g. contention-based) radio resources used for unscheduled uplink transmission to meet the specific needs of a network slice.

Congestion control

During periods of congestion, un-scheduled transmissions over an uplink common channel need to be controlled by the RAN to prevent overload on the channel and loss of service to its UEs. In particular, congestion experienced by traffic in one network slice should not adversely impact other network slices. In some cases, this may require allocation and segregation of common uplink radio resources on a per-slice basis. In the case of radio resources shared by multiple network slices, the RAN must be able to apply access barring and congestion control mechanisms on a per-slice basis so as to guarantee the  isolation requirement. The detailed control mechanism for common and shared radio resources is FFS and needed for RAN2 as well.

Observation 7	The RAN must be able to apply control mechanisms for common and shared resources on a per-slice basis.

From a RAN perspective, providing the appropriate treatment to traffic associated with a network slice involves:

associating traffic to and from a UE with a corresponding network slice [3].

identifying the set of traffic management parameters associated with the UE.

identifying the set of traffic management parameters associated with the selected slice.

ensuring that traffic to and from the UE conforms to both UE-specific per-slice parameters and aggregate slice traffic management parameters.

ensuring that traffic associated with one network slice does not unduly impact traffic associated with other slices.

Proposal

Based on the following observations:

Observation 1	Network slices may be dynamically commissioned or re-configured to meet traffic demand or may be pre-configured.

Observation 2	Pre-allocation of radio resources to a network slice ("hard slicing") may be viable in some business scenarios but may not be a viable solution in other scenarios.

Observation 3	Without pre-allocation of radio resources to a network slice, the RAN must rely on scheduling to multiplex UEs operating in different slices onto the available shared radio resources.

Observation 4	The RAN requires per-UE traffic management parameters to manage per-slice and aggregate UE traffic.

Observation 5	The RAN requires policies to resolve scheduling conflicts within a slice and across slices.

Observation 6	The RAN must be able to segregate and configure common (e.g. contention-based) radio resources used for unscheduled uplink transmission to meet the specific needs of a network slice.

Observation 7	The RAN must be able to apply control mechanisms for common and shared resources on a per-slice basis.

we propose to capture the following text in TR 38.801 [1]:

START OF CHANGES

9.	Realization of Network Slicing

9.y	RAN configuration of network slices

In addition to a customised set of network functions, each slice may have a distinct set of performance requirements that must be met to ensure that traffic associated with the slice satisfies those business and service needs. From a RAN perspective, providing the appropriate treatment to traffic associated with a network slice involves:

- associating traffic to and from a UE with a corresponding network slice.

identifying the set of traffic management parameters associated with the UE.

identifying the set of traffic management parameters associated with the selected slice.

ensuring that traffic to and from the UE conforms to both UE-specific per-slice parameters and aggregate slice traffic management parameters.

ensuring that traffic associated with one network slice does not unduly impact traffic associated with other slices.

To accomplish this, the RAN needs per-UE traffic management parameters.

9.y.1	Per-UE traffic management

Per-UE traffic management parameters are used to ensure that traffic to and from a particular UE meets the terms of the service agreement between the user and the MNO. These parameters may be further classified as:

per-UE, per-slice traffic management parameters that govern traffic associated with a particular network slice.

per-UE aggregate traffic management parameters that govern the amalgamated traffic across all slices that a UE may be associated with.

9.y.2	Application of policies

When a conflict amongst UE resource requirements is detected, the RAN uses policies associated with the UE and, if necessary, policies associated with the network slice to resolve the conflict. Per-UE priority and pre-emption policies are associated with a particular UE within a particular network slice. If the application of per-UE policies does not resolve the conflict, the RAN uses priority and pre-emption policies associated with a particular slice within the PLMN.

9.y.3	Control of unscheduled uplink transmissions

Un-scheduled transmissions over an uplink common channel (e.g. a contention-based channel) need to be controlled by the RAN to ensure that performance metrics associated with a network slice can be met. In particular, congestion experienced by traffic in one network slice should not adversely impact other network slices.

9.y.4 Congestion control

During periods of congestion, un-scheduled transmissions over an uplink common channel need to be controlled by the RAN to prevent overload on the channel and loss of service to its UEs. In the case of radio resources shared by multiple network slices, the RAN must be able to apply access barring and congestion control mechanisms on a per-slice basis in order to guarantee the isolation requirement.

END OF CHANGES

References

- 3GPP TR 38.801 "Study on New Radio Access Technology: Radio Access Architecture and Interface", V0.2.0 (2016-06), RAN3.

- 3GPP TR 23.799 "Study on Architecture for Next Generation System", V0.7.0 (2016-08), SA2.

- R3-161759, Huawei, "RAN support for core network slice selection", RAN3#93, Aug 2016.