3GPP TSG-RAN WG3 Meeting #93	R3-161773

Gothenburg, Sweden, 22-26 August 2016

Source: 	AT&T

Title:	Preferred NR Functional Splits and Considerations

Agenda Item:	10.5.1

Document for:	Discussion and Decision

# Introduction

The draft of the RAN3 TR 38.801 [1] captures 8 functional split options, which are provided in Figure 1.

Figure 1 - Function Splits Under consideration between central and distributed unit

This contribution provides views on the preferred functional splits to fully standardize as well as some important considerations on those options.

# Discussion

## Option 2 (PDCP/RLC)

The PDCP/RLC split option should be the most straightforward split option to standardize. Fundamentals for achieving a PDCP-RLC split have already been standardized for LTE Dual Connectivity Alternative 3C. The effort to standardize a functional split between PDCP and RLC for NR and for eLTE should be relatively small thereby allowing support for Dual Connectivity in both these cases. Additionally, for multi-connectivity this option allows traffic aggregation from multiple NR and eLTE transmission points to be centralized, further allowing easier traffic distribution between NR and eLTE transmission points.

Observation 1: The functional split depicted as Option 2 is the most straightforward split option to standardize and the incremental effort to standardize this option should be relatively small.

Proposal 1: The functional split depicted as Option 2 should be considered for full standardization.

## Option 3 (High RLC/Low RLC) Considerations

This option splits the RLC sublayer into High RLC and Low RLC sublayers such that for RLC Acknowledge Mode operation, the ARQ and packet ordering functions may be performed at the High RLC sublayer residing in the central unit, while the segmentation and reassembly functions may be performed at the Low RLC sublayer residing in the distributed unit. Compared to the PDCP-RLC split, this split option has the advantage of being more robust under unreliable transport conditions because the ARQ and packet ordering is performed at the central unit. Additionally, this split option may also have better flow control across the split interface.

If Option 3 is standardized for NR, it may be advantageous if the corresponding High RLC/Low RLC interface for eLTE is also standardized for better aggregation at the High RLC layer across NR and eLTE. However, in the event that such a split option is standardized for NR but not eLTE, it may still be possible to operate Dual/Multi Connectivity across NR and eLTE as shown in Figure 2 below.

Due to the above considerations, Option 3 may have benefits over Option 2, as a high-layer functional split option to be considered for standardization.

Figure 2 – Multi Connectivity between NR and LTE for Option 3

Observation 2: The functional split depicted as Option 3 may have advantages over Option 2 in the form of better resiliency against unreliable transport and better flow control.

Proposal 2: The functional split depicted as Option 3 should be considered for full standardization.

## Option 7 (Within PHY): Benefits and Considerations

The choice of a lower layer split within the PHY layer requires careful consideration of various issues. We believe that a desirable mid-PHY split must satisfy ALL of the following three requirements:

- It should not preclude the implementation of advanced receivers (more on this below)

- It should maintain the ability to perform joint processing (both JR and JT) across multiple TPs

- It should keep transport bandwidth requirements reasonable

Figure 3 below shows an example block diagram of an advanced iterative receiver for NR. While LTE receivers have typically been single stage receivers, we believe that more advanced receivers (e.g. turbo equalizers) should become the model for NR. This is not only due to advances in computational power but also because many physical layer design aspects for NR are different from LTE. Some of the main considerations here are:

- Higher order MIMO in UL: For NR there may be a need to decode multiple spatial layers in the UL when using MU-MIMO. As the number of MIMO streams increases, the benefits of advanced receivers may become more significant due to an increase in inter-stream interference.

- Cyclic Prefix (CP) less design of waveform: If phase 2 of NR evolves to a CP less design of the waveform, then in addition to experiencing inter-stream interference, the NR signal may experience inter-symbol interference as well, thereby benefiting further from such advanced receivers.

- High frequency bands: For NR deployment in high frequency bands, advanced iterative receivers not only help with the UL link budget but can also have some implications on the power consumption at the UE.

Figure 3 – Generalized View of PHY Level Functions (Receiver View) (including SIC receiver and Turbo Equalizers)

Due to the above reasons, we strongly believe that any PHY layer split option should not preclude or limit the deployment of any type of advanced receivers for NR.

Further investigating PHY split options in light of the above described advanced iterative receivers for NR, we realize that it is difficult to achieve a common PHY layer split option between the uplink and downlink that simultaneously satisfies all three requirements listed earlier in this section. Thus, it appears that a good PHY split option may be asymmetric in the downlink and uplink as described below:

Observation 3: The functional split depicted as Option 7 may need to be asymmetric in the downlink and uplink in order to fully support deployment of advanced receivers for NR, not preclude joint processing techniques for the uplink or downlink, and have reasonable transport bandwidth requirement.

2.3.1 Downlink

A good PHY split option may be somewhat easier to achieve for the downlink than the uplink. This is because in the downlink FEC and antenna processing do not need to be co-located. The FEC processing can reside in the central unit and the antenna precoding can reside in the remote unit. The precoding weights can be sent from the RU to the CU on the control plane. This may also allow for transmission across multiple TP (RU) such as for joint transmission (JT). The benefit of this approach is that the number of layers on the interface between the RU and CU is the number of spatial layers, Ns. On the contrary, if the antenna precoding were to reside at the CU then the number of layers on the interface between the RU and CU would be the number of transceivers, Nt.  Since usually Nt>>Ns, keeping the precoding at the RU makes sense to keep the transport bandwidth requirements reasonable. Hence, an Option 7 PHY split that satisfies all three requirements listed above may be reasonable to achieve.

Observation 4: The functional split depicted as Option 7 may beneficial and feasible for the downlink by allowing the FEC processing to reside in the central unit and antenna processing to reside in the remote unit.

Proposal 3: The functional split depicted as Option 7 should be considered for full standardization for the downlink.

2.3.2 Uplink

For the uplink PHY processing, it becomes necessary to keep the equalization and FEC decoding at the CU since this allows for more advanced receivers such as turbo equalization or SIC, thereby satisfying the first requirement listed earlier in this section. Also notice that keeping the antenna processing at the CU allows for joint reception (JR) across multiple transmission points, thereby satisfying the second listed requirement as well. However, the problem is that the number of layers that need to be transported between the CU and RU becomes equal to the number of transceivers (since equalization is being done at the CU). It may be observed that even though the number of layers may be large, the signal may be highly correlated since the number of PHY layers Ns<<Nt. This observation leads to the possibility of using linear compression techniques to reduce the transport bandwidth requirement on this interface, thereby trying to satisfy the third listed requirement for the uplink PHY split as well.

Figure 4 below shows an example advanced iterative receiver for the uplink with a PHY split using the above described linear compression technique that may be capable of satisfying all three listed requirements for the uplink.

Figure 4 – Generalized View of PHY Level Functions (Receiver View) (including SIC receiver and Turbo Equalizers) with Uplink PHY Split

As may be observed from the figure, a linear spatial compression technique may be able to reduce the number of PHY layers that are transported over the RU-CU interface from Nt to Ns. At the CU a corresponding spatial decompression may then be able to recover the Nt layers, thereby allowing the CU to perform channel estimation and equalization with the full Nt dimensions. It is possible that there may be some degradation in performance as the compression de-compression stage is likely to introduce some errors, but the reduction in bandwidth due to this approach is likely to outweigh any degradation. Moreover, such an approach may indeed make it feasible to achieve a good PHY split option that satisfies all three requirements listed earlier.

Observation 5: The functional split depicted as Option 7 may be beneficial and feasible for the uplink with the introduction of a linear compression technique that allows the reduction of PHY layers to be transported over the RU-CU interface from Nt to Ns.

Proposal 4: The functional split depicted as Option 7 should be considered for full standardization for the uplink based on the use of a linear compression technique.

## Option 8 (PHY/RF)

LTE deployments may use a traditional PHY/RF functional split between the Baseband Unit (BBU) and Remote Radio Unit (RRU) in LTE deployments via the CPRI interface. For NR, such a PHY/RF split (Option 8) has many practical issues. The deployment of massive MIMO systems for NR, in combination with very high system bandwidths, and very low latency performance needs, place extreme requirements on the transport between central and distributed units. For example, Table 6.1.2.2.2-1 of draft TR 38.801 [1] shows bandwidth requirements for a PHY/RF split as a function of different number of antenna ports and system bandwidth. Even with the evolution of compression techniques this functional split option may remain very challenging to deploy. Additionally for NR, the PHY/RF function split may place significant restrictions on the possible fiber-distance between central and distributed unit because any latency over the fronthaul interface gets added to the over-the-air transmission latency.

Due to the above observations, we believe that Option 8 should not be considered for standardization.

# Summary of Preferred Options for Standardized Functional Splits

Based on the discussion and observations above, we propose that 3 functional splits be fully standardized.  These are:

- Option 2

- Option 3

- Option 7

# Conclusions

The following observations were made:

Observation 1: The functional split depicted as Option 2 is the most straightforward split option to standardize and the incremental effort to standardize this option should be relatively small.

Observation 2: The functional split depicted as Option 3 may have advantages over Option 2 in the form of better resiliency against unreliable transport and better flow control.

Observation 3: The functional split depicted as Option 7 may need to be asymmetric in the downlink and uplink in order to fully support deployment of advanced receivers for NR, not preclude joint processing techniques for the uplink or downlink, and have reasonable transport bandwidth requirement.

Observation 4: The functional split depicted as Option 7 may beneficial and feasible for the downlink by allowing the FEC processing to reside in the central unit and antenna processing to reside in the remote unit.

Observation 5: The functional split depicted as Option 7 may be beneficial and feasible for the uplink with the introduction of a linear compression technique that allows the reduction of PHY layers to be transported over the RU-CU interface from Nt to Ns.

The observations lead to the following proposal(s):

Proposal 1: The functional split depicted as Option 2 should be considered for full standardization.

Proposal 2: The functional split depicted as Option 3 should be considered for full standardization.

Proposal 3: The functional split depicted as Option 7 should be considered for full standardization for the downlink.

Proposal 4: The functional split depicted as Option 7 should be considered for full standardization for the uplink based on the use of a linear compression technique.

# References

- TR 38.801, “Study on New Radio Access Technology; Radio Access Architecture and Interfaces (Release 14)”.