3GPP TSG-RAN WG3 Meeting #93	R3-161999

Gothenburg, Sweden, 22-26 August 2016

Source: 	AT&T

Title:	Text Proposal for: Migration Towards New RAN

Agenda Item:	10.3.2

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide a text proposal to resolve the following comeback related to R3-161772 [2]:

CB: # 50_TP_MP_3724

-  describes the migration path 3/7/ 2/4

- use “step’ no phase

- remove implementation aspects

- capture the FFS

# Text Proposal

Beginning of Text Proposal for TR 38.801

# 14	     Migration towards New RAN

Editor’s notes: Intention is to capture how migration from existing E-UTRAN to the RAN supporting new RAT may occur (i.e. to capture the required modification/upgrades to E-UTRAN in order to evolve to the RAN supporting new RAT).

..................................BEGIN NEW TEXT ………………………………………………………………

14.X.1    Identified Operator Migration Steps

One identified  sequence of migration from existing E-UTRAN to the New RAN is as follows:

Step 1:  Early 5G Deployment utilizing Option 3

Step 2:  Migration to Option 7 – Includes Simultaneous Support for Option 3

Step 3:  Migration to Option 4 and/or 2 – Plus Simultaneous Support for Option 3 & Option 7

NOTE: Options 2, 3, 4 and 7 are described in section 6.3.1.

- Option 3 may include 3 and/or 3a.

- Option 4 may include 4 and/or 4a

- Option 7 may include 7 and/or 7a.

14.X.1.1    Mobility and Service Continuity between NR and (Evolved) E-UTRA

The applicable mobility and service continuity scenarios should be evaluated for each migration step.

Editor’s note: Intention for the text below is to establish a placeholder to begin to capture a description of service continuity mobility scenarios that are required to be supported during migration steps. It is FFS on whether every mobility and service continuity applies to each migration path/step and how to capture this.

The following mobility and service continuity scenarios should be supported during operation of Steps 1-3 above:

- [e.g. Data Mobility between Evolved E-UTRA and NR with service continuity for data -   FFS]

- [e.g. Voice Mobility between Evolved E-UTRA and NR with service continuity for data] - FFS]

- [e.g. Data Mobility between E-UTRA and NR with service continuity for data – FFS]

- [e.g. Voice Mobility between E-UTRA and NR with service continuity for data -  FFS]

- [e.g. Other mobility scenarios - FFS]

# References

[1] R3-161772 “AT&T Views on 5G Architecture Evolution”