3GPP TSG-RAN WG3 Meeting # 93bis	R3-162176

Sophia Antipolis, France, Oct. 10th – 14th, 2016

Agenda item:		10.5.2

Source:	LG Electronics Inc.

Title:	Inter-RAT Mobility for Standalone Operation

Document for:		Discussion

1. Introduction

One issue was about the inter-RAT handover, however, the full picture of all the cases is needed. In this paper, we try to list up all the cases and the corresponding TP is also proposed for TR 38.801.

2. Discussion

In the past RAN3 meeting, we have agreed the following scenarios on inter-RAT handover as shown in Fig. 1:

Fig. 1. Inter-RAT handover with LTE scenarios.

In order to have a full picture on all the scenarios of inter-RAT handover with LTE, it is better to have the definitions for the RAN nodes to help the analysis, which are proposed in [5] and also given as follows:

- LTE eNB: The LTE eNB supports connectivity to EPC.

- eLTE eNB: The eLTE eNB is the evolution of eNB that supports connectivity to EPC and NG-Core.

- NR gNB: The NR gNB supports the connectivity to NG-Core.

With the definition above, it can be seen that the LTE eNB should be considered for the inter-RAT handover. The list is given below:

- HO between LTE eNB and eLTE eNB:

- S1 handover

- X2 handover

- HO between LTE eNB and NR gNB:

- Inter-RAT handover based on CN interface: to be changed in scenario 1 of Fig. 1

- HO between eLTE eNB and NR gNB:

- NG handover: scenario 2 in Fig. 1

- Xn handover: scenario 3 in Fig. 1

- Inter-RAT handover based on CN interface: scenario 1 in Fig. 1

So it is better to capture all the cases above as a full picture for inter-RAT handover.

Based on the analysis above, the following proposal is suggested to RAN3:

Proposal): To capture the TP for TR 38.801 as a full picture of inter-RAT handover.

3. Conclusion

In this contribution, the inter-RAT HO was analysed. The following proposal is suggested to RAN3:

Proposal): To capture the TP for TR 38.801 as a full picture of inter-RAT handover.

4. References

- RP-160576, “New SID Proposal: Study on New Radio Access Technology”

- RP-160562, “TP on RAN Architecture Requirements”

- RP-161266, “5G Architecture Options-full list”

- TR 38.801

- RP-161715, “Consideration on the Migration path”, LGE

Text Proposal

----------------Start of the Change---------------

7.2.1.2	Inter-RAT handover with LTE

There are three inter-RAT handover with LTE scenarios as shown in Figure 7.2.1.2-1.

Figure 7.2.1.2-1: Inter-RAT handover with LTE scenarios

NOTE 1:	The interface between EPC and 5G CN is pending to SA2.

NOTE 2:	Further justification on Scenario 2 is needed.

NOTE 3: The handover between LTE eNB and eLTE eNB is supported by legacy S1 and X2 handover.

In the scenario 1 of Fig. 1, the following cases are supported:

- S1/NG Handover between LTE eNB and NR gNB

- S1/NG Handover between eLTE eNB and NR gNB

In the scenario 2 of Fig. 1, the following case is supported:

- NG Handover between eLTE eNB and NR gNB

In the scenario 3 of Fig. 1, the following case is supported:

- Xn Handover between eLTE eNB and NR gNB

----------------End of the Change---------------