3GPP TSG-RAN WG3 Meeting #94	R3-162985

Reno, Nevada, USA, 14 – 18, November, 2016

Agenda Item:	   10.4

Source: 	   Ericsson

Title: 	   TP on Availability of Network Slices

Document for:	Discussion and Decision

# Introduction

In R3-162984 a discussion on network slice availability was taken. The discussion leads to a number of proposals that are reflected in the TP below.

# TP to 38.801

-----------------------------------After this point changes can be found-----------------------------------

## 8.1	Key principles for support of Network Slicing in RAN

Network Slicing is a new concept to allow differentiated treatment depending on each customer requirements. With slicing, it is now possible for Mobile Network Operators (MNO) to consider customers as belonging to different tenant types with each having different service requirements that govern in terms of what slice types each tenant is eligible to use based on Service Level Agreement (SLA) and subscriptions.

The following key principles apply for support of Network Slicing in RAN

RAN awareness of slices

-	RAN shall support a differentiated handling of traffic for different network slices which have been pre-configured. How RAN supports the slice enabling in terms of RAN functions (i.e. the set of network functions that comprise each slice) is implementation dependent.

Editor’s note: It is still FFS if 3GPP will still additionally want to standardize a few basic slices and the network functions that comprise them (e.g. eMBB, Massive MTC).

Selection of RAN part of the network slice

-	RAN shall support the selection of the RAN part of the network slice, by a slice ID provided by the UE which unambiguously identifies one of the pre-configured network slices in the PLMN.

Editor’s note: How the UE gets this unambiguous slice ID is FFS and to be decided with SA2. The ID could be sent to the UE by the CN after the CN has selected the slice (e.g. similar to eDECOR feature) or it could be pre-configured in the UE.

Editor’s note: it is FFS how the RAN verifies that the UE is authorized to select the slice and when this verification happens.

Editor’s note: It is FFS if the RAN may also select the slice based on specific resources accessed by the UE.

Resource management between slices

-	RAN shall support policy enforcement between slices as per service level agreements. It should be possible for a single RAN node to support multiple slices. The RAN should be free to apply the best RRM policy for the SLA in place to each supported slice.

Editor’s note: How RAN handles the requirements coming from the service level agreements is to be discussed with SA2.

Support of QoS

-	RAN shall support QoS differentiation within a slice.

Editor’s note: It is FFS if RAN shall additionally support QoS enforcement independently per slice.

RAN selection of CN entity

-	RAN shall support initial selection of the CN entity for initial routing of uplink messages based on received slice ID and a mapping in the RAN node (CN entity, slices supported). If no slice ID is received, the RAN selects the CN entity based on NNSF like function, e.g. UE temporary ID.

Resource isolation between slices

-	RAN shall support resource isolation between slices. RAN resource isolation may be achieved by means of RRM policies and protection mechanisms that should avoid that shortage of shared resources in one slice breaks the service level agreement for another slice. It should be possible to fully dedicate RAN resources to a certain slice

Editor’s note: Resource isolation needs to be clarified: It is unclear if resource isolation would imply that multiple slices cannot share control plane (respectively user plane) resources or processing resources in common. It is unclear if resource isolation would imply that cryptographic means should be used to isolate CP and UP traffic between slices.

Slice Availability

-	Whether a network slice is considered as a service and whether network slices are available over different cells of the RAN in a scattered or continuous way needs to be studied. As an example, the effects of network slice availability over idle and connected mode mobility shall be analysed.
Different levels of slice availability does not imply the need to broadcast slice information. The RAN and the CN are responsible to handle a service request for a slice that may or may not be available in a given area. Admission or rejection of access to a slice may depend by factors such as support for the slice, availability of resources, support of the requested service by other slices

-----------------------------------We have run out of changes-----------------------------------