3GPP TSG-RAN WG3 Meeting # 95	R3-170408

Athens, Greece, Feb. 13th – 17th, 2017

Agenda item:		10.6.1

Source:	LG Electronics Inc.

Title:	Text Proposal for Option 4 and 7 on procedures

Document for:		Discussion

1. Text Proposal

----------------Start of the First Change---------------

10.1.3	Option 4/4a

10.1.3.1	Architectural aspects

Radio Protocol Architecture for the User Plane is shown below.

Figure 10.1.3.1-1: Radio Protocol Architecture for split bearer and SCG bearer in Option 4/4a

Network interface configurations can be defined in Figure 10.1.3.1-2 and Figure 10.1.3.1-3.

Figure 10.1.3.1-2: C-Plane connectivity for Option 4/4a

Figure 10.1.3.1-3: U-Plane connectivity for Option 4/4a

10.1.3.2	Procedural aspects

. The DC procedures in section 7.3.1.3 on Xn interface are the very basic procedures to be supported for this option. In addition, the following procedures should be supported.

Change of Secondary Node

This procedure is very similar to the legacy DC option and is applied in the case that the master node decides to change the Secondary node in case of UE’s moving out of the original Secondary Node while it is still in the coverage of the master node. This can be realized by the Secondary Node Release + the Secondary Node Addition.

Secondary Cell Group (SCG) Change

This procedure refers to a synchronous SCG reconfiguration procedure towards the UE involving random access on PSCell. The procedure should be supported for the two options, but details are pending to RAN2.

Master Node Handover to a Single Connection, or Single Connection Handover to Master Node

The first procedure above is also very similar to the Rel-12 DC scenario. That is the case that the master node decides to handover to the neighbor node with a single connection, while the secondary node is released. This procedure can be designed based on the optimized Xn handover together with Secondary Node Release procedure.

The second procedure is the case that a single connection node decides to handover to a neighbor master node, which set up the dual connection during the handover. This procedure can be designed based on the optimized Xn handover together with Secondary Node Addition procedure.

Inter Master Node Handover without Secondary Node change

This procedure is very similar to the Rel-13 eDC scenario. That is the case that the master node decides to handover to the neighbor master node while the secondary node can be kept. This procedure can be designed based on the optimized Xn handover together with Secondary Node Addition procedure.

NOTE:	The details on the procedures above and the potential additional procedures will be investigated in the normative phase.

----------------End of the First Change---------------

----------------Start of the Second Change---------------

10.1.4	Option 7/7a

10.1.4.1	Architectural aspects

Radio Protocol Architecture for the User Plane is shown below.

Figure 10.1.4.1-1: Radio Protocol Architecture for split bearer and SCG bearer in Option 7/7a

Network interface configurations can be defined in Figure 10.1.4.1-2 and Figure 10.1.4.1-3.

Figure 10.1.4.1-2: C-Plane connectivity for Option 7/7a

Figure 10.1.4.1-3: U-Plane connectivity for Option 7/7a

10.1.4.2	Procedural aspects

The basic and generic procedures defined under section 10.1.3.2 for option 4/4a also apply to this option. The difference is the master node role exchange between eLTE eNB and gNB.

NOTE:	The details on the procedures and the potential additional procedures will be investigated in the normative phase.

----------------End of the Second Change---------------