3GPP TSG-RAN WG3 Meeting #95 	R3-170617

# Athens, Grace, 13th -17th  February 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.413 | CR | 1480 | rev | 3 | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of eDECOR in RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eDECOR-UTRA_LTE-Core |  |  |  |  |  |  | Date: |  | 2017-02-03 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | eDECOR has been finalized in SA2 and a Work Item to introduced in eDECOR functionality in RAN has been approved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | DCN ID is added in Initial UE Message. ServedDCNs is added in S1AP setup and MME Configuration Update procedure. Revision 3 Reason for change is modified. IE Served DCNs is updated. Specification is Rebased. Revision 2 Change the DCN-ID encoding to FFS. Remove Editor’s Note. Update ServedDCNs. Revision 1 Add DCN-ID in Definitions. Remove the impact on REROUTE NAS REQUEST |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | eDECOR is not supported in EUTRAN system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 8.6.2.1, 8.7.3.2, 8.7.5.2, 9.1.7.1, 9.1.8.5, 9.1.8.10, 9.3.3, 9.3.4, 9.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS/TR 36.331 CR 2577 TS/TR 36.300 CR |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. 
A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

ACL functionality: A functionality controlling the access to network nodes. In case of Access Control Lists (ACL) functionality is applied in a network node the network node may only accept connections from other peer network nodes once the source addresses of the sending network node is already known in the target node.

CSG Cell: an E-UTRAN cell broadcasting a CSG indicator set to true and a CSG identity. This cell operates in Closed Access Mode as defined in TS 22.220 [28].

DCN-ID: DCN identity identifies a specific decicated core network (DCN).

Dual Connectivity: as defined in TS 36.300 [14].

Elementary Procedure: S1AP consists of Elementary Procedures (EPs). An Elementary Procedure is a unit of interaction between eNBs and the EPC. These Elementary Procedures are defined separately and are intended to be used to build up complete sequences in a flexible manner. If the independence between some EPs is restricted, it is described under the relevant EP description. Unless otherwise stated by the restrictions, the EPs may be invoked independently of each other as standalone procedures, which can be active in parallel. The usage of several S1AP EPs together or together with EPs from other interfaces is specified in stage 2 specifications (e.g., TS 23.401 [11] and TS 36.300 [14]).

An EP consists of an initiating message and possibly a response message. Two kinds of EPs are used:

-	Class 1: Elementary Procedures with response (success and/or failure).

-	Class 2: Elementary Procedures without response.

For Class 1 EPs, the types of responses can be as follows:

Successful:

-	A signalling message explicitly indicates that the elementary procedure successfully completed with the receipt of the response.

Unsuccessful:

-	A signalling message explicitly indicates that the EP failed.

-	On time supervision expiry (i.e., absence of expected response).

Successful and Unsuccessful:

-	One signalling message reports both successful and unsuccessful outcome for the different included requests. The response message used is the one defined for successful outcome.

Class 2 EPs are considered always successful.

eNB UE S1AP ID: as defined in TS 36.401 [2].

Hybrid Cell: an E-UTRAN cell broadcasting a CSG indicator set to false and a CSG identity. This cell operates in Hybrid Access Mode as defined in TS 22.220 [28].

MME UE S1AP ID: as defined in TS 36.401 [2].

E-RAB: as defined in TS 36.401 [2].

NOTE 1:	The E-RAB is either a default E-RAB or a dedicated E-RAB.

E-RAB ID: the E-RAB ID uniquely identifies an E-RAB for one UE.

NOTE 2:	The E-RAB ID remains unique for the UE even if the UE-associated logical S1-connection is released during periods of user inactivity.

Data Radio Bearer: the Data Radio bearer transports the packets of an E-RAB between a UE and an eNB. There is a one-to-one mapping between the E-RAB and the Data Radio Bearer.

Secondary Cell Group: as defined in TS 36.300 [14].

UE-associated signalling: When S1-AP messages associated to one UE uses the UE-associated logical S1-connection for association of the message to the UE in eNB and EPC.

UE-associated logical S1-connection: The UE-associated logical S1-connection uses the identities MME UE S1AP ID and eNB UE S1AP ID according to definition in TS 23.401 [11]. For a received UE associated S1-AP message the MME identifies the associated UE based on the MME UE S1AP ID IE and the eNB identifies the associated UE based on the eNB UE S1AP ID IE. The UE-associated logical S1-connection may exist before the S1 UE context is setup in eNB.

------------------------------------------

Skip to next change

------------------------------------------

## 8.6	NAS transport

### 8.6.1	General

The purpose of the NAS Transport procedure is to carry UE – MME signalling over the S1 Interface. The NAS messages are not interpreted by the eNB, and their content is outside the scope of this specification. The procedure may use an existing UE-associated logical S1-connection. If no UE-associated logical S1-connection exists, the establishment of the UE-associated logical S1-connection is initiated (and may be established) as part of the procedure.

The NAS messages are transported in an IE of the INITIAL UE MESSAGE, DOWNLINK NAS TRANSPORT, UPLINK NAS TRANSPORT or REROUTE NAS REQUEST messages.

### 8.6.2	Successful Operations

#### 8.6.2.1	Initial UE Message

Figure 8.6.2.1-1: Initial UE Message procedure

When the eNB has received from the radio interface the first UL NAS message transmitted on an RRC connection to be forwarded to an MME, the eNB shall invoke the NAS Transport procedure and send the INITIAL UE MESSAGE message to the MME including the NAS message as a NAS-PDU IE. The eNB shall allocate a unique eNB UE S1AP ID to be used for the UE and the eNB shall include this identity in the INITIAL UE MESSAGE message. In case of network sharing, the selected PLMN is indicated by the PLMN Identity IE within the TAI IE included in the INITIAL UE MESSAGE message. When the eNB has received from the radio interface the S-TMSI IE, it shall include it in the INITIAL UE MESSAGE message. If the eNB does not support NNSF and the eNB has received from the radio interface the GUMMEI IE, the eNB may include it in the INITIAL UE MESSAGE message. If the eNB does not support NNSF and the eNB has received from the radio interface the GUMMEI Type IE, the eNB may include it in the INITIAL UE MESSAGE message.

If the establishment of the UE-associated logical S1-connection towards the CN is performed due to an RRC connection establishment originating from a CSG cell, the CSG Id IE shall be included in the INITIAL UE MESSAGE message.

If the establishment of the UE-associated logical S1-connection towards the CN is performed due to an RRC connection establishment originating from a Hybrid cell, the CSG Id IE and the Cell Access Mode IE shall be included in the INITIAL UE MESSAGE message.

If the establishment of the UE-associated logical S1-connection towards the CN is performed due to an RRC connection establishment triggered by a Relay Node as defined in TS 36.300 [14], the GW Transport Layer Address IE and the Relay Node Indicator IE shall be included in the INITIAL UE MESSAGE message.

If the eNB has a L-GW function for LIPA operation, it shall include the GW Transport Layer Address IE in the INITIAL UE MESSAGE message.

If the SIPTO L-GW Transport Layer Address IE is received in the INITIAL UE MESSAGE message, the MME shall, if supported, use it for SIPTO@LN operation as sepecified in TS 23.401 [11].

If the LHN ID IE is included in the INITIAL UE MESSAGE message, the MME shall, if supported, use it as specified in TS 23.401 [11].

If the Tunnel Information for BBF IE is received in the INITIAL UE MESSAGE message, the MME shall, if supported, use it in the core network as specified in TS 23.139 [37].

If the MME Group ID IE is included in the INITIAL UE MESSAGE message this indicates that the message is a rerouted message and the MME shall, if supported, use the IE as described in TS 23.401 [11].

If the UE-Usage-Type IE is included in the INITIAL UE MESSAGE message, then the selected MME in the DCN shall if supported, use it as defined in TS 23.401 [48].

If the DCN ID IE is included in the INITIAL UE MESSAGE message, then the selected MME in the DCN shall if supported, use it as defined in TS 23.401 [48].

NOTE:	The first UL NAS message is always received in the RRC CONNECTION SETUP COMPLETE message.

------------------------------------------

Skip to next change

------------------------------------------

### 8.7.3	S1 Setup

#### 8.7.3.1	General

The purpose of the S1 Setup procedure is to exchange application level data needed for the eNB and the MME to correctly interoperate on the S1 interface. This procedure shall be the first S1AP procedure triggered after the TNL association has become operational. The procedure uses non-UE associated signalling.

This procedure erases any existing application level configuration data in the two nodes and replaces it by the one received and clears MME overload state information at the eNB. If the eNB and MME do not agree on retaining the UE Contexts this procedure also re-initialises the E-UTRAN S1AP UE-related contexts (if any) and erases all related signalling connections in the two nodes like a Reset procedure would do. If the eNB initiating the S1 Setup procedure supports a CSG cell, the procedure shall report the CSG ID(s) of the supported CSGs.

#### 8.7.3.2	Successful Operation

Figure 8.7.3.2-1: S1 Setup procedure: Successful Operation.

The eNB initiates the procedure by sending a S1 SETUP REQUEST message including the appropriate data to the MME. The MME responds with a S1 SETUP RESPONSE message including the appropriate data.

The exchanged data shall be stored in respective node and used for the duration of the TNL association. When this procedure is finished, the S1 interface is operational and other S1 messages can be exchanged.

If the eNB initiating the S1 SETUP procedure supports one (or more) CSG cell(s), the S1 SETUP REQUEST message shall contain the CSG ID(s) of the supported CSG(s).

If the S1 SETUP REQUEST message contains the eNB Name IE the MME may use this IE as a human readable name of the eNB.

If the S1 SETUP RESPONSE message contains the MME Name IE the eNB may use this IE as a human readable name of the MME.

If the MME Relay Support Indicator IE is included in the S1 SETUP RESPONSE message, the eNB shall consider this information when selecting an appropriate MME for the Relay Node.

If the UE Retention Information IE set to “ues-retained“ was included in the S1 SETUP REQUEST message, then the MME may accept the proposal to retain the existing UE related contexts and signalling connections by including the UE Retention Information IE set to “ues-retained“ in the S1 SETUP RESPONSE message.

If the NB-IoT Default Paging DRX IE is included in the S1 SETUP REQUEST message, the MME will take it into account as specified in TS36.300 [14].

If the S1 SETUP RESPONSE message contains the ServedDCNs IE then the eNB shall if supported, use it as defined in TS 23.401 [48].

#### 8.7.3.3	Unsuccessful Operation

Figure 8.7.3.3-1: S1 Setup procedure: Unsuccessful Operation.

If the MME cannot accept the setup, it should respond with a S1 SETUP FAILURE and appropriate cause value.

If the S1 SETUP FAILURE message includes the Time To Wait IE, the eNB shall wait at least for the indicated time before reinitiating the S1 setup towards the same MME.

#### 8.7.3.4	Abnormal Conditions

If the eNB initiates the procedure by sending a S1 SETUP REQUEST message including the PLMN Identity IEs and none of the PLMNs provided by the eNB is identified by the MME, then the MME shall reject the eNB S1 Setup Request procedure with the appropriate cause value, e.g., “Unknown PLMN”.

------------------------------------------

Skip to next change

------------------------------------------

### 8.7.5	MME Configuration Update

#### 8.7.5.1	General

The purpose of the MME Configuration Update procedure is to update application level configuration data needed for the eNB and MME to interoperate correctly on the S1 interface. This procedure does not affect existing UE-related contexts, if any.

#### 8.7.5.2	Successful Operation

Figure 8.7.5.2-1: MME Configuration Update procedure: Successful Operation.

The MME initiates the procedure by sending an MME CONFIGURATION UPDATE message including the appropriate updated configuration data to the eNB. The eNB responds with an MME CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If information element(s) is/are not included in the MME CONFIGURATION UPDATE message, the eNB shall interpret that the corresponding configuration data is not changed and shall continue to operate the S1 with the existing related configuration data.

If the served PLMNs is/are to be updated, the eNB shall overwrite the whole list of PLMNs.

If the MME CONFIGURATION UPDATE message contains the MME Name IE, the eNB may use this IE as a human readable name of the MME.

The updated configuration data shall be stored in the respective node and used for the duration of the TNL association or until any further update is performed from the MME.

The MME may initiate a further MME Configuration Update procedure only after a previous MME Configuration Update procedure has been completed.

If the MME CONFIGURATION UPDATE message contains the ServedDCNs IE then the eNB shall if supported, use it as defined in TS 23.401 [48].

#### 8.7.5.3	Unsuccessful Operation

Figure 8.7.5.3-1: MME Configuration Update: Unsuccessful Operation.

If the eNB cannot accept the update, it shall respond with an MME CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the MME CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE the MME shall wait at least for the indicated time before reinitiating the MME Configuration Update procedure towards the same eNB. Both nodes shall continue to operate the S1 with the existing configuration data.

#### 8.7.5.4	Abnormal Conditions

If the MME neither receives an MME CONFIGURATION UPDATE ACKOWLEDGE nor an MME CONFIGURATION UPDATE FAILURE message, the MME may reinitiate MME Configuration Update procedure towards the same eNB provided that the content of the new MME CONFIGURATION UPDATE message is identical to the content of the previously unacknowledged MME CONFIGURATION UPDATE message.

------------------------------------------

Skip to next change

------------------------------------------

### 9.1.7	NAS Transport Messages

#### 9.1.7.1	INITIAL UE MESSAGE

This message is sent by the eNB to transfer the initial layer 3 message to the MME over the S1 interface.

Direction: eNB  MME

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| eNB UE S1AP ID | M |  | 9.2.3.4 |  | YES | reject |
| NAS-PDU | M |  | 9.2.3.5 |  | YES | reject |
| TAI | M |  | 9.2.3.16 | Indicating the Tracking Area from which the UE has sent the NAS message. | YES | reject |
| E-UTRAN CGI | M |  | 9.2.1.38 | Indicating the E-UTRAN CGI from which the UE has sent the NAS message. | YES | ignore |
| RRC Establishment Cause | M |  | 9.2.1.3a |  | YES | ignore |
| S-TMSI | O |  | 9.2.3.6 |  | YES | reject |
| CSG Id | O |  | 9.2.1.62 |  | YES | reject |
| GUMMEI | O |  | 9.2.3.9 |  | YES | reject |
| Cell Access Mode | O |  | 9.2.1.74 |  | YES | reject |
| GW Transport Layer Address | O |  | Transport Layer Address 9.2.2.1 | Indicating GW Transport Layer Address if the GW is collocated with eNB. | YES | ignore |
| Relay Node Indicator | O |  | 9.2.1.79 | Indicating a relay node. | YES | reject |
| GUMMEI Type | O |  | ENUMERATED (native, mapped, …) |  | YES | ignore |
| Tunnel Information for BBF | O |  | Tunnel Information 9.2.2.3 | Indicating HeNB’s Local IP Address assigned by the broadband access provider, UDP port Number. | YES | ignore |
| SIPTO L-GW Transport Layer Address | O |  | Transport Layer Address 9.2.2.1 | Indicating SIPTO L-GW Transport Layer Address if the SIPTO L-GW is collocated with eNB. | YES | ignore |
| LHN ID | O |  | 9.2.1.92 |  | YES | ignore |
| MME Group ID | O |  | 9.2.3.44 |  | YES | ignore |
| UE Usage Type | O |  | INTEGER (0..255) |  | YES | ignore |
| DCN ID | O |  | OCTET STRING (SIZE(FFS)) |  | YES | ignore |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.1.8.5	S1 SETUP RESPONSE

This message is sent by the MME to transfer information for a TNL association.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME Name | O |  | PrintableString(SIZE(1..150,…)) |  | YES | ignore |
| Served GUMMEIs |  | 1..<maxnoofRATs> |  | The LTE related pool configuration is included on the first place in the list. | GLOBAL | reject |
| >Served PLMNs |  | 1..<maxnoofPLMNsPerMME> |  |  | - |  |
| >>PLMN Identity | M |  | 9.2.3.8 |  | - |  |
| >Served GroupIDs |  | 1..<maxnoofGroupIDs> |  |  | - |  |
| >>MME Group ID | M |  | OCTET STRING (SIZE(2)) |  | - |  |
| >Served MMECs |  | 1..<maxnoofMMECs> |  |  | - |  |
| >>MME Code | M |  | 9.2.3.12 |  | - |  |
| Relative MME Capacity | M |  | 9.2.3.17 |  | YES | ignore |
| MME Relay Support Indicator | O |  | 9.2.1.82 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.1.21 |  | YES | ignore |
| UE Retention Information | O |  | 9.2.1.112 |  | YES | ignore |
| Served DCNs |  | 0..<maxnoofDCNs> |  |  | GLOBAL | ignore |
| >Served DCNs Items | M |  | 9.2.1.XX |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPLMNsPerMME | Maximum no. of PLMNs per MME. Value is 32. |
| maxnoofRATs | Maximum no. of RATs. Value is 8. |
| maxnoofGroupIDs | Maximum no. of GroupIDs per node per RAT. Value is 65535. |
| maxnoofMMECs | Maximum no. of MMECs per node per RAT. Value is 256. |
| maxnoofDCNs | Maximum no. of DCNs servered by one MME. Value is 32. |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.1.8.10	MME CONFIGURATION UPDATE

This message is sent by the MME to transfer updated information for a TNL association.

Direction: MME  eNB

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | reject |
| MME Name | O |  | PrintableString(SIZE(1..150,…)) |  | YES | ignore |
| Served GUMMEIs |  | 0..<maxnoofRATs> |  | The LTE related pool configuration is included on the first place in the list. | GLOBAL | reject |
| >Served PLMNs |  | 1..<maxnoofPLMNsPerMME> |  |  | - |  |
| >>PLMN Identity | M |  | 9.2.3.8 |  | - |  |
| >Served GroupIDs |  | 1..<maxnoofGroupIDs> |  |  | - |  |
| >>MME GroupID | M |  | OCTET STRING (SIZE(2)) |  | - |  |
| >Served MMECs |  | 1..<maxnoofMMECs> |  |  | - |  |
| >>MME Code | M |  | 9.2.3.12 |  | - |  |
| Relative MME Capacity | O |  | 9.2.3.17 |  | YES | Reject |
| Served DCNs |  | 0..<maxnoofDCNs> |  |  | GLOBAL | ignore |
| >Served DCNs Items | M |  | 9.2.1.XX |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPLMNsPerMME | Maximum no. of PLMNs per MME. Value is 32. |
| maxnoofRATs | Maximum no. of RATs. Value is 8. |
| maxnoofGroupIDs | Maximum no. of GroupIDs per node per RAT. Value is 65535. |
| maxnoofMMECs | Maximum no. of MMECs per node per RAT. Value is 256. |
| maxnoofDCNs | Maximum no. of DCNs servered by one MME. Value is 32. |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.2.1.XX	Served DCNs Items

The Served DCNs Items indicates the relative processing capacity for a DCN-ID in the MME as defined in TS 23.401 [11].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Served DCNs Items |  |  |  |  |
| >DCN ID | M |  | OCTET STRING (SIZE(FFS)) |  |
| >Relative DCN Capacity | M |  | Relative MME Capacity 9.2.3.17 | Relatvie capacity per DCN in one MME |

### 9.3.3	PDU Definitions

-- **************************************************************

--

-- PDU definitions for S1AP.

--

-- **************************************************************

S1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

UEAggregateMaximumBitrate,

BearerType,

Cause,

CellAccessMode,

------------------------------------------

Skip to next change

------------------------------------------

MME-Group-ID,

Additional-GUTI,

PWSfailedECGIList,

CellIdentifierAndCELevelForCECapableUEs,

AssistanceDataForPaging,

InformationOnRecommendedCellsAndENBsForPaging,

UE-Usage-Type,

UEUserPlaneCIoTSupportIndicator,

NB-IoT-DefaultPagingDRX,

NB-IoT-Paging-eDRXInformation,

DCN-ID,

ServedDCNs

FROM S1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-SingleContainer{},

S1AP-PRIVATE-IES,

S1AP-PROTOCOL-EXTENSION,

S1AP-PROTOCOL-IES,

S1AP-PROTOCOL-IES-PAIR

------------------------------------------

Skip to next change

------------------------------------------

id-PWSfailedECGIList,

id-PWSFailureIndication,

id-UE-Usage-Type,

id-UEUserPlaneCIoTSupportIndicator,

id-NB-IoT-DefaultPagingDRX,

id-NB-IoT-Paging-eDRXInformation,

id-DCN-ID,

id-ServedDCNs

FROM S1AP-Constants;

------------------------------------------

Skip to next change

------------------------------------------

-- **************************************************************

--

##### -- INITIAL UE MESSAGE

--

-- **************************************************************

InitialUEMessage ::= SEQUENCE {

protocolIEs                     ProtocolIE-Container       {{InitialUEMessage-IEs}},

...

}

InitialUEMessage-IEs S1AP-PROTOCOL-IES ::= {

{ ID id-eNB-UE-S1AP-ID				CRITICALITY reject	TYPE ENB-UE-S1AP-ID					PRESENCE mandatory}|

{ ID id-NAS-PDU						CRITICALITY reject	TYPE NAS-PDU						PRESENCE mandatory}|

{ ID id-TAI							CRITICALITY reject	TYPE TAI							PRESENCE mandatory}|

{ ID id-EUTRAN-CGI					CRITICALITY ignore	TYPE EUTRAN-CGI						PRESENCE mandatory}|

{ ID id-RRC-Establishment-Cause		CRITICALITY ignore	TYPE RRC-Establishment-Cause		PRESENCE mandatory}|

{ ID id-S-TMSI						CRITICALITY reject	TYPE S-TMSI							PRESENCE optional}|

{ ID id-CSG-Id						CRITICALITY reject	TYPE CSG-Id							PRESENCE optional}|

{ ID id-GUMMEI-ID					CRITICALITY reject	TYPE GUMMEI							PRESENCE optional}|

{ ID id-CellAccessMode				CRITICALITY reject	TYPE CellAccessMode					PRESENCE optional}|

{ ID id-GW-TransportLayerAddress	CRITICALITY ignore	TYPE TransportLayerAddress			PRESENCE optional}|

{ ID id-RelayNode-Indicator			CRITICALITY reject	TYPE RelayNode-Indicator			PRESENCE optional}|

{ ID id-GUMMEIType					CRITICALITY ignore	TYPE GUMMEIType						PRESENCE optional}|

-- Extension for Release 11 to support BBAI --

{ ID id-Tunnel-Information-for-BBF	CRITICALITY ignore	TYPE TunnelInformation				PRESENCE optional}|

{ ID id-SIPTO-L-GW-TransportLayerAddress	CRITICALITY ignore	TYPE TransportLayerAddress	PRESENCE optional}|

{ ID id-LHN-ID						CRITICALITY ignore	TYPE LHN-ID							PRESENCE optional}|

{ ID id-MME-Group-ID				CRITICALITY ignore	TYPE MME-Group-ID					PRESENCE optional}|

{ ID id-UE-Usage-Type				CRITICALITY ignore	TYPE UE-Usage-Type					PRESENCE optional}|

{ ID id-DCN-ID						CRITICALITY ignore	TYPE DCN-ID							PRESENCE optional},

...

}

------------------------------------------

Skip to next change

------------------------------------------

-- **************************************************************

--

##### -- S1 Setup Response

--

-- **************************************************************

S1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {S1SetupResponseIEs} },

...

}

S1SetupResponseIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MMEname						CRITICALITY ignore	TYPE MMEname					PRESENCE optional}|

{ ID id-ServedGUMMEIs				CRITICALITY reject	TYPE ServedGUMMEIs				PRESENCE mandatory}|

{ ID id-RelativeMMECapacity			CRITICALITY ignore	TYPE RelativeMMECapacity		PRESENCE mandatory}|

{ ID id-MMERelaySupportIndicator	CRITICALITY ignore	TYPE MMERelaySupportIndicator	PRESENCE optional}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional}|

{ ID id-UE-RetentionInformation		CRITICALITY ignore	TYPE UE-RetentionInformation	PRESENCE optional}|

{ ID id-ServedDCNs					CRITICALITY ignore	TYPE ServedDCNs					PRESENCE optional},

...

}

------------------------------------------

Skip to next change

------------------------------------------

-- **************************************************************

--

##### -- MME Configuration Update

--

-- **************************************************************

MMEConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {MMEConfigurationUpdateIEs} },

...

}

MMEConfigurationUpdateIEs S1AP-PROTOCOL-IES ::= {

{ ID id-MMEname					CRITICALITY ignore	TYPE MMEname				PRESENCE optional	}|

{ ID id-ServedGUMMEIs			CRITICALITY reject	TYPE ServedGUMMEIs			PRESENCE optional	}|

{ ID id-RelativeMMECapacity		CRITICALITY reject	TYPE RelativeMMECapacity	PRESENCE optional	}|

{ ID id-ServedDCNs				CRITICALITY ignore	TYPE ServedDCNs				PRESENCE optional},

...

}

------------------------------------------

Skip to next change

------------------------------------------

### 9.3.4	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

S1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

------------------------------------------

Skip to next change

------------------------------------------

maxnoofCSGs,

maxnoofE-RABs,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofPLMNsPerMME,

maxnoofTACs,

maxnoofEPLMNs,

maxnoofEPLMNsPlusOne,

maxnoofForbLACs,

maxnoofForbTACs,

maxnoofCells,

maxnoofCellID,

maxnoofDCNs,

maxnoofEmergencyAreaID,

maxnoofTAIforWarning,

maxnoofCellinTAI,

maxnoofCellinEAI,

maxnoofeNBX2TLAs,

maxnoofeNBX2ExtTLAs,

maxnoofeNBX2GTPTLAs,

maxnoofRATs,

maxnoofGroupIDs,

maxnoofMMECs,

maxnoofTAforMDT,

maxnoofCellIDforMDT,

maxnoofMDTPLMNs,

maxnoofCellsforRestart,

maxnoofRestartTAIs,

maxnoofRestartEmergencyAreaIDs,

maxnoofMBSFNAreaMDT,

maxEARFCN,

maxnoofCellsineNB,

maxnoofRecommendedCells,

maxnoofRecommendedENBs

------------------------------------------

Skip to next change

------------------------------------------

#### -- D

DataCodingScheme ::= BIT STRING (SIZE (8))

DCN-ID ::= OCTET STRING (SIZE(FFS))

ServedDCNs ::= SEQUENCE (SIZE(0..maxnoofDCNs)) OF ServedDCNsItem

ServedDCNsItem ::= SEQUENCE {

dCN-ID						DCN-ID,

relativeDCNCapacity			RelativeMMECapacity,

iE-Extensions			ProtocolExtensionContainer { {ServedDCNsItem-ExtIEs} }	OPTIONAL,

...

}

ServedDCNsItem-ExtIEs S1AP-PROTOCOL-EXTENSION ::= {

...

}

DL-Forwarding ::= ENUMERATED {

dL-Forwarding-proposed,

...

}

Direct-Forwarding-Path-Availability ::= ENUMERATED {

directPathAvailable,

...

}

Data-Forwarding-Not-Possible ::= ENUMERATED {

data-Forwarding-not-Possible,

...

}

#### -- E

------------------------------------------

Skip to next change

------------------------------------------

### 9.3.6	Constant Definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

S1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) s1ap (1) version1 (1) s1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

------------------------------------------

Skip to next change

------------------------------------------

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxnoofCSGs								INTEGER ::= 256

maxnoofE-RABs							INTEGER ::= 256

maxnoofTAIs								INTEGER ::= 256

maxnoofTACs								INTEGER ::= 256

maxnoofErrors							INTEGER ::= 256

maxnoofBPLMNs							INTEGER ::= 6

maxnoofPLMNsPerMME						INTEGER ::= 32

maxnoofEPLMNs							INTEGER ::= 15

maxnoofEPLMNsPlusOne					INTEGER ::= 16

maxnoofForbLACs							INTEGER ::= 4096

maxnoofForbTACs							INTEGER ::= 4096

maxnoofIndividualS1ConnectionsToReset	INTEGER ::= 256

maxnoofCells							INTEGER ::= 16

maxnoofCellsineNB						INTEGER ::= 256

maxnoofTAIforWarning					INTEGER ::= 65535

maxnoofCellID							INTEGER ::= 65535

maxnoofDCNs								INTEGER ::= 32

maxnoofEmergencyAreaID					INTEGER ::= 65535

maxnoofCellinTAI						INTEGER ::= 65535

maxnoofCellinEAI						INTEGER ::= 65535

maxnoofeNBX2TLAs						INTEGER ::= 2

maxnoofeNBX2ExtTLAs						INTEGER ::= 16

maxnoofeNBX2GTPTLAs						INTEGER ::= 16

maxnoofRATs								INTEGER ::= 8

maxnoofGroupIDs							INTEGER ::= 65535

maxnoofMMECs							INTEGER ::= 256

maxnoofCellIDforMDT						INTEGER ::= 32

maxnoofTAforMDT							INTEGER ::= 8

maxnoofMDTPLMNs							INTEGER ::= 16

maxnoofCellsforRestart					INTEGER ::= 256

maxnoofRestartTAIs						INTEGER ::= 2048

maxnoofRestartEmergencyAreaIDs			INTEGER ::= 256

maxEARFCN								INTEGER ::= 262143

maxnoofMBSFNAreaMDT						INTEGER ::= 8

maxnoofRecommendedCells					INTEGER ::= 16

maxnoofRecommendedENBs					INTEGER ::= 16

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

------------------------------------------

Skip to next change

------------------------------------------

id-UE-RetentionInformation							ProtocolIE-ID ::= 228

id-UE-Usage-Type									ProtocolIE-ID ::= 230

id-extended-UEIdentityIndexValue					ProtocolIE-ID ::= 231

id-RAT-Type											ProtocolIE-ID ::= 232

id-BearerType										ProtocolIE-ID ::= 233

id-NB-IoT-DefaultPagingDRX							ProtocolIE-ID ::= 234

id-E-RABFailedToResumeListResumeReq					ProtocolIE-ID ::= 235

id-E-RABFailedToResumeItemResumeReq					ProtocolIE-ID ::= 236

id-E-RABFailedToResumeListResumeRes					ProtocolIE-ID ::= 237

id-E-RABFailedToResumeItemResumeRes					ProtocolIE-ID ::= 238

id-NB-IoT-Paging-eDRXInformation					ProtocolIE-ID ::= 239

id-UEUserPlaneCIoTSupportIndicator 					ProtocolIE-ID ::= 241

id-CE-mode-B-SupportIndicator 						ProtocolIE-ID ::= 242

id-SRVCCOperationNotPossible						ProtocolIE-ID ::= 243

id-NB-IoT-UEIdentityIndexValue 						ProtocolIE-ID ::= 244

id-DCN-ID											ProtocolIE-ID ::= XXX

id-ServedDCNs					 					ProtocolIE-ID ::= XXX+1

END