3GPP TSG RAN WG3 #95	                                    			R3-170708

Athens, Greece, 13th – 17th February 2017

Agenda Item:	   28.2

Source: 	   Ericsson

Title: 	   TP on conclusions for long backhaul delays issue

Document for:	Discussion and Decision

# Introduction

In R3-170707 a description of how the issue on long backhaul delays reported in TR36.933 can be solved has been presented. This TP for TR36.933 captures the aspects described in that paper.

# Text Proposal to TR36.933

------------------------------------------First Change------------------------------------------

### 5.5.2 	Solution 2: Static acceptance or rejection of adaptation of LTE radio resource allocation for the call

At multimedia call establishment, the E-UTRAN can inform IMS multimedia gateway or other entities in the Core network about its capability of receiving and handling accordingly, a notification from UE indicating critical multimedia data is pending to be sent on UE bearer’s queue. The IMS multimedia gateway or other entities in the Core network may then decide to send an indication to the E-UTRAN to accept or reject the notification from UE. This will be based on its own policy (e.g. depending on user subscription, network load, etc.) and applied to all notifications received from UE.

Editor’s note: It is FFS about how to validate the UE’s notification to protect the network against malicious UEs that request for prioritized resource allocation from the E-UTRAN.

### 6 	Conclusions

In order to address the issue on backhaul long latency it is concluded that the principles in section 5.1 of TR36.933 need to be respected. A solution based on local caching is feasible provided that the locally cached content to be delivered to the UE can be handled by local CN functionalities, currently standardised, supporting the functions listed in section 5.1 of this TR and necessary for establishing communication and deliver content to a UE.

------------------------------------------End of Changes------------------------------------------