3GPP TSG-RAN WG3 Meeting # 95	R3-170854

Athens, Greece, Feb. 13th – 17th, 2017

Agenda item:		10.6.1

Source:	LG Electronics Inc.

Title:	Corrections on Option 3/3a/3x

Document for:		Discussion

1. Text Proposal

----------------Start of the Change---------------

10.1.2.4.1	Evaluation on SCG split bearer

The following evaluation can be considered for the analysis of this bearer type:

-	Signalling between Master node and Secondary node

-	The backhaul of Xx interface

-	The Xx interface has to offer sufficient capacity to cope with LTE bitrates.

-	Signalling  to CN due to mobility in/out of Secondary node coverage

The following evaluation can be considered for the analysis of this bearer type:

Signalling between Master node and Secondary node

The following additional functions need to be supported on top of LTE Dual Connectivity specified in TS 36.300 [12].

-	LTE eNB (MeNB) provides its tunnel endpoint information in SENB ADDITION REQUEST or SENB MODIFICATION REQUEST message to receive DL data transmission from the gNB;

-	gNB (SeNB) provides its tunnel endpoint information in SENB ADDITION REQUEST ACKNOWLEDGE or SENB MODIFICATION REQUEST ACKNOWLEDGE message to receive UL data transmission from the LTE eNB;

-	Flow control procedure needs to be applied to the other direction, i.e. LTE eNB signals DL DATA DELIVERY STATUS to the gNB, and the gNB signals DL USER DATA to the LTE eNB.

There is no difference on the number of signalling messages between SCG bearer and SCG split bearer with some enhancements to LTE DC specification.

----------------End of the Change---------------