3GPP TSG-RAN WG3 Meeting #95bis	R3-171178

# Spokane, USA, 3rd -7th  April 2017

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 25.413 | CR | 1319 | rev |  | Current version: | 13.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Application Layer Measurement Failure Handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | UMTS_QMC_Streaming-Core |  |  |  |  |  |  | Date: |  | 2017-03-17 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | There is no failure handling for UE Application Layer Measurement failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new UE Application Layer Measurement Failure Reporting |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The target RNC may receive QoE measruement reporting which it does not wish to support |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X.Y, X.Y.1, X.Y.2, X.Y.3, 9.1.X, 9.3.2, 9.3.3, 9.3.4, 9.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | 25.331 CR 5931 |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## X.Y	UE Application Layer Measurement Failure Reporting

### X.Y.1	General

The UE Application Layer Measurement Failure Reporting procedure is initiated by a RNC to report detected errors in the initiated UE Application Layer Measurement.

The UE Application Layer Measurement Failure Reporting procedure uses connection oriented signalling..

### X.Y.2	Successful Operation

Figure XX: UE Application Layer Measurement Failure Reporting procedure. Successful operation.

When the UE Application Layer Measurement fails, the UE Application Layer Measurement Failure Reporting procedure is initiated by an UE APPLICATION LAYER MEASUREMENT FAILURE REPORTING message sent from RNC.

The UE APPLICATION LAYER MEASUREMENT FAILURE REPORTING message shall contain at least the FailureCause IE.

### X.Y.3	Abnormal Conditions

Not applicable.

### 9.1.X	UE APPLICATION LAYER MEASUREMENT FAILURE REPORTING

This message is sent by the RNC to report to the CN the UE Application Layer Measurement failure.

Direction: RNC  CN.

Signalling bearer mode: Connection oriented.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.1.1 |  | YES | ignore |
| Failure Cause | M |  | ENUMERATED (configuration unsupported, unsupported measurement, invalid application layer measurement container, ...) |  | YES | ignore |

### 9.3.2	Elementary Procedure Definitions

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

RANAP-PDU-Descriptions {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

umts-Access (20) modules (3) ranap (0) version1 (1) ranap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode

FROM RANAP-CommonDataTypes

Iu-ReleaseCommand,

Iu-ReleaseComplete,

RelocationCommand,

RelocationPreparationFailure,

RelocationRequired,

RelocationRequest,

RelocationRequestAcknowledge,

RelocationFailure,

RelocationCancel,

------------------------------------------

Skip to next change

------------------------------------------

MBMSRABReleaseRequest,

MBMSRABRelease,

MBMSRABReleaseFailure,

SRVCC-CSKeysRequest,

SRVCC-CSKeysResponse,

UeApplicationLayerMeasurementFailureReporting,

UeRadioCapabilityMatchRequest,

UeRadioCapabilityMatchResponse,

UeRegistrationQueryRequest,

UeRegistrationQueryResponse,

RerouteNASRequest

FROM RANAP-PDU-Contents

id-LocationRelatedData,

id-CN-DeactivateTrace,

id-CN-InvokeTrace,

id-CommonID,

------------------------------------------

Skip to next change

------------------------------------------

id-MBMSSessionStop,

id-MBMSUELinking,

id-MBMSRegistration,

id-MBMSCNDe-Registration-Procedure,

id-MBMSRABEstablishmentIndication,

id-MBMSRABRelease,

id-SRVCCPreparation,

id-UeApplicationLayerMeasurementFailureReporting,

id-UeRadioCapabilityMatch,

id-UeRegistrationQuery,

id-RerouteNASRequest

FROM RANAP-Constants;

-- **************************************************************

--

#### -- Interface Elementary Procedure Class

--

-- **************************************************************

RANAP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome				OPTIONAL,

&UnsuccessfulOutcome				OPTIONAL,

&Outcome					OPTIONAL,

&procedureCode			ProcedureCode 	UNIQUE,

&criticality			Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE		&InitiatingMessage

[SUCCESSFUL OUTCOME		&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

[OUTCOME			&Outcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY			&criticality]

}

-- **************************************************************

--

#### -- Interface PDU Definition

--

-- **************************************************************

RANAP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

outcome			Outcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	RANAP-ELEMENTARY-PROCEDURE.&procedureCode	({RANAP-ELEMENTARY-PROCEDURES}),

criticality	RANAP-ELEMENTARY-PROCEDURE.&criticality		({RANAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value		RANAP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({RANAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	RANAP-ELEMENTARY-PROCEDURE.&procedureCode	({RANAP-ELEMENTARY-PROCEDURES}),

criticality	RANAP-ELEMENTARY-PROCEDURE.&criticality		({RANAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value		RANAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({RANAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	RANAP-ELEMENTARY-PROCEDURE.&procedureCode	({RANAP-ELEMENTARY-PROCEDURES}),

criticality	RANAP-ELEMENTARY-PROCEDURE.&criticality		({RANAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value		RANAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({RANAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

Outcome ::= SEQUENCE {

procedureCode	RANAP-ELEMENTARY-PROCEDURE.&procedureCode	({RANAP-ELEMENTARY-PROCEDURES}),

criticality	RANAP-ELEMENTARY-PROCEDURE.&criticality		({RANAP-ELEMENTARY-PROCEDURES}{@procedureCode}),

value		RANAP-ELEMENTARY-PROCEDURE.&Outcome		({RANAP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

-- **************************************************************

--

#### -- Interface Elementary Procedure List

--

-- **************************************************************

RANAP-ELEMENTARY-PROCEDURES RANAP-ELEMENTARY-PROCEDURE ::= {

RANAP-ELEMENTARY-PROCEDURES-CLASS-1	|

RANAP-ELEMENTARY-PROCEDURES-CLASS-2	|

RANAP-ELEMENTARY-PROCEDURES-CLASS-3	,

...

}

RANAP-ELEMENTARY-PROCEDURES-CLASS-1 RANAP-ELEMENTARY-PROCEDURE ::= {

iu-Release			|

relocationPreparation		|

relocationResourceAllocation	|

relocationCancel		|

sRNS-ContextTransfer		|

securityModeControl		|

dataVolumeReport		|

reset		|

resetResource				,

...,

locationRelatedData		|

informationTransfer		|

uplinkInformationExchange	|

mBMSSessionStart			|

mBMSSessionUpdate			|

mBMSSessionStop				|

mBMSUELinking				|

mBMSRegistration			|

mBMSCNDe-Registration		|

mBMSRABRelease				|

enhancedRelocationComplete	|

rANAP-enhancedRelocation	|

sRVCCPreparation			|

ueRadioCapabilityMatch		|

ueRegistrationQuery }

RANAP-ELEMENTARY-PROCEDURES-CLASS-2 RANAP-ELEMENTARY-PROCEDURE ::= {

rAB-ReleaseRequest		|

iu-ReleaseRequest		|

relocationDetect		|

relocationComplete		|

paging					|

commonID				|

cN-InvokeTrace			|

cN-DeactivateTrace		|

locationReportingControl	|

locationReport			|

initialUE-Message		|

directTransfer			|

overloadControl			|

errorIndication			|

sRNS-DataForward		|

forwardSRNS-Context		|

privateMessage			|

rANAP-Relocation		,

...,

rAB-ModifyRequest		|

uESpecificInformation	|

directInformationTransfer	|

mBMSRABEstablishmentIndication	|

enhancedRelocationCompleteConfirm	|

rerouteNASRequest|

ueApplicationLayerMeasurementFailureReporting

}

RANAP-ELEMENTARY-PROCEDURES-CLASS-3 RANAP-ELEMENTARY-PROCEDURE ::= {

rAB-Assignment			,

...

}

-- **************************************************************

--

#### -- Interface Elementary Procedures

--

-- **************************************************************

iu-Release RANAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE	Iu-ReleaseCommand

SUCCESSFUL OUTCOME	Iu-ReleaseComplete

PROCEDURE CODE		id-Iu-Release

CRITICALITY		reject

}

------------------------------------------

Skip to next change

------------------------------------------

ueApplicationLayerMeasurementFailureReporting RANAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE	UeApplicationLayerMeasurementFailureReporting

PROCEDURE CODE		id-UeApplicationLayerMeasurementFailureReporting

CRITICALITY			ignore

}

ueRadioCapabilityMatch RANAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UeRadioCapabilityMatchRequest

OUTCOME					UeRadioCapabilityMatchResponse

PROCEDURE CODE			id-UeRadioCapabilityMatch

CRITICALITY				ignore

}

ueRegistrationQuery RANAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE		UeRegistrationQueryRequest

OUTCOME					UeRegistrationQueryResponse

PROCEDURE CODE			id-UeRegistrationQuery

CRITICALITY				ignore

}

rerouteNASRequest RANAP-ELEMENTARY-PROCEDURE ::= {

INITIATING MESSAGE	RerouteNASRequest

PROCEDURE CODE		id-RerouteNASRequest

CRITICALITY			reject

}

END

### 9.3.3	PDU Definitions

-- **************************************************************

--

-- PDU definitions for RANAP.

--

-- **************************************************************

RANAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

umts-Access (20) modules (3) ranap (0) version1 (1) ranap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

AccuracyFulfilmentIndicator,

APN,

BroadcastAssistanceDataDecipheringKeys,

LocationRelatedDataRequestType,

LocationRelatedDataRequestTypeSpecificToGERANIuMode,

DataVolumeReference,

CellLoadInformation,

AreaIdentity,

CN-DomainIndicator,

Cause,

Cell-Access-Mode,

ClientType,

CriticalityDiagnostics,

ChosenEncryptionAlgorithm,

ChosenIntegrityProtectionAlgorithm,

ClassmarkInformation2,

ClassmarkInformation3,

CSG-Id,

CSG-Id-List,

CSG-Membership-Status,

DL-GTP-PDU-SequenceNumber,

DL-N-PDU-SequenceNumber,

DataVolumeReportingIndication,

DeltaRAListofIdleModeUEs,

DRX-CycleLengthCoefficient,

EncryptionInformation,

EncryptionKey,

E-UTRAN-Service-Handover,

ExtendedRNC-ID,

FailureCause,

FrequenceLayerConvergenceFlag,

GERAN-BSC-Container,

GERAN-Classmark,

GlobalCN-ID,

GlobalRNC-ID,

GTP-TEI,

IncludeVelocity,

InformationExchangeID,

InformationExchangeType,

------------------------------------------

Skip to next change

------------------------------------------

TunnelInformation,

LHN-ID,

Session-Re-establishment-Indicator,

Additional-CSPS-coordination-information,

UERegistrationQueryResult,

SGSN-Group-Identity,

P-TMSI,

BarometricPressure,

CivicAddress,

UE-Usage-Type

FROM RANAP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-ContainerList{},

ProtocolIE-ContainerPair{},

ProtocolIE-ContainerPairList{},

ProtocolIE-Container{},

RANAP-PRIVATE-IES,

RANAP-PROTOCOL-EXTENSION,

RANAP-PROTOCOL-IES,

RANAP-PROTOCOL-IES-PAIR

FROM RANAP-Containers

maxNrOfDTs,

maxNrOfErrors,

maxNrOfIuSigConIds,

maxNrOfRABs,

maxNrOfVol,

maxnoofMulticastServicesPerUE,

id-AccuracyFulfilmentIndicator,

id-APN,

id-AreaIdentity,

id-Alt-RAB-Parameters,

id-Ass-RAB-Parameters,

id-BroadcastAssistanceDataDecipheringKeys,

id-LocationRelatedDataRequestType,

id-CN-DomainIndicator,

id-Cause,

id-Cell-Access-Mode,

id-ChosenEncryptionAlgorithm,

id-ChosenIntegrityProtectionAlgorithm,

id-ClassmarkInformation2,

id-ClassmarkInformation3,

id-ClientType,

id-CNMBMSLinkingInformation,

id-CriticalityDiagnostics,

id-CSG-Id,

id-CSG-Id-List,

id-CSG-Membership-Status,

id-DeltaRAListofIdleModeUEs,

id-DRX-CycleLengthCoefficient,

id-DirectTransferInformationItem-RANAP-RelocInf,

id-DirectTransferInformationList-RANAP-RelocInf,

id-DL-GTP-PDU-SequenceNumber,

id-EncryptionInformation,

id-EncryptionKey,

id-ExtendedRNC-ID,

id-FailureCause,

id-FrequenceLayerConvergenceFlag,

id-GERAN-BSC-Container,

id-GERAN-Classmark,

id-GERAN-Iumode-RAB-Failed-RABAssgntResponse-Item,

id-GERAN-Iumode-RAB-FailedList-RABAssgntResponse,

id-GlobalCN-ID,

id-GlobalCN-IDCS,

id-GlobalCN-IDPS,

id-GlobalRNC-ID,

id-IncludeVelocity,

id-InformationExchangeID,

------------------------------------------

Skip to next change

------------------------------------------

-- **************************************************************

--

##### -- Uplink Information Exchange Failure

--

-- **************************************************************

UplinkInformationExchangeFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container			{ { UplinkInformationExchangeFailureIEs} },

protocolExtensions		ProtocolExtensionContainer	{ { UplinkInformationExchangeFailureExtensions} }				OPTIONAL,

...

}

UplinkInformationExchangeFailureIEs RANAP-PROTOCOL-IES ::= {

{ ID id-InformationExchangeID		CRITICALITY ignore	TYPE InformationExchangeID		PRESENCE mandatory	} |

{ ID id-CN-DomainIndicator			CRITICALITY ignore	TYPE CN-DomainIndicator			PRESENCE mandatory	} |

{ ID id-GlobalCN-ID					CRITICALITY ignore	TYPE GlobalCN-ID				PRESENCE optional		} |

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	} |

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional		} ,

...

}

UplinkInformationExchangeFailureExtensions RANAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- UE APPLICATION LAYER MEASUREMENT FAILURE REPORTING ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- UE APPLICATION LAYER MEASUREMENT FAILURE REPORTING

--

-- **************************************************************

UeApplicationLayerMeasurementFailureReporting ::= SEQUENCE {

protocolIEs				ProtocolIE-Container		{ { UeApplicationLayerMeasurementFailureReportingIEs} },

protocolExtensions		ProtocolExtensionContainer	{ { UeApplicationLayerMeasurementFailureReportingExtensions} }				OPTIONAL,

...

}

UeApplicationLayerMeasurementFailureReportingIEs RANAP-PROTOCOL-IES ::= {

{ ID id-FailureCause					CRITICALITY ignore	TYPE FailureCause						PRESENCE optional	},

...

}

UeApplicationLayerMeasurementFailureReportingExtensions RANAP-PROTOCOL-EXTENSION ::= {

...

}

-- **************************************************************

--

#### -- MBMS SESSION START PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- MBMS Session Start

--

-- **************************************************************

MBMSSessionStart ::= SEQUENCE {

protocolIEs				ProtocolIE-Container		{ { MBMSSessionStartIEs} },

protocolExtensions		ProtocolExtensionContainer	{ { MBMSSessionStartExtensions} }			OPTIONAL,

...

}

------------------------------------------

Skip to next change

------------------------------------------

### 9.3.4	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

RANAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

umts-Access (20) modules (3) ranap (0) version1 (1) ranap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

maxNrOfCSGs,

maxNrOfErrors,

maxNrOfPDPDirections,

maxNrOfPoints,

maxNrOfRABs,

maxNrOfSRBs,

maxNrOfSeparateTrafficDirections,

maxRAB-Subflows,

------------------------------------------

Skip to next change

------------------------------------------

#### -- F

FrameSequenceNumber		::=	INTEGER(0..15)

FrequenceLayerConvergenceFlag ::= ENUMERATED {

no-FLC-flag,

...

}

FailureCause ::= ENUMERATED {

configuration unsupported,

unsupported measurement,

invalid application layer measurement container,

...

}

#### -- G

### 9.3.6	Constant Definitions

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

RANAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

umts-Access (20) modules (3) ranap (0) version1 (1) ranap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-RAB-Assignment					INTEGER ::= 0

id-Iu-Release						INTEGER ::= 1

id-RelocationPreparation			INTEGER ::= 2

id-RelocationResourceAllocation		INTEGER ::= 3

id-RelocationCancel					INTEGER ::= 4

id-SRNS-ContextTransfer				INTEGER ::= 5

------------------------------------------

Skip to next change

------------------------------------------

id-MBMSRegistration					INTEGER ::= 39

id-MBMSCNDe-Registration-Procedure	INTEGER ::= 40

id-MBMSRABEstablishmentIndication	INTEGER ::= 41

id-MBMSRABRelease					INTEGER ::= 42

id-enhancedRelocationComplete		INTEGER ::= 43

id-enhancedRelocationCompleteConfirm	INTEGER ::= 44

id-RANAPenhancedRelocation			INTEGER ::= 45

id-SRVCCPreparation					INTEGER ::= 46

id-UeRadioCapabilityMatch					INTEGER ::= 47

id-UeRegistrationQuery						INTEGER ::= 48

id-RerouteNASRequest						INTEGER ::= 49

id-UeApplicationLayerMeasurementFailureReporting						INTEGER ::= xx

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs 						INTEGER ::= 65535

maxProtocolExtensions 				INTEGER ::= 65535

maxProtocolIEs						INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNrOfDTs							INTEGER ::= 15

maxNrOfErrors						INTEGER ::= 256

maxNrOfIuSigConIds					INTEGER ::= 250

maxNrOfPDPDirections				INTEGER ::= 2

maxNrOfPoints						INTEGER ::= 15

maxNrOfRABs							INTEGER ::= 256

maxNrOfSeparateTrafficDirections	INTEGER ::= 2

maxNrOfSRBs							INTEGER ::= 8

maxNrOfVol							INTEGER ::= 2

maxNrOfLevels						INTEGER ::= 256

maxNrOfAltValues					INTEGER ::= 16

maxNrOfPLMNsSN						INTEGER ::= 32

maxNrOfLAs							INTEGER ::= 65536

maxNrOfSNAs							INTEGER ::= 65536

maxNrOfUEsToBeTraced				INTEGER ::= 64

maxNrOfInterfaces					INTEGER ::= 16

maxRAB-Subflows						INTEGER ::= 7

maxRAB-SubflowCombination			INTEGER ::= 64

maxSet								INTEGER ::= 9

maxNrOfHSDSCHMACdFlows-1			INTEGER	::= 7

maxnoofMulticastServicesPerUE		INTEGER ::= 128

maxnoofMulticastServicesPerRNC		INTEGER ::= 512

maxMBMSSA							INTEGER ::= 256

maxMBMSRA							INTEGER ::= 65536

maxNrOfEDCHMACdFlows-1				INTEGER ::= 7

maxGANSSSet							INTEGER ::= 9

maxNrOfCSGs							INTEGER ::= 256

maxNrOfEUTRAFreqs					INTEGER ::= 8

maxNrOfCellIds						INTEGER ::= 32

maxNrOfRAIs							INTEGER ::= 8

maxNrOfLAIs							INTEGER ::= 8

maxSizeOfIMSInfo					INTEGER ::= 32

maxnoofMDTPLMNs						INTEGER ::= 16

maxAddPosSet						INTEGER ::= 8

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-AreaIdentity 								INTEGER ::= 0

id-CN-DomainIndicator 							INTEGER ::= 3

id-Cause 										INTEGER ::= 4

id-ChosenEncryptionAlgorithm 					INTEGER ::= 5

id-ChosenIntegrityProtectionAlgorithm 			INTEGER ::= 6

id-ClassmarkInformation2 						INTEGER ::= 7

id-ClassmarkInformation3 						INTEGER ::= 8

id-CriticalityDiagnostics 						INTEGER ::= 9

------------------------------------------

Skip to next change

------------------------------------------

id-SGSN-Group-Identity									INTEGER ::= 286

id-P-TMSI												INTEGER ::= 287

id-RANAP-Message										INTEGER ::= 288

id-PowerSavingIndicator									INTEGER ::= 289

id-UE-Usage-Type										INTEGER ::= 290

id-FailureCause											INTEGER ::= XXX

END