3GPP TSG-RAN WG3 Meeting #95bis	R3-171181

Spokane, USA, 3rd – 7th April 2017

Agenda Item:	31.1

Source:	Ericsson

Title:	Introduction of UE Application Layer Measurement Capability

Document for:	Discussions & Approval

# 1	Background

At RAN3#95 meeting the work item QoE [1] was completed.

This paper discusses how to avoid unnecessarily signalling from CN to RAN for the initiation of the UE Application Measurement.

# 2	Discussion

It has been agreed that the supporting of the UE Application Layer Measurement will be introduced in RAN2, in the Measurement Capability.

However when CN is initiating the UE Application Layer Measurement, it does not have the knowledge that for the specific UE, if the measurement is supported or not. It will initiate the measurement in CN INVOKE TRACE, which contains a big sized configuration container, and then it is up to RAN to decide if to start the measurement or not due to the UE measurement capability.

We thus propose that RNC to send the UE Application Layer Measurement to CN as soon as this information is available in RAN.

Proposal 1: RAN3 to agree to include UE Application Layer Measurement Capability in INITIAL UE MESSAGE.

CR is submitted in [2].

# 3	Proposals

Proposal 1: RAN3 to agree to include UE Application Layer Measurement Capability in INITIAL UE MESSAGE.

# 4	Reference

- [1]	RP-161917	New WI proposal: Quality of Experience (QoE) Measurement Collection for streaming services in UTRAN, China Unicom

- [2] 	R3-171182	Introduction of UE Application Layer Measurement Capability, Ericsson