3GPP TSG-RAN WG3 Meeting #96	R3-171751

Hangzhou, China, 15th – 19th May 2017

Agenda Item:	31.1

Source:	Ericsson

Title:	Correction on Dual Cell E-DCH operation enhancements

Document for:	Discussions & Approval

# 1	Background

During the review work, we detect the incorrect implementation of the abnormal case related to the DTX Information in the Dual Cell E-DCH operation enhancement feature.

This paper collects the errors that should be corrected.

# 2	Discussion

We have agreed that when setting up Dual Cell E-DCH operation enhancements configuration, the DTX Information for both carriers should use the set of values for 10ms TTI. Failing to do so would result the procedure failure. Thus abnormal case has been introduced in the dedicated Radio Link procedure to realize the requirement. The below errors are detected in the current specification:

- DTX Information IE is mandatory in the Continuous Packet Connectivity DTX-DRX Information IE, thus the current procedural text “If the Dual Cell E-DCH operation enhancements configuration is setup but the DTX Information, if included in the Continuous Packet Connectivity DTX-DRX Information IE” is incorrect. However DTX related information is optional present in some other procedure. This is not clearly reflected in the procedural text.

- DTX related Information is called differently and is included in different IEs when different Radio Link handling procedure is used, for example. This is not clearly reflected in the procedural text.

- As the Information Element who includes the DTX Information is optional present, during Radio Link Addition and Radio Link Reconfiguration procedure, when Dual Cell E-DCH operation enhancements configuration is setup or configured, the DTX Information may not present. According to the protocol rule, the early signalled DTX Information will be kept and used (unless it is explicitly modified or removed.) The abnormal case for this situation is missing and may result ambiguous network implementation.

Proposal 1: RAN3 to discuss and agree on the corrections included in [1] and [2].

# 3	Proposals

Proposal 1: RAN3 to discuss and agree on the corrections included in [1] and [2].

# 4	Reference

- [1]	R3-171752	Correction of the abnormal case in Dual Cell E-DCH operation enhancements configuration, Ericsson

- [2]	R3-171753	Correction on Dual Cell E-DCH operation enhancements, Ericsson