3GPP TSG-RAN Meeting #97-bis 	R3-174205

# Prague, Czechia, EU, 9-13 Oct 2017

Agenda item:	10.8.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Completion of the RRC tunnelling in MR-DC

Document for:	Decission

# 1	Introduction

At RAN3 meeting #97-bis, corrections to the RRC transfer has been discussed [1]. This document provides the necessary changes to the baseline CR to TS 38.423.

# References

- R3-174132, RAN3 #97-bis

# Text proposal for baseline CR for TS 38.423

First change, omitted text not changed

### 8.3.8	RRC Transfer

#### 8.3.8.1	General

The purpose of the RRC Transfer procedure is to deliver an LTE RRC message encapsulated in a PDCP-C PDU to the S-NG-RAN-NODE that it may then be forwarded to the UE, or from the S-NG-RAN-NODE, if it was received from the UE. Delivery status may also be provided from the S-NG-RAN-NODE to the M-NG-RAN-NODE using the RRC Transfer [Whether to use RRC transfer to provide delivery status is FFS].

The procedure is also to enable transfer of the NR RRC message container with the NR measurements from the M-NG-RAN-NODE to the S-NG-RAN-NODE, when received from the UE.

The procedure uses UE-associated signalling.

#### 8.3.8.2	Successful Operation

Figure 8.X.Y.2-1: RRC Transfer procedure, successful operation.

The M-NG-RAN-NODE initiates the procedure by sending the RRC TRANSFER message to the S-NG-RAN-NODE or the S-NG-RAN-NODE initiates the procedure by sending the RRC TRANSFER message to the M-NG-RAN-NODE.

#### 8.3.8.3	Abnormal Conditions

In case of the MCG split SRBs, the receiving node may ignore the message, if the M-NG-RAN-NODE has not indicated possibility of RRC transfer at the bearer setup.

Next change, omitted text not changed

#### 9.1.2.15	RRC TRANSFER

This message is sent by the M-NG-RAN-NODE to the S-NG-RAN-NODE to transfer an RRC message.

Direction: M-NG-RAN node  S-NG-RAN node or S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the S-NG-RAN node | YES | reject |
| MCG split SRB |  | 0..1 |  |  |  |  |
| > RRC Container | M |  | OCTET STRING | RRC message encapsulated in a PDCP-C PDU and ciphered with the key of the MeNB | YES | reject |
| > SRB Type | M |  | ENUMERATED (srb1, srb2, ...) | The SRB type | YES | reject |
| > Delivery status | O |  | [FFS] |  | YES | reject |
| NR UE Measurement Report |  | 0..1 |  |  |  |  |
| > RRC Container | M |  | OCTET STRING | NR UE Measurement report (see TS 38.331 [X]) | YES | reject |

Remaining text not changed