3GPP TSG-RAN WG3 #98	R3-174321

Reno, US, 27 Nov – 1 Dec 2017

Agenda item:	10.8.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Switching of the UL link

Document for:	Discussion and Decision

# 1	Introduction

In [1], the solution needed to address the problem provided to RAN3 in LS from RAN2 [2] is proposed. It is applicable to Xn, too. This TP provides the needed changes in the XnAP.

# References

- R3-174319, RAN3 #98

- R2-1712042, RAN2 #99-bis

# Text proposal for TS 38.423

The changes below are proposed to be incorporated in the BL CR for TS 36.423.

First change, ommited text not changed

#### 9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <reference> |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | C- ifSCGorSCGsplitBearerOption |  | <reference> |  | YES | reject |
| SgNB Security Key | C-ifSCGorSCGsplitBearerOption |  | <reference> | The S-KgNB which is provided by the M-NG-RAN node, see xxx. Editor’s Note: terminology “S-KgNB” to be fixed with SA3 and RAN2 | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate <reference> | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Serving PLMN | O |  | PLMN Identity <reference> | The serving PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Handover Restriction List | O |  | <reference> |  | YES | ignore |
| PDU sessions To Be Added List |  | 1 |  |  | YES | reject |
| >PDU sessions To Be Added Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | reject |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>SCG Bearer |  |  |  |  |  |  |
| >>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>S-NSSAI | O |  | <reference> |  | – | – |
| >>>>PDU session Level QoS Parameters | M |  | <reference> | Includes necessary QoS parameters | – | – |
| >>>>DL Forwarding | O |  | <reference> |  | – | – |
| >>>>NG UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | UPF endpoint of the NG transport bearer. For delivery of UL PDUs. | – | – |
| >>>>SIPTO Correlation ID | O |  | Correlation ID <reference> |  | – | – |
| >>>Split Bearer |  |  |  |  |  |  |
| >>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>S-NSSAI | O |  | <reference> |  | – | – |
| >>>>PDU session Level QoS Parameters | M |  | <reference> | Includes necessary QoS parameters | – | – |
| >>>>M-NG-RAN node GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | M-NG-RAN node endpoint of the Xn transport bearer. For delivery of UL PDUs. | – | – |
| >>>>UL configuration | M |  | <reference> | Information about UL usage in the S-NG-RAN node | – | – |
| >>>SCG split Bearer |  |  |  |  |  |  |
| >>>>SCG split Bearer S-NG-RAN node Addition Item |  | 1 |  |  | YES | reject |
| >>>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>>S-NSSAI | O |  | <reference> |  | – | – |
| >>>>>Total PDU session Level QoS Parameters | M |  | PDU session Level QoS Parameters <reference> | Includes the PDU session Level QoS parameters as received on NG-AMF for the PDU session | – | – |
| >>>>>M-NG-RAN node PDU session Level QoS Parameters | O |  | PDU session Level QoS Parameters <reference> | Includes, if applicable, the share the M-NG-RAN node is offering to take | – | – |
| >>>>>DL Forwarding | O |  | <reference> |  | – | – |
| >>>>>M-NG-RAN node GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | M-NG-RAN node endpoint of the Xn transport bearer. For delivery of DL PDUs. | – | – |
| >>>>>NG UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | UPF endpoint of the NG transport bearer. For delivery of UL PDUs. | – | – |
| >>>>>SIPTO Correlation ID | O |  | Correlation ID <reference> |  | – | – |
| M-NG-RAN node to S-NG-RAN node Container | M |  | OCTET STRING | Includes the SCG-ConfigInfo message as defined in xxx Editor’s Note: to be checked with RAN2 | YES | reject |
| S-NG-RAN node UE XnAP ID | O |  | NG-RAN node UE XnAP ID <reference> | Allocated at the S-NG-RAN node | YES | reject |
| Expected UE Behaviour | O |  | <reference> |  | YES | ignore |
| Requested MCG split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates that resources for MCG Split SRB are requested. | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSCGorSCGsplitBearerOption | This IE shall be present if the Bearer Option IE is set to the value "SCG bearer" or "SCG split bearer". |

#### 9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

Editor’s Note: The tabular below is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <reference> |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the S-NG-RAN node | YES | reject |
| PDU sessions Admitted To Be Added List |  | 1 |  |  | YES | ignore |
| >PDU sessions Admitted To Be Added Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>SCG Bearer |  |  |  |  |  |  |
| >>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>NG DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – | – |
| >>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer used for forwarding of DL PDUs | – | – |
| >>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer used for forwarding of UL PDUs | – | – |
| >>>Split Bearer |  |  |  |  |  |  |
| >>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>S-NG-RAN node GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | Endpoint of the Xn transport bearer at the S-NG-RAN node. | – | – |
| >>>SCG split Bearer |  |  |  |  |  |  |
| >>>>SCG split Bearer S-NG-RAN node Addition Item |  | 1 |  |  | YES | reject |
| >>>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>>NG DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – | – |
| >>>>>S-NG-RAN node GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | S-NG-RAN node endpoint of the Xn transport bearer. For delivery of UL PDUs. | – | – |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer used for forwarding of DL PDUs. | – | – |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer used for forwarding of UL PDUs. | – | – |
| >>>>UL configuration | M |  | <reference> | Information about UL usage in the S-NG-RAN node | – | – |
| >>QoS Flows Admitted List |  | 0..1 |  |  | - |  |
| >>>>QoS Flows List | O |  | 9.2.10 |  |  |  |
| >>QoS Flows not Admitted List |  | 0..1 |  |  |  |  |
| >>>>QoS Flows List | O |  | 9.2.10 |  |  |  |
| PDU sessions Not Admitted List | O |  | PDU session List <reference> | A value for PDU session ID shall only be present once in PDU sessions Admitted List IE and in PDU sessions Not Admitted List IE. | YES | ignore |
| S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the SCG-Config message as defined in TS 36.331 [9]. Editor’s Note: To be checked with RAN2 if the same RRC message is applicable for SCG split bearer. | YES | reject |
| Criticality Diagnostics | O |  | <reference> |  | YES | ignore |
| SIPTO L-GW Transport Layer Address | O |  | BIT STRING (1..160, ...) | Indicating SIPTO L-GW Transport Layer Address. | YES | ignore |
| Admitted MCG split SRBs | O |  | ENUMERATED (srb1, srb2, srb1&2, ...) | Indicates admitted SRBs | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

Omitted text not changed

#### 9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation to modify S-NG-RAN node resources for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

Editor’s Note: The tabular below is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <reference> |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | <reference> |  | YES | ignore |
| SCG Change Indication | O |  | <reference> |  | YES | ignore |
| Serving PLMN | O |  | PLMN Identity <reference> | The serving PLMN of the SCG in the S-NG-RAN node. | YES | ignore |
| Handover Restriction List | O |  | <reference> |  | YES | ignore |
| SCG Configuration Query | O |  | 9.2.13 |  | YES | ignore |
| UE Context Information |  | 0..1 |  |  | YES | reject |
| >UE Security Capabilities | O |  | <reference> |  | – | – |
| >SgNB Security Key | O |  | <reference> | Editor’s Note: terminology “S-KgNB” to be fixed with SA3 and RAN2 | – | – |
| >S-NG-RAN node UE Aggregate Maximum Bit Rate | O |  | UE Aggregate Maximum Bit Rate <reference> |  | – | – |
| > PDU sessions To Be Added List |  | 0..1 |  |  | – | – |
| >> PDU sessions To Be Added Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>> PDU sessions ID | M |  | <reference> |  | – | – |
| >>>>> PDU sessions Level QoS Parameters | M |  | <reference> | Includes necessary QoS parameters | – | – |
| >>>>>DL Forwarding | O |  | <reference> |  | – | – |
| >>>>>NG UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | UPF endpoint of the NG transport bearer. For delivery of UL PDUs. | – | – |
| >>>>>Correlation ID | O |  | Correlation ID <reference> |  | – | – |
| >>>>>SIPTO Correlation ID | O |  | Correlation ID <reference> |  | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>> PDU sessions ID | M |  | <reference> |  | – | – |
| >>>>> PDU sessions Level QoS Parameters | M |  | <reference> | Includes necessary QoS parameters | – | – |
| >>>>>M-NG-RAN node GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | M-NG-RAN node endpoint of the Xn transport bearer. For delivery of UL PDUs. | – | – |
| >>>>UL configuration | M |  | <reference> | Information about UL usage in the S-NG-RAN node | – | – |
| > PDU sessions To Be Modified List |  | 0..1 |  |  | – | – |
| >> PDU sessions To Be Modified Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>> PDU sessions ID | M |  | <reference> |  | – | – |
| >>>>> PDU sessions Level QoS Parameters | O |  | <reference> | Includes QoS parameters to be modified | – | – |
| >>>>>NG UL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | UPF endpoint of the NG transport bearer. For delivery of UL PDUs. | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>> PDU sessions ID | M |  | <reference> |  | – | – |
| >>>>> PDU sessions Level QoS Parameters | O |  | <reference> | Includes QoS parameters to be modified | – | – |
| >>>>>M-NG-RAN node GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | M-NG-RAN node endpoint of the Xn transport bearer. For delivery of UL PDUs. | – | – |
| >>>>UL configuration | O |  | <reference> | Information about UL usage in the S-NG-RAN node | – | – |
| > PDU sessions To Be Released List |  | 0..1 |  |  | – | – |
| >> PDU sessions To Be Released Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>> PDU sessions ID | M |  | <reference> |  | – | – |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer used for forwarding of DL PDUs | – | – |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer. used for forwarding of UL PDUs | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>> PDU sessions ID | M |  | <reference> |  | – | – |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer used for forwarding of DL PDUs | – | – |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the SCG-ConfigInfo message as defined in TS 38.331 [x] | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of E-RABs. Value is 256 |

#### 9.1.2.6	S-NODE MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node’s request to modify the S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

Editor’s Note: The tabular below is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <reference> |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the S-NG-RAN node | YES | ignore |
| PDU sessions Admitted List |  | 0..1 |  |  | YES | ignore |
| >PDU sessions Admitted To Be Added List |  | 1 |  |  | – | – |
| >>PDU sessions Admitted To Be Added Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>>NG DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – | – |
| >>>>>DL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer used for forwarding of DL PDUs | – | – |
| >>>>>UL Forwarding GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Identifies the Xn transport bearer used for forwarding of UL PDUs | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>>S-NG-RAN node GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint <reference> | Endpoint of the Xn transport bearer at the S-NG-RAN node. | – | – |
| >>>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>>UL configuration | M |  | <reference> | Information about UL usage in the S-NG-RAN node | – | – |
| >>>QoS Flows Admitted List |  | 0..1 |  |  | - |  |
| >>>>QoS Flows List | O |  | 9.2.10 |  |  |  |
| >>>QoS Flows not Admitted List |  | 0..1 |  |  |  |  |
| >>>>QoS Flows List | O |  | 9.2.10 |  |  |  |
| >PDU sessions Admitted To Be Modified List |  | 0..1 |  |  | – | – |
| >>PDU sessions Admitted To Be Modified Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>>NG DL GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | S-NG-RAN node endpoint of the NG transport bearer. For delivery of DL PDUs. | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>>S-NG-RAN node GTP Tunnel Endpoint | O |  | GTP Tunnel Endpoint <reference> | Endpoint of the Xn transport bearer at the S-NG-RAN node. | – | – |
| >>>QoS Flows Admitted List |  | 0..1 |  |  | - |  |
| >>>>QoS Flows List | O |  | 9.2.10 |  | - |  |
| >>>QoS Flows not Admitted List |  | 0..1 |  |  |  |  |
| >>>>QoS Flows List | O |  | 9.2.10 |  | - |  |
| >PDU sessions Admitted To Be Released List |  | 0..1 |  |  | – | – |
| >>PDU sessions Admitted To Be Released Item |  | 1 .. <maxnoPDUsessions> |  |  | EACH | ignore |
| >>>CHOICE Bearer Option | M |  |  |  |  |  |
| >>>>SCG Bearer |  |  |  |  |  |  |
| >>>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>Split Bearer |  |  |  |  |  |  |
| >>>>>PDU session ID | M |  | <reference> |  | – | – |
| PDU sessions Not Admitted List | O |  | PDU session List <reference> | A value for PDU session ID shall only be present once in PDU sessions Admitted List IE and in PDU sessions Not Admitted List IE. | YES | ignore |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Includes the SCG-Config message as defined in TS xx.xxx [x] Editor’s Note: to be checked with RAN2 | YES | ignore |
| Criticality Diagnostics | O |  | <reference> |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

Omitted text not changed

#### 9.1.2.8	S-NODE MODIFICATION REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to request the modification of S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

Editor’s Note: The tabular below is FFS.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | <reference> |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID <reference> | Allocated at the S-NG-RAN node | YES | reject |
| Cause | M |  | <reference> |  | YES | ignore |
| SCG Change Indication | O |  | <reference> |  | YES | ignore |
| PDU sessions To Be Released List |  | 0..1 |  |  | YES | ignore |
| >PDU sessions To Be Released Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>PDU session ID | M |  | <reference> |  | – | – |
| >>Cause | M |  | <reference> |  | – | – |
| >PDU sessions To Be Modified Item |  | 1 .. <maxnoofPDUsessions> |  |  | EACH | ignore |
| >>CHOICE Bearer Option | M |  | <reference> |  | – | – |
| >>>SCG Split Bearer |  |  |  |  |  |  |
| >>>>PDU session ID | M |  | <reference> |  | – | – |
| >>>>UL configuration | O |  | <reference> | Information about UL usage in the M-NG-RAN node | – | – |
| S-NG-RAN node to M-NG-RAN node Container | O |  | OCTET STRING | Includes the SCG-Config message as defined in TS yy.xxx [x] Editor’s Note: to be checked with RAN2 | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |

Remaing text not changed