3GPP TSG-RAN WG3#98 	R3-174526

Reno, USA, 27 November – 01 December 2017

Agenda item:	10.7.3

Source:	Nokia, Nokia Shanghai Bell

Title:	Text Proposal for 4g to 5g Handover

Document for:	Discussion and Decision

# 1	Description

At the last RAN3#97bis, the 5g to 4g handover solution has been agreed whereby the source SMF indicates to the UPF how to map the tunnel of one PDU session of the source side onto one or several E-RAB tunnels from target side. Then the source SMF indicates to the source gNB to which TNL address the gNB should send the forwarded packets for that PDU session in the HO Command message.

The solution for the reverse direction from 4g to 5g has been elaborated in R3-174525 including Text Proposal for TS 38.300.

Proposal 1: It is proposed to agree the following corresponding text proposal for TS 38.413.

# 2	Text Proposal for TS 38.413

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE.

Editor’s Note:	Further details are FFS.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

The AMF initiates the procedure by sending the HANDOVER REQUEST message to the target NG-RAN node.

If the Masked IMEISV IE is contained in the HANDOVER REQUEST message the target NG-RAN node shall, if supported, use it to determine the characteristics of the UE for subsequent handling.

Upon receipt of the HANDOVER REQUEST message the NG-RAN node shall

-	attempt to execute the requested PDU session configuration;

-	store the UE Aggregate Maximum Bit Rate in the UE context, and use the received UE Aggregate Maximum Bit Rate for non-GBR Bearers for the concerned UE; [FFS]

-	store the received Handover Restriction List in the UE context;

-	store the received UE Security Capabilities in the UE context;

-	store the received Security Key in the UE context, take it into use and associate it with the initial value of NCC as defined in TS xx.xxx [xx]; [FFS pending SA3]

For the Initial Context Setup an initial value for the Next Hop Chaining Count is stored in the UE context. [FFS pending SA3]

Upon receiving the PDU Session Resource To Be Setup List IE contained in the HANDOVER REQUEST message, the NG-RAN node shall behave the same as defined in the PDU Session Resource Setup procedure. The NG-RAN node shall then report in the HANDOVER REQUEST ACKNOWLEDGE message the successful establishment of the result for all the requested PDU sessions. When the NG-RAN node reports the unsuccessful establishment of a PDU session, the cause value should be precise enough to enable the AMF to know the reason for the unsuccessful establishment.

The NG-RAN node shall use the information in the Handover Restriction List IE if present in the HANDOVER REQUEST message to

-	determine a target for subsequent mobility action for which the NG-RAN node provides information about the target of the mobility action towards the UE;

-	select a proper SCG during dual connectivity operation.

If the Handover Restriction List IE is not contained in the HANOVER REQUEST message, the NG-RAN node shall consider that no roaming and no access restriction apply to the UE. The NG-RAN node shall also consider that no roaming and no access restriction apply to the UE when:

-	one of the setup PDU sessions includes a particular ARP value (TS 23.501 [9]). [FFS, need to align with PDU Session Management]

If the Trace Activation IE is included in the HANDOVER REQUEST message the NG-RAN node shall, if supported, initiate the requested trace function as described in TS 32.422 [11]. In particular, the NG-RAN node shall, if supported: [FFS pending RAN2 and SA5]

-	if the Trace Activation IE does not include the MDT Configuration IE, initiate the requested trace session as described in TS 32.422 [11];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Immediate MDT and Trace”, initiate the requested trace session and MDT session as described in TS 32.422 [11];

-	if the Trace Activation IE includes the MDT Activation IE, within the MDT Configuration IE, set to “Immediate MDT Only”, or “Logged MDT only”, initiate the requested MDT session as described in TS 32.422 [11] and the NG-RAN node shall ignore the Interfaces To Trace IE and Trace Depth IE;

-	if the Trace Activation IE includes the MDT Location Information IE, within the MDT Configuration IE, store this information and take it into account in the requested MDT session;

-	if the Trace Activation IE includes the Signalling based MDT PLMN List IE, within the MDT Configuration IE, the NG-RAN node may use it to propagate the MDT Configuration as described in TS 37.320 [12].

If the UE Security Capabilities IE is included in the HANDOVER REQUEST message, the NG-RAN node shall handle it accordingly. [FFS pending SA3]

If the Management Based MDT Allowed IE is contained in the HANDOVER REQUEST message, the NG-RAN node shall use it, if supported, together with information in the Management Based MDT PLMN List IE, if available in the UE context, to allow subsequent selection of the UE for management based MDT defined in TS 32.422 [11]. [FFS pending RAN2 and SA5].

If the RRC Inactive Assistance Information IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC-INACTIVE state decision and configuration for the UE and RAN paging if any for a UE in RRC-INACTIVE state, as specified in TS 38.300 [8].

After all necessary resources for the admitted PDU Session Resources have been allocated, the target NG-RAN node shall generate the HANDOVER REQUEST ACKNOWLEDGE message.

In case of inter-system handover, for each QoS Flow that the target gNB has decided to admit and for which DL forwarding IE is set to “DL forwarding proposed” for the corresponding source E-RAB, the target gNB may include the Forwarding Indicator set to “accepted” to indicate that it accepts the proposed forwarding of downlink data for this QoS Flow. If the target gNB accepts the downlink forwarding for at least one QoS Flow of a given admitted PDU session it shall include the Forward DL Transport Layer Information IE in the PDU Session Setup Response Transfer IE for that PDU session within the PDU Sessions Admitted List IE of the HANDOVER REQUEST ACKNOWLEDGE message.

Editor’s Note:	Further details are FFS.

Next change

#### 9.3.1.12	PDU Session Setup Response Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Transport Layer Information | M |  | 9.3.2.2 |  | - |  |
| Forward DL Transport Layer Information | O |  | 9.3.2.2 |  | - |  |
| QoS Flows Setup List |  | 0..1 |  |  | - |  |
| >QoS Flows Setup Item IEs |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Indicator | M |  | <ref> |  | EACH | reject |
| >>Forwarding Indicator | O |  | ENUMERATED(accepted, …) |  | EACH | reject |
| QoS Flows Failed to Setup List | O |  | QoS Flow List 9.3.1.24 |  | - |  |

Next change

#### 9.3.1.31	Source to Target Transparent Container

The Source to Target Transparent Container IE is used to transparently pass radio related information from the handover source to the handover target through the core network; it is produced by the source RAN node and is transmitted to the target RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Source to Target Transparent Container | M |  | OCTET STRING | This IE includes a transparent container from the source RAN node to the target RAN node. For intra NG-RAN handover, the octets of the OCTET STRING are encoded according to the specifications of the target system. For inter-system handover, the octets of the OCTET STRING are encoded according to the specifications of the LTE system. |