3GPP TSG-RAN WG3 Meeting #98	R3-174998

Reno, USA, November 27th – December 1st 2017                      (Revision of R3-174667)

Agenda item:	10.8.1.2. General

Source:	HTC, Huawei

Title:	TP for SGNB CHANGE REQUIRED

Document for:	Discussion & Decision

- Introduction

About the SN inititated SN change procedure, the current stage 2 TS [1] captures “The source SN initiates the SN change procedure by sending SgNB Change Required message which contains target SN ID information and may include the SCG configuration (to support delta configuration) and measurement results related to the target SN.” Therefore, in the stage 3 specification, we have the following proposed changes.

Proposal: The SgNB to MeNB Container in the SgNB Change Required message should carry the SCG-Config message.

References

[1] TS 37.340 v1.2.0

# 2 Text Proposal to TS 36.423 BL

Start of Change

#### 9.1.X.15	SGNB CHANGE REQUIRED

This message is sent by the SgNB to the MeNB to request the change of SgNB for a specific UE.

Direction: SgNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB. | YES | reject |
| SgNB UE X2AP ID | M |  | gNB UE X2AP ID 9.2.XX | Allocated at the SgNB. | YES | reject |
| Target SgNB ID Information | M |  | 9.2.ZZ |  | YES | reject |
| Cause | M |  | 9.2.6 |  | YES | ignore |
| SgNB to MeNB Container | O |  | OCTET STRING | Includes the SCG-Config message as defined in TS 38.331 [xx]. | YES | reject |

| Range bound | Explanation |
| --- | --- |

End of Changes