3GPP TSG-RAN WG3#99 	R3-180877

Athens, Greece, 26 Feb – 2 March 2018

Source: 	NEC

Title:	Further discussion on resource coordination in LTE-NR

Agenda Item:	10.14.1

Document for:	Discussion and Decision

# Introduction

In RAN3#98 and RAN3#NR-AH-1801 meetings, RAN3 discussed the topic of LTE-NR coexistence in overlapping and adjacent spectrum and reached the following agreements:

RAN3#98 Agreement: Signalling for the DL sharing scenario (Scenario 1) can support both indication of time/frequency resources (e.g. FDM/TDM patterns). Detailed Stage 2/3 CRs on the signalling details are encouraged for the next meeting, e.g. whether/how to indicate LTE/NR RS configurations/patterns vs. time/frequency bitmap(s). Signalling for the UL sharing scenario (Scenario 2) can support both indication of time/frequency resources (e.g. FDM/TDM patterns). Detailed Stage 2/3 CRs on the signalling details are encouraged for the next meeting. WA: adopt PRB-level granularity for frequency resources (LTE) WA: adopt subframe-level granularity for time resources (LTE) WA: FFS granularity (NR) Continue discussion in the next meeting on the working assumption for the adjacent spectrum scenario (Scenario 3), especially on the need for indicating a DL/UL switching pattern. A bi-directional non-UE-associated procedure is introduced, i.e. both eNB and gNB can initiate the coordination. Cell-level granularity is the basis for signalling on X2/Xn. RAN3#NR-AH-1801 Agreement: Solution considered shall be future-proof WA Resources allocated to LTE/NR control channels and reference signals shall not be impacted WA Resource signaling for LTE-NR coexistence shall support LTE PRBs/symbols/subframe granularity

In this contribution, we discuss the resource signaling granularity for LTE-NR coexistence.

# Discussion

RAN1 introduced the following text on PDSCH resource mapping in TS 38.214 [3]:

### 5.1.4 PDSCH resource mapping The REs corresponding to the union of configured or dynamically indicated resources in sections 5.1.4.1 and 5.1.4.2 are rate-matched by UE’s PDSCH. #### 5.1.4.1 PDSCH resource mapping with RB symbol level granularity To decode PDSCH according to a decoded PDCCH, a UE may be configured with any of the higher layer parameters: - Resource-set-BWP configuring up to [4] rate-match-PDSCH-resource-set(s) which may contain: - within a BWP, a pair of reserved resources. in numerology. of the BWP indicated by an RB level bitmap (higher layer parameter rate-match-PDSCH-bitmap1) with 1RB granularity and a symbol level bitmap spanning one or two slots (higher layer parameters rate-match-PDSCH-bitmap2) for which the reserved RBs apply…. […] #### 5.1.4.2 PDSCH resource mapping with RE level granularity To decode PDSCH according to a decoded PDCCH, a UE may be configured with any of higher layer parameters: - LTE-carrier configuring common RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell. The configuration contains rate-match-resources-v-shift consisting of LTE-CRS-vshift(s), … […]

If the coordination is initiated by gNB, then the signaling mechanism defined for NR PDSCH resource mapping (in TS 38.214 [3]), i.e. PRB-level bitmap and/or symbol-level bitmap, can be used for resource coordination between gNB and eNB in LTE-NR coexistence scenarios. The PRB-level bitmap, with a PRB granularity, can be represented by up to 275 bits, i.e. one bit corresponding to one PRB in frequency domain for a given numerology. Note that the symbol-level bitmap can also be used to indicate slot/subframe granularity by blanking all symbols in the corresponding slot/subframe. The signaling of these bitmaps can be used to avoid collisions with LTE signals or channels, e.g., PDCCH, PRACH, PUCCH.

On the other hand, if the coordination signaling is initiated by eNB, then this signaling should use LTE PRB-level and LTE-subframe/symbol level granularity. This way, at least the LTE PDCCH is protected from NR interference. For example, the LTE eNB initiates coordination signaling using LTE-PRB-level (frequency domain) and LTE-symbol-level granularity (e.g. 3 OFDM symbols at the beginning of the subframe). Additionally, the inference from NR to LTE always-on signals (e.g. CRS, and periodic CSI-RS), can be avoided by using NR rate matching. This way, NR REs corresponding to LTE CRS can be blanked by using already defined RAN1 signaling (e.g. using the rate-match-resources-v-shift).

Proposal 1: RAN3 to agree the WA “Resources allocated to LTE/NR control channels and reference signals shall not be impacted”.

Proposal 2: RAN3 to agree the WA “Resource signaling for LTE-NR coexistence shall support LTE PRBs/symbols/subframe granularity”.

# Conclusion

This contribution, we discussed resource coordination and signaling granularity for LTE-NR coexistence. The following are the proposals:

Proposal 1: RAN3 to agree the WA “Resources allocated to LTE/NR control channels and reference signals shall not be impacted”.

Proposal 2: RAN3 to agree the WA “Resource signaling for LTE-NR coexistence shall support LTE PRBs/symbols/subframe granularity”.

# References

[1] RAN3#98 Chairman report

[2] RAN3#NR-AH-1801 Chairman report

[2] R3-174986, Way Forward on CB: # 58_ LTE-NR_Coex, AT&T.

[3] 3GPP TS 38.214 V1.3.0, NR; Physical layer procedures for data.