3GPP TSG-RAN WG3#99	R3-180945

Athens, Greece, 26 February – 02 March 2018

Agenda item:	10.5.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Correction of handover and user plane Security Policy in 38.423

Document for:	Discussion and Decision

# Description

The sending of security policy has been agreed at last RAN3 NR Adhoc in tdocs [3], [4].

Some corrections have been proposed in [6] to take into account a special rejection cause when the NG-RAN node cannot accept the security policy at setup or handover as described in [5]:

The NG-RAN may reject establishment of UP resources for the PDU Session when it cannot fulfill requirements in User Plane Security Enforcement information. In this case the SMF releases the PDU session.

The same applies at Xn handover. An appropriate cause value is necessary for that and is proposed in the TP here-below.

| UP integrity protection not possible | The PDU session cannot be accepted according to the required security policy. |
| --- | --- |

# Proposal

It is proposed to agree the Text Proposal for TS 38.423 presented here-below to introduce an appropriate cause value in TS 38.423 in relation to the possible PDU session handover failure case for the new security policy for UP (pCR mirror of [6] and aligned with [5]).

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO

- 3GPP TS 23.501, Architecture for Next Generation System, SA2

- R3-180592, User Plane Security Impact on NG Interface

- R3-180638, User Plane Security Impact on Xn Interface

- S2-181304, Architectural solution for User Plane security policy and user plane integrity protection

- R3-180944, Correction of handover and user plane security policy for TS 38.413

# Text Proposal for TS 38.423

#### 9.2.2.X	Cause

The purpose of the Cause IE is to indicate the reason for a particular event for the NGAP protocol.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| CHOICE Cause Group | M |  |  |  |
| >Radio Network Layer |  |  |  |  |
| >>Radio Network Layer Cause | M |  | ENUMERATED (Unspecified, UP integrity protection not possible, …) |  |
| >Transport Layer |  |  |  |  |
| >>Transport Layer Cause | M |  | ENUMERATED (Unspecified, …) |  |
| >NAS |  |  |  |  |
| >>NAS Cause | M |  | ENUMERATED ( Unspecified, …) |  |
| >Protocol |  |  |  |  |
| >>Protocol Cause | M |  | ENUMERATED (Transfer Syntax Error, Abstract Syntax Error (Reject), Abstract Syntax Error (Ignore and Notify), Message not Compatible with Receiver State, Semantic Error, Abstract Syntax Error (Falsely Constructed Message), Unspecified, …) |  |
| >Misc |  |  |  |  |
| >>Miscellaneous Cause | M |  | ENUMERATED (, …) |  |

The meaning of the different cause values is described in the following table. In general, “not supported” cause values indicate that the related capability is missing. On the other hand, “not available” cause values indicate that the related capability is present, but insufficient resources were available to perform the requested action.

| Radio Network Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent for radio network layer cause when none of the specified cause values applies. |
| UP integrity protection not possible | The PDU session cannot be accepted according to the required security policy. |

| Transport Layer cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is Transport Network Layer related. |

| NAS cause | Meaning |
| --- | --- |
| Unspecified | Sent when none of the above cause values applies but still the cause is NAS related. |

| Protocol cause | Meaning |
| --- | --- |
| Transfer Syntax Error | The received message included a transfer syntax error. |
| Abstract Syntax Error (Reject) | The received message included an abstract syntax error and the concerning criticality indicated “reject”. |
| Abstract Syntax Error (Ignore And Notify) | The received message included an abstract syntax error and the concerning criticality indicated “ignore and notify”. |
| Message Not Compatible With Receiver State | The received message was not compatible with the receiver state. |
| Semantic Error | The received message included a semantic error. |
| Abstract Syntax Error (Falsely Constructed Message) | The received message contained IEs or IE groups in wrong order or with too many occurrences. |
| Unspecified | Sent when none of the above cause values applies but still the cause is Protocol related. |

| Miscellaneous cause | Meaning |
| --- | --- |

Editor’s Note:	Further details FFS.