3GPP TSG RAN WG3 Meeting #99	Tdoc R3-181316

Athens, Greece, 22 - 26 January 2018

Agenda Item:	31.3.1

Source:	Ericsson

Title:	(TP for NSA BL CR) Corrections on Transaction ID

Document for:	pCR 38.473 (TP for NSA BL CR)

# Introduction

During the last RAN3 meeting, it was agreed to introduce the Transaction ID to all the Class-1 and Class-2 common procedures. Additionally, the encoding of the Transaction ID was discussed, and the following sentence was captured in the Chairman notes [1]:

- Whether to change encoding of Transaction ID: to be continued…

In this contribution, we discuss the reasons for changing the encoding of the Transaction ID.

# Discussion

The Transaction ID is used to associate all the messages belonging to the same procedure. Messages belonging to the same procedure use the same Transaction ID, which is determined by the initiating peer of a procedure. The Transaction ID is used in several 3GPP protocols, such as LPPa [2] and RNSAP [3].

In all previous RAN3 protocols, the Transaction ID is included in the hearer of the messages. For example, in RNSAP the Transaction ID is included in the header of the messages, together with the Procedure ID and the Criticality, using the following encoding:

InitiatingMessage ::= SEQUENCE {

procedureID	RNSAP-ELEMENTARY-PROCEDURE.&procedureID		({RNSAP-ELEMENTARY-PROCEDURES}),

criticality	RNSAP-ELEMENTARY-PROCEDURE.&criticality		({RNSAP-ELEMENTARY-PROCEDURES}{@procedureID}),

transactionID	TransactionID,

value		RNSAP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({RNSAP-ELEMENTARY-PROCEDURES}{@procedureID})

}

SuccessfulOutcome ::= SEQUENCE {

procedureID	RNSAP-ELEMENTARY-PROCEDURE.&procedureID		({RNSAP-ELEMENTARY-PROCEDURES}),

criticality	RNSAP-ELEMENTARY-PROCEDURE.&criticality		({RNSAP-ELEMENTARY-PROCEDURES}{@procedureID}),

transactionID	TransactionID,

value		RNSAP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({RNSAP-ELEMENTARY-PROCEDURES}{@procedureID})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureID	RNSAP-ELEMENTARY-PROCEDURE.&procedureID		({RNSAP-ELEMENTARY-PROCEDURES}),

criticality	RNSAP-ELEMENTARY-PROCEDURE.&criticality		({RNSAP-ELEMENTARY-PROCEDURES}{@procedureID}),

transactionID	TransactionID,

value		RNSAP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({RNSAP-ELEMENTARY-PROCEDURES}{@procedureID})

}

The technical reasons for including the Transaction ID in the header of the messages is that the receiving node can read it and match it to the Procedure ID without the need for decoding the entire message. Therefore, the receiving node can uniquely identify the message before decoding it. This makes the overall processing at the receiving node simpler and more efficient.

In the current version of TS 38.473 [4] the Transaction ID is encoded as a normal IE in the F1AP non-UE-associated messages. This is not in-line with the previous RAN3 specifications and makes the message decoding at the receiving node more complex, slow, and inefficient. In fact, the receiving node needs to decode the header to find the Procedure ID, and then decode the rest of the message to find the Transaction ID, while keeping the procedure code ‘on hold’. Only afterwards the node can uniquely identify the message.

To summarize, ASN.1 post-processing is a heavy process and RAN3 should make effort to avoid increasing protocol overhead and latency. Including the Transaction ID in the payload of the message increases the decoding effort unnecessarily. Including the Transaction ID in the message header simplifies processing.

Observation 1:	Including the Transaction ID in the header of the messages makes the decoding at the receiving node faster and more efficient. This approach has been adopted in all previous RAN3 specifications.

Even though it may seem less relevant to have a Transaction ID for dedicated message, it may still be useful to be able to pinpoint the unique transaction without decoding a message. Moreover, we should consider that the F1 may evolve in the future with the introduction of new functions, such as UE-related measurements that are periodically exchanged between gNB-DU and gNB-CU. These functions may benefit from removing the restriction on running parallel procedures for the same UE and introducing instead a Transaction ID. Therefore, adding the Transaction ID in the UE-associated messages may have some advantages in terms of future-proofness. An implementation that does not want to make use of this information can simply ignore it. The information would furthermore only have to be defined in one place in the specification and only once.

Observation 2:	Even though it may seem less relevant to have a Transaction ID for UE-dedicated messages, it may be beneficial for future-proofness and error handling.

Based on the above discussions and observations:

Proposal 1:	Encode the Transaction ID in the hearer of the F1AP messages.

Proposal 2:	RAN3 is kindly asked to agree with the TP for TS 38.473 in Annex (BL CR for EN-DC).

# Conclusion

In this contribution, we provided clarification on the use of the Transaction ID on the F1 interface.

Proposal 1:	Encode the Transaction ID in the hearer of the F1AP messages.

Proposal 2:	RAN3 is kindly asked to agree with the TP for TS 38.473 in Annex (BL CR for EN-DC).

# References

Chairman notes, RAN3 NR_AH_18001, Sophia Antipolis, January 2018.

TS 38.455, E-UTRA; LTE Positioning Protocol A (LPPa)

TS 25.423, Radio Network Subsystem Application Part (RNSAP)

TS 38.473, NG-RAN; F1 Application Protocol (F1AP)

# Annex

Start of Text Proposal 1 for TS 38.473

### 9.2.2	UE Context Management messages

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NCGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | YES | reject |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9]. | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 0 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NCGI 9.3.1.12 | SCell Identifier in gNB | - | - |
| >>SCell Index | M |  | INTEGER (0..63) |  |  |  |
| SRB to Be Setup List |  | 0..1 |  |  |  |  |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs>] |  |  |  |  |
| >SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Setup List |  | 1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>E-UTRAN QoS | O |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >> Tunnels to be setup List |  | 1 |  |  |  |  |
| >>>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofULTunnels> |  |  |  |  |
| >>>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - | - |
| >> RLC Mode | M |  | 9.3.1.27 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 64. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULTunnels | Maximum no. of tunnels allowed towards one DRB, the maximum value is 2. |

#### 9.2.2.2	UE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DU To CU RRC Information | M |  | 9.3.1.26 |  | YES | reject |
| C-RNTI | O |  | 9.3.1.28 | C-RNTI allocated at the gNB-DU | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9]. | YES | ignore |
| SRB Setup List |  | 0..1 |  | The List of SRBs which are successfully established. | YES | ignore |
| >SRB Setup Item Iist |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| DRB Setup List |  | 1 |  | The List of DRBs which are successfully established. | YES | ignore |
| >DRB Setup Item Iist |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >> Tunnels to be setup List |  | 1 |  |  |  |  |
| >>>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofDLTunnels> |  |  |  |  |
| >>>>DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. |  |  |
| SRB Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >SRB Failed to Setup Item |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.7 |  | - | - |
| >>Cause | O |  | 9.3.1.2 |  | YES | ignore |
| DRB Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >DRB Failed to Setup Item |  | 1 .. <maxnoofDRBs> |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - | - |
| >>Cause | O |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 64. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLTunnels | Maximum no. of tunnels allowed towards one DRB, the maximum value is 2. |

#### 9.2.2.3	UE CONTEXT SETUP FAILURE

This message is sent by the gNB-DU to indicate that the setup of the UE context was unsuccessful.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.2.4	UE CONTEXT RELEASE REQUEST

This message is sent by the gNB-DU to request the gNB-CU to release the UE-associated logical F1.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.2.5	UE CONTEXT RELEASE COMMAND

This message is sent by the gNB-CU to request the gNB-DU to release the UE-associated logical F1 connection.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

#### 9.2.2.6	UE CONTEXT RELEASE COMPLETE

This message is sent by the gNB-DU to confirm the release of the UE-associated logical F1 connection.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NCGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16] | YES | Ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Stop Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9]. | YES | ignore |
| RRC-Container | O |  | 9.3.1.6 |  | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | <0 .. maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NCGI 9.3.1.12 | SCell Identifier in gNB | - | - |
| >>SCell Index | O |  | INTEGER (0..15) |  |  |  |
| SCell To Be Removed List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Removed Item IEs |  | <0 .. maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NCGI 9.3.1.12 | SCell Identifier in gNB | - | - |
| SRB to Be Setup List |  | 0..1 |  |  |  |  |
| >SRB to Be Setup Item IEs |  | <1..maxnoofSRBs> |  |  |  |  |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>E-UTRAN QoS | O |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >> Tunnels to be setup List |  | 1 |  |  |  |  |
| >>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofULTunnels> |  |  |  |  |
| >>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - | - |
| >> RLC Mode | M |  | 9.3.1.27 |  | - |  |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>E-UTRAN QoS | O |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >> Tunnels to be setup List |  | 0..1 |  |  |  |  |
| >>>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofULTunnels> |  |  |  |  |
| >>>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - | - |
| SRB TO Be Released List |  | 0..1 |  |  |  |  |
| >SRB To Be Released Item IEs |  | <1..maxnoofSRBs> |  |  |  |  |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 64. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLTunnels | Maximum no. of tunnels allowed towards one DRB, the maximum value is 2. |

#### 9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9]. | YES | ignore |
| DU To CU RRC Information | O |  | 9.3.1.26 |  | YES | reject |
| DRB Setup List |  | 0..1 |  | The List of DRBs which are successfully established. | YES | ignore |
| >DRB Setup Item IEs |  | 1 .. <maxnoofDRBs |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >> Tunnels to be setup List |  | 1 |  |  |  |  |
| >>>Tunnels to Be Setup Item IEs |  | <1 .. maxnoofDLTunnels> |  |  |  |  |
| >>>>DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. |  |  |
| DRB Modified List |  | 0..1 |  | The List of DRBs which are successfully modified. | YES | ignore |
| >DRB Modified Item IEs |  | 1 .. <maxnoofDRBs |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >> Tunnels to be setup List |  | 1 |  |  |  |  |
| >>>Tunnels to Be Setup Item IEs |  | <1 .. maxnoofDLTunnels> |  |  |  |  |
| >>>>DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of DL PDUs. |  |  |
| SRB Failed to be Setup List |  | 0..1 |  | The List of SRBs which are failed to be established. | YES | ignore |
| >SRB Failed to be Setup Item IEs |  | 1 .. <maxnoofSRBs |  |  | EACH | ignore |
| >>SRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - | - |
| DRB Failed to be Setup List |  | 0..1 |  | The List of DRBs which are failed to be setup. | YES | ignore |
| >DRB Failed to be Setup Item IEs |  | 1 .. <maxnoofDRBs |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - | - |
| DRB Failed to be Modified List |  | 0..1 |  | The List of DRBs which are failed to be modified. | YES | ignore |
| >DRB Failed to be Modified Item IEs |  | 1 .. <maxnoofDRBs |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - | - |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLTunnels | Maximum no. of tunnels allowed towards one DRB, the maximum value is 2. |

#### 9.2.2.9	UE CONTEXT MODIFICATION FAILURE

This message is sent by the gNB-DU to indicate a context modification failure.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

#### 9.2.2.10	UE CONTEXT MODIFICATION REQUIRED

This message is sent by the gNB-DU to request the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the SgNB Resource Coordination Information IE as defined in subclause 9.2.117 of TS 36.423 [9]. | YES | ignore |
| DU To CU RRC Information | O |  | 9.3.1.26 |  | YES | reject |
| DRB Required to Be Modified List |  | 0..1 |  |  | EACH | reject |
| >DRB Required to Be Modified Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Tunnels to be setup List |  | 0..1 |  |  |  |  |
| >>>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofDLTunnels> |  |  |  |  |
| >>>>DL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of DL PDUs. | - | - |
| SRB Required to be Released List |  | 0..1 |  |  | EACH | reject |
| >SRB Required to be Released List Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>SRB ID | M |  | 9.3.1.7 |  | - |  |
| DRB Required to be Released List |  | 0..1 |  |  | EACH | reject |
| >DRB Required to be Released List Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLTunnels | Maximum no. of tunnels allowed towards one DRB, the maximum value is 2. |

#### 9.2.2.11	UE CONTEXT MODIFICATION CONFIRM

This message is sent by the gNB-CU to inform the gNB-DU the successful modification.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.2.116 of TS 36.423 [9]. | YES | ignore |
| DRB Modified List |  | 0..1 |  | The List of DRBs which are successfully modified. | YES | ignore |
| >DRB Modified Item IEs |  | 1 .. <maxnoofDRBs |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Tunnels to be setup List |  | 1 |  |  |  |  |
| >>>Tunnels to Be Setup Item IEs |  | <1 .. maxnoofULTunnels> |  |  |  |  |
| >>>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-DU endpoint of the F1 transport bearer. For delivery of UL PDUs. |  |  |
| DRB Failed to be Modified List |  | 0..1 |  | The list of DRBs which are failed to be modified. | YES | ignore |
| >DRB Failed to be Modified Item IEs |  | 1 .. <maxnoofDRBs |  |  | EACH | ignore |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>Cause | O |  | 9.3.1.2 |  | - | - |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULTunnels | Maximum no. of tunnels allowed towards one DRB, the maximum value is 2. |

### 9.2.3	RRC Message Transfer messages

#### 9.2.3.1	Void

Reserved for future use.

#### 9.2.3.2	DL RRC MESSAGE TRANSFER

This message is sent by the gNB-CU to transfer the layer 3 message to the gNB-DU over the F1 interface.

Direction: gNB-CU gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| old gNB-DU UE F1AP ID | O |  | 9.3.1.5 | Include it if RRCConnectionReestablishment is included in RRC-Container | YES | reject |
| SRB ID | M |  | 9.3.1.7 |  | YES | reject |
| RRC-Container | M |  | 9.3.1.6 |  | YES | reject |

#### 9.2.3.3	UL RRC MESSAGE TRANSFER

This message is sent by the gNB-DU to transfer the layer 3 message to the gNB-CU over the F1 interface.

Direction: gNB-DU gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SRB ID | M |  | 9.3.1.7 |  | YES | reject |
| RRC-Container | M |  | 9.3.1.6 |  | YES | reject |

Start of Text Proposal 2 for TS 38.473

### 9.4.3	Elementary Procedure Definitions

-- **************************************************************

--

-- Elementary Procedure definitions

--

-- **************************************************************

F1AP-PDU-Descriptions  {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Descriptions (0)}

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Criticality,

ProcedureCode,

TransactionID

FROM F1AP-CommonDataTypes

Reset,

ResetAcknowledge,

F1SetupRequest,

F1SetupResponse,

F1SetupFailure,

GNBDUConfigurationUpdate,

GNBDUConfigurationUpdateAcknowledge,

GNBDUConfigurationUpdateFailure,

GNBCUConfigurationUpdate,

GNBCUConfigurationUpdateAcknowledge,

GNBCUConfigurationUpdateFailure,

UEContextSetupRequest,

UEContextSetupResponse,

UEContextSetupFailure,

UEContextReleaseCommand,

UEContextReleaseComplete,

UEContextModificationRequest,

UEContextModificationResponse,

UEContextModificationFailure,

UEContextModificationRequired,

UEContextModificationConfirm,

ErrorIndication,

UEContextReleaseRequest,

DLRRCMessageTransfer,

ULRRCMessageTransfer,

PrivateMessage

FROM F1AP-PDU-Contents

id-Reset,

id-F1Setup,

id-gNBDUConfigurationUpdate,

id-gNBCUConfigurationUpdate,

id-UEContextSetup,

id-UEContextRelease,

id-UEContextModification,

id-UEContextModificationRequired,

id-ErrorIndication,

id-UEContextReleaseRequest,

id-DLRRCMessageTransfer,

id-ULRRCMessageTransfer,

id-privateMessage

FROM F1AP-Constants;

-- **************************************************************

--

-- Interface Elementary Procedure Class

--

-- **************************************************************

F1AP-ELEMENTARY-PROCEDURE ::= CLASS {

&InitiatingMessage				,

&SuccessfulOutcome							OPTIONAL,

&UnsuccessfulOutcome						OPTIONAL,

&procedureCode				ProcedureCode 	UNIQUE,

&criticality				Criticality 	DEFAULT ignore

}

WITH SYNTAX {

INITIATING MESSAGE			&InitiatingMessage

[SUCCESSFUL OUTCOME			&SuccessfulOutcome]

[UNSUCCESSFUL OUTCOME		&UnsuccessfulOutcome]

PROCEDURE CODE				&procedureCode

[CRITICALITY				&criticality]

}

-- **************************************************************

--

-- Interface PDU Definition

--

-- **************************************************************

F1AP-PDU ::= CHOICE {

initiatingMessage	InitiatingMessage,

successfulOutcome	SuccessfulOutcome,

unsuccessfulOutcome	UnsuccessfulOutcome,

...

}

InitiatingMessage ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

transactionID	TransactionID,

value			F1AP-ELEMENTARY-PROCEDURE.&InitiatingMessage	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

SuccessfulOutcome ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

transactionID	TransactionID,

value			F1AP-ELEMENTARY-PROCEDURE.&SuccessfulOutcome	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

UnsuccessfulOutcome ::= SEQUENCE {

procedureCode	F1AP-ELEMENTARY-PROCEDURE.&procedureCode		({F1AP-ELEMENTARY-PROCEDURES}),

criticality		F1AP-ELEMENTARY-PROCEDURE.&criticality			({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode}),

transactionID	TransactionID,

value			F1AP-ELEMENTARY-PROCEDURE.&UnsuccessfulOutcome	({F1AP-ELEMENTARY-PROCEDURES}{@procedureCode})

}

Start of Text Proposal 3 for TS 38.473

### 9.4.4	PDU Definitions

-- **************************************************************

--

-- PDU definitions for F1AP.

--

-- **************************************************************

F1AP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

Cause,

Cells-Failed-to-be-Activated-List-Item,

Cells-to-be-Activated-List-Item,

Cells-to-be-Deactivated-List-Item,

CriticalityDiagnostics,

C-RNTI,

CUtoDURRCInformation,

DRBID,

DRBs-FailedToBeModifiedConf-Item,

DRBs-FailedToBeModified-Item,

DRBs-FailedToBeSetup-Item,

DRBs-FailedToBeSetupMod-Item,

DRBs-ModifiedConf-Item,

DRBs-Modified-Item,

DRBs-Required-ToBeModified-Item,

DRBs-Required-ToBeReleased-Item,

DRBs-Setup-Item,

DRBs-SetupMod-Item,

DRBs-ToBeModified-Item,

DRBs-ToBeReleased-Item,

DRBs-ToBeSetup-Item,

DRBs-ToBeSetupMod-Item,

DRXCycle,

DUtoCURRCInformation,

EUTRANQoS,

GNB-CU-F1AP-ID,

GNB-DU-F1AP-ID,

GNB-DU-ID,

GNB-DU-Served-Cells-Item,

GNB-DU-System-Information,

GNB-Name,

GTPTunnelEndpoint,

NCGI,

PCI,

ResourceCoordinationTransferContainer,

RRCContainer,

SCellIndex,

SCell-ToBeRemoved-Item,

SCell-ToBeSetup-Item,

SCell-ToBeSetupMod-Item,

Served-Cell-Information,

Served-Cells-To-Add-Item,

Served-Cells-To-Delete-Item,

Served-Cells-To-Modify-Item,

SRBID,

SRBs-FailedToBeSetup-Item,

SRBs-FailedToBeSetupMod-Item,

SRBs-Required-ToBeReleased-Item,

SRBs-Setup-Item,

SRBs-ToBeReleased-Item,

SRBs-ToBeSetup-Item,

SRBs-ToBeSetupMod-Item,

TimeToWait,

TransactionID,

TransmissionStopIndicator,

UE-associatedLogicalF1-ConnectionItem,

ULTunnels-ToBeSetup-Item

FROM F1AP-IEs

PrivateIE-Container{},

ProtocolExtensionContainer{},

ProtocolIE-Container{},

ProtocolIE-ContainerPair{},

ProtocolIE-SingleContainer{},

F1AP-PRIVATE-IES,

F1AP-PROTOCOL-EXTENSION,

F1AP-PROTOCOL-IES,

F1AP-PROTOCOL-IES-PAIR

FROM F1AP-Containers

id-Cause,

id-Cells-Failed-to-be-Activated-List,

id-Cells-Failed-to-be-Activated-List-Item,

id-Cells-to-be-Activated-List,

id-Cells-to-be-Activated-List-Item,

id-Cells-to-be-Deactivated-List,

id-Cells-to-be-Deactivated-List-Item,

id-CriticalityDiagnostics,

id-C-RNTI,

id-CUtoDURRCInformation,

id-DRBs-FailedToBeModifiedConf-Item,

id-DRBs-FailedToBeModifiedConf-List,

id-DRBs-FailedToBeModified-Item,

id-DRBs-FailedToBeModified-List,

id-DRBs-FailedToBeSetup-Item,

id-DRBs-FailedToBeSetup-List,

id-DRBs-FailedToBeSetupMod-Item,

id-DRBs-FailedToBeSetupMod-List,

id-DRBs-ModifiedConf-Item,

id-DRBs-ModifiedConf-List,

id-DRBs-Modified-Item,

id-DRBs-Modified-List,

id-DRBs-Required-ToBeModified-Item,

id-DRBs-Required-ToBeModified-List,

id-DRBs-Required-ToBeReleased-Item,

id-DRBs-Required-ToBeReleased-List,

id-DRBs-Setup-Item,

id-DRBs-Setup-List,

id-DRBs-SetupMod-Item,

id-DRBs-SetupMod-List,

id-DRBs-ToBeModified-Item,

id-DRBs-ToBeModified-List,

id-DRBs-ToBeReleased-Item,

id-DRBs-ToBeReleased-List,

id-DRBs-ToBeSetup-Item,

id-DRBs-ToBeSetup-List,

id-DRBs-ToBeSetupMod-Item,

id-DRBs-ToBeSetupMod-List,

id-DRXCycle,

id-DUtoCURRCInformation,

id-gNB-CU-F1AP-ID,

id-gNB-DU-F1AP-ID,

id-gNB-DU-ID,

id-GNB-DU-Served-Cells-Item,

id-gNB-DU-Served-Cells-List,

id-gNB-Name,

id-oldgNB-DU-F1AP-ID,

id-ResetType,

id-ResourceCoordinationTransferContainer,

id-RRCContainer,

id-SCell-ToBeRemoved-Item,

id-SCell-ToBeRemoved-List,

id-SCell-ToBeSetup-Item,

id-SCell-ToBeSetup-List,

id-SCell-ToBeSetupMod-Item,

id-SCell-ToBeSetupMod-List,

id-Served-Cells-To-Add-Item,

id-Served-Cells-To-Add-List,

id-Served-Cells-To-Delete-Item,

id-Served-Cells-To-Delete-List,

id-Served-Cells-To-Modify-Item,

id-Served-Cells-To-Modify-List,

id-SpCell-ID,

id-SRBID,

id-SRBs-FailedToBeSetup-Item,

id-SRBs-FailedToBeSetup-List,

id-SRBs-FailedToBeSetupMod-Item,

id-SRBs-FailedToBeSetupMod-List,

id-SRBs-Required-ToBeReleased-Item,

id-SRBs-Required-ToBeReleased-List,

id-SRBs-Setup-Item,

id-SRBs-Setup-List,

id-SRBs-ToBeReleased-Item,

id-SRBs-ToBeReleased-List,

id-SRBs-ToBeSetup-Item,

id-SRBs-ToBeSetup-List,

id-SRBs-ToBeSetupMod-Item,

id-SRBs-ToBeSetupMod-List,

id-TimeToWait,

id-TransmissionStopIndicator,

id-UE-associatedLogicalF1-ConnectionItem,

id-UE-associatedLogicalF1-ConnectionListResAck,

maxCellingNBDU,

maxnoofDRBs,

maxnoofErrors,

maxnoofIndividualF1ConnectionsToReset,

maxnoofSCells,

maxnoofSRBs

FROM F1AP-Constants;

-- **************************************************************

--

#### -- RESET ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Reset

--

-- **************************************************************

Reset ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetIEs} },

...

}

ResetIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-ResetType					CRITICALITY reject	TYPE ResetType					PRESENCE mandatory	},

...

}

ResetType ::= CHOICE {

f1-Interface					ResetAll,

partOfF1-Interface				UE-associatedLogicalF1-ConnectionListRes,

...

}

ResetAll ::= ENUMERATED {

reset-all,

...

}

UE-associatedLogicalF1-ConnectionListRes ::= SEQUENCE (SIZE(1.. maxnoofIndividualF1ConnectionsToReset)) OF ProtocolIE-SingleContainer { { UE-associatedLogicalF1-ConnectionItemRes } }

UE-associatedLogicalF1-ConnectionItemRes F1AP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalF1-ConnectionItem	CRITICALITY reject	TYPE UE-associatedLogicalF1-ConnectionItem	PRESENCE mandatory},

...

}

-- **************************************************************

--

-- Reset Acknowledge

--

-- **************************************************************

ResetAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {ResetAcknowledgeIEs} },

...

}

ResetAcknowledgeIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalF1-ConnectionListResAck		CRITICALITY ignore	TYPE UE-associatedLogicalF1-ConnectionListResAck			PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics			PRESENCE optional	},

...

}

UE-associatedLogicalF1-ConnectionListResAck ::= SEQUENCE (SIZE(1.. maxnoofIndividualF1ConnectionsToReset)) OF ProtocolIE-SingleContainer { { UE-associatedLogicalF1-ConnectionItemResAck } }

UE-associatedLogicalF1-ConnectionItemResAck 	F1AP-PROTOCOL-IES ::= {

{ ID id-UE-associatedLogicalF1-ConnectionItem	 CRITICALITY ignore 	TYPE UE-associatedLogicalF1-ConnectionItem  	PRESENCE mandatory },

...

}

-- **************************************************************

--

#### -- ERROR INDICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- Error Indication

--

-- **************************************************************

ErrorIndication ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ErrorIndicationIEs}},

...

}

ErrorIndicationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-F1AP-ID				CRITICALITY ignore	TYPE GNB-CU-F1AP-ID				PRESENCE optional	}|

{ ID id-gNB-DU-F1AP-ID				CRITICALITY ignore	TYPE GNB-DU-F1AP-ID				PRESENCE optional	}|

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- F1 SETUP ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- F1 Setup Request

--

-- **************************************************************

F1SetupRequest ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupRequestIEs} },

...

}

F1SetupRequestIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-DU-ID					CRITICALITY reject	TYPE GNB-DU-ID					PRESENCE mandatory	}|

{ ID id-gNB-Name						CRITICALITY ignore	TYPE	 GNB-Name					PRESENCE optional	}|

{ ID id-gNB-DU-Served-Cells-List		CRITICALITY reject	TYPE	 GNB-DU-Served-Cells-List	PRESENCE mandatory	},

...

}

GNB-DU-Served-Cells-List 	::= SEQUENCE (SIZE(1.. maxCellingNBDU)) OF ProtocolIE-SingleContainer { { GNB-DU-Served-Cells-ItemIEs } }

GNB-DU-Served-Cells-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-GNB-DU-Served-Cells-Item		CRITICALITY reject	TYPE		GNB-DU-Served-Cells-Item	PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- F1 Setup Response

--

-- **************************************************************

F1SetupResponse ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupResponseIEs} },

...

}

F1SetupResponseIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Cells-to-be-Activated-List	CRITICALITY reject	TYPE	 Cells-to-be-Activated-List		PRESENCE optional	},

...

}

Cells-to-be-Activated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-to-be-Activated-List-ItemIEs } }

Cells-to-be-Activated-List-ItemIEs	F1AP-PROTOCOL-IES::= {

{ ID id-Cells-to-be-Activated-List-Item				CRITICALITY reject	TYPE	 Cells-to-be-Activated-List-Item						PRESENCE mandatory}	,

...

}

-- **************************************************************

--

-- F1 Setup Failure

--

-- **************************************************************

F1SetupFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {F1SetupFailureIEs} },

...

}

F1SetupFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-DU CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- GNB-DU CONFIGURATION UPDATE

--

-- **************************************************************

GNBDUConfigurationUpdate::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNBDUConfigurationUpdateIEs} },

...

}

GNBDUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Served-Cells-To-Add-List		CRITICALITY reject	TYPE	 Served-Cells-To-Add-List					PRESENCE optional	}|

{ ID id-Served-Cells-To-Modify-List		CRITICALITY reject	TYPE	 Served-Cells-To-Modify-List				PRESENCE optional	}|

{ ID id-Served-Cells-To-Delete-List		CRITICALITY reject	TYPE	 Served-Cells-To-Delete-List				PRESENCE optional	},

...

}

Served-Cells-To-Add-List		::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Served-Cells-To-Add-ItemIEs } }

Served-Cells-To-Modify-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Served-Cells-To-Modify-ItemIEs } }

Served-Cells-To-Delete-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Served-Cells-To-Delete-ItemIEs } }

Served-Cells-To-Add-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Served-Cells-To-Add-Item		CRITICALITY reject	TYPE	 Served-Cells-To-Add-Item				PRESENCE mandatory	},

...

}

Served-Cells-To-Modify-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Served-Cells-To-Modify-Item			CRITICALITY reject	TYPE		Served-Cells-To-Modify-Item							PRESENCE mandatory	}	,

...

}

Served-Cells-To-Delete-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Served-Cells-To-Delete-Item				CRITICALITY reject	TYPE		Served-Cells-To-Delete-Item					PRESENCE mandatory	},

...

}

-- **************************************************************

--

-- GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

GNBDUConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNBDUConfigurationUpdateAcknowledgeIEs} },

...

}

GNBDUConfigurationUpdateAcknowledgeIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Cells-to-be-Activated-List		CRITICALITY reject	TYPE	 Cells-to-be-Activated-List	PRESENCE optional	}|

{ ID id-CriticalityDiagnostics			CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

-- GNB-DU CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

GNBDUConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { {GNBDUConfigurationUpdateFailureIEs} },

...

}

GNBDUConfigurationUpdateFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

-- **************************************************************

--

#### -- GNB-CU CONFIGURATION UPDATE ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

-- GNB-CU CONFIGURATION UPDATE

--

-- **************************************************************

GNBCUConfigurationUpdate ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateIEs} },

...

}

GNBCUConfigurationUpdateIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Cells-to-be-Activated-List		CRITICALITY reject	TYPE	 Cells-to-be-Activated-List				PRESENCE optional	}|

{ ID id-Cells-to-be-Deactivated-List		CRITICALITY reject	TYPE	 Cells-to-be-Deactivated-List			PRESENCE optional	},

...

}

Cells-to-be-Deactivated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-to-be-Deactivated-List-ItemIEs } }

Cells-to-be-Deactivated-List-ItemIEs F1AP-PROTOCOL-IES	::= {

{ ID id-Cells-to-be-Deactivated-List-Item						CRITICALITY reject	TYPE	 Cells-to-be-Deactivated-List-Item					PRESENCE mandatory	},

...}

-- **************************************************************

--

-- GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE

--

-- **************************************************************

GNBCUConfigurationUpdateAcknowledge ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateAcknowledgeIEs} },

...

}

GNBCUConfigurationUpdateAcknowledgeIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Cells-Failed-to-be-Activated-List	CRITICALITY reject	TYPE	 Cells-Failed-to-be-Activated-List		PRESENCE optional}|

{ ID id-CriticalityDiagnostics				CRITICALITY ignore	TYPE CriticalityDiagnostics						PRESENCE optional	},

...

}

Cells-Failed-to-be-Activated-List	::= SEQUENCE (SIZE(1.. maxCellingNBDU))	OF ProtocolIE-SingleContainer { { Cells-Failed-to-be-Activated-List-ItemIEs } }

Cells-Failed-to-be-Activated-List-ItemIEs F1AP-PROTOCOL-IES		::= {

{ ID id-Cells-Failed-to-be-Activated-List-Item							CRITICALITY reject	TYPE	 Cells-Failed-to-be-Activated-List-Item		PRESENCE mandatory	}	,

...

}

-- **************************************************************

--

-- GNB-CU CONFIGURATION UPDATE FAILURE

--

-- **************************************************************

GNBCUConfigurationUpdateFailure ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       { { GNBCUConfigurationUpdateFailureIEs} },

...

}

GNBCUConfigurationUpdateFailureIEs F1AP-PROTOCOL-IES ::= {

{ ID id-Cause						CRITICALITY ignore	TYPE Cause						PRESENCE mandatory	}|

{ ID id-TimeToWait					CRITICALITY ignore	TYPE TimeToWait					PRESENCE optional	}|

{ ID id-CriticalityDiagnostics		CRITICALITY ignore	TYPE CriticalityDiagnostics		PRESENCE optional	},

...

}

Start of Text Proposal 4 for TS 38.473

### 9.4.5	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

maxnoofErrors,

maxnoofBPLMNs,

maxnoofDLTunnels,

maxnoofULTunnels

FROM F1AP-Constants

Criticality,

TransactionID,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM F1AP-CommonDataTypes

ProtocolExtensionContainer{},

F1AP-PROTOCOL-EXTENSION,

ProtocolIE-SingleContainer{},

F1AP-PROTOCOL-IES

FROM F1AP-Containers;