3GPP TSG-RAN WG3 #99	Tdoc R3-181340

Athens, Greece, 26 February – 2 March 2018

Agenda Item:	23.3

Source:	Ericsson

Title:	Bearer Release procedure

Document for:	pCR TS 38.463

# Introduction

As illustrated in TR 38.806 [1], during the study item it was agreed to define the following E1 bearer management procedures:

- DRB setup: this procedure allows the CU-CP to setup DRBs in the CU-UP, including the QoS-flow to DRB mapping configuration.

- DRB modification: this procedure allows the CU-CP to modify DRBs in the CU-UP, including the modification of the QoS-flow to DRB mapping configuration. It can also be used to suspend/resume data transmission.

- DRB release: this procedure allows the CU-CP to release DRBs in the CU-UP. It also allows the CU-UP to request the release of DRBs to the CU-CP (e.g., due to inactivity).

- DL Data Notification: this procedure allows the CU-CP to inform the CU-UP in case of new incoming DL traffic for a bearer that was previously suspended.

In this contribution, we discuss the initial content of the E1 Bearer Release procedure.

# Discussion

As defined in TR 38.806 [1], the purpose of the Bearer Release procedure (CU-CP initiated) is to allow the CU-CP to release existing bearer resources. This procedure employs UE-associated signalling. We propose that the Bearer Release procedure includes the following messages:

- Bearer Release Request (CU-CP  CU-UP);

- Bearer Release Response (CU-UP  CU-CP);

Proposal 1: 	The Bearer Release procedure (CU-CP initiated) is used to release existing bearers. It includes: Bearer Release Request and Bearer Release Response message.

Since the Bearer Release messages use UE-associated signalling, they should include the CU-CP UE E1AP ID and the CU-UP UE E1AP ID. In NG-RAN, the bearer release procedure should also allow to release PDU Session Resources and QoS-flows. It should be discussed whether the Bearer Release Request message may also include cause values.

Proposal 2: 	The Bearer Release procedure should allow the release of PDU Session, DRB and QoS-flow resources.

Proposal 3: 	The Bearer Release messages should include the CU-CP UE E1AP ID and the CU-UP UE E1AP ID.

A Bearer Release procedure initiated by the CU-UP is also needed. This is because the CU-UP may need to request the release of some bearer resources in case of sudden load and/or hardware related problems that does not allow it to serve some bearers, PDU Sessions and/or QoS flows any longer. Therefore, we believe that it would be beneficial to introduce over E1 a Bearer Release Required procedure (CU-UP initiated). Following the same approach adopted over the F1 for the gNB-DU-initiated UE Context Release procedure, the Bearer Release Required procedure can be Class-2 procedure including the following message:

- Bearer Release Required (CU-UP  CU-CP);

After receiving the Bearer Release Required message, the CU-CP should initiate the Bearer Release procedure to remove the corresponding bearer resources.

Proposal 4: 	The Bearer Release Required procedure (CU-UP initiated) is used to release existing bearers. It is a Class-2 procedure including the Bearer Release Required message.

# Conclusion

In this contribution, we discussed the E1 Bearer Setup procedure.

Proposal 1: 	The Bearer Release procedure (CU-CP initiated) is used to release existing bearers. It includes: Bearer Release Request and Bearer Release Response message.

Proposal 2: 	The Bearer Release procedure should allow the release of PDU Session, DRB and QoS-flow resources.

Proposal 3: 	The Bearer Release messages should include the CU-CP UE E1AP ID and the CU-UP UE E1AP ID.

Proposal 4: 	The Bearer Release Required procedure (CU-UP initiated) is used to release existing bearers. It is a Class-2 procedure including the Bearer Release Required message.

Proposal 5: 	RAN3 is kindly asked to agree with the TP in Annex I.

# References

TR 38.806, Study of separation of NR Control Plane (CP) and User Plane (UP) for split option 2.

# Annex I: TP for 38.463

Start of Text Proposal 1 for TS 38.463

Table 1: Class 1 procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |
| --- | --- | --- | --- |
|  |  | Response message | Response message |
| Bearer Release (CU-CP initiated) | BEARER RELEASE REQUEST | BEARER RELEASE RESPONSE |  |

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Bearer Release Required (CU-UP initiated) | BEARER RELEASE REQUIRED |

Start of Text Proposal 2 for TS 38.463

### 8.3.X	Bearer Release (CU-CP initiated)

#### 8.3.X.1	General

The purpose of the Bearer Release procedure is to enable the CU-CP to request the CU-UP to release bearer resources. The procedure uses UE-associated signalling.

#### 8.3.X.2	Successful Operation

Figure 8.3.x.2-1: Bearer Release procedure (CU-CP initiated). Successful Operation.

The CU-CP initiates the procedure by sending the BEARER RELEASE REQUEST message to the CU-UP.

Upon reception of the BEARER RELEASE REQUEST message, the CU-UP shall release the related bearer resources and reply with the BEARER RELEASE COMPLETE message

#### 8.2.X.3	Abnormal Conditions

Not applicable.

### 8.3.Y	Bearer Release Required (CU-UP initiated)

#### 8.3.Y.1	General

The purpose of the Bearer Release Required procedure is to enable the CU-UP to request the CU-CP to release related bearer resources. The procedure uses UE-associated signalling.

#### 8.3.Y.2	Successful Operation

Figure 8.3.Y.2-1: Bearer Release Required procedure (CU-UP initiated). Successful operation.

The CU-UP initiates the procedure by generating a BEARER RELEASE REQUIRED message towards the CU-CP.  The BEARER RELEASE REQUIRED message shall indicate the appropriate cause value.

Interactions with Bearer Release procedure:

The Bearer Release procedure may be initiated upon reception of a BEARER RELEASE REQUIRED message.

#### 8.3.Y.3	Abnormal Conditions

Not applicable.

Start of Text Proposal 3 for TS 38.463

#### 9.2.1.X	BEARER RELEASE REQUEST

This message is sent by the CU-CP to request the CU-UP to release bearer resources.

Direction: CU-CP  CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.X |  | YES | reject |
| CU-CP UE E1AP ID | M |  | 9.3.1.Y |  | YES | reject |
| CU-UP UE E1AP ID | M |  | 9.3.1.Y |  | YES | reject |

Editor’s note: Further details are FFS.

#### 9.2.1.X	BEARER RELEASE RESPONSE

This message is sent by the CU-UP to confirm the release of the bearer resources.

Direction: CU-UP  CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.X |  | YES | reject |
| CU-CP UE E1AP ID | M |  | 9.3.1.Y |  | YES | reject |
| CU-UP UE E1AP ID | M |  | 9.3.1.Z |  | YES | reject |
| Criticality Diagnostics | O |  | 9.3.1.A |  | YES | ignore |

Editor’s note: Further details are FFS.

#### 9.2.1.X	BEARER RELEASE REQUIRED

This message is sent by the CU-UP to request the request the release of bearer resources.

Direction: CU-UP  CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.X |  | YES | reject |
| CU-CP UE E1AP ID | M |  | 9.3.1.Y |  | YES | reject |
| CU-UP UE E1AP ID | M |  | 9.3.1.Y |  | YES | reject |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

Editor’s note: Further details are FFS.

End of Text Proposal for TS 38.463