3GPP TSG-RAN WG3#99bis	R3-181669

Sanya, China, 16 – 20 April 2018

Agenda item:	10.1.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Signalling 5QI in QoS Profile of TS 38.423

Document for:	Discussion and Decision

# Description and proposal

The QoS profile has been cleaned up with a new structure at last RAN3 through tdoc [3] as follows:

#### 9.3.1.23	QoS Flow Level QoS Parameters

Editor’s Note:	Further details FFS.

This IE defines the QoS parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| 5QI | O (FFS) |  | INTEGER (0..255) | Details in TS 23.501 [9]. | YES | reject |
| CHOICE QoS Characteristics | M |  |  |  | YES | reject |
| >Non-dynamic 5QI |  |  |  |  | - |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.40 |  | - |  |
| >Dynamic 5QI |  |  |  |  | - |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.29 |  | - |  |
| Allocation and Retention Priority | M |  | 9.3.1.30 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.10 | This IE shall be present for GBR QoS Flows only. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED(subject to, …) | Details in TS 23.501 [9]. This IE applies to non-GBR flows only and shall be ignored otherwise. | - |  |

The presence of 5QI within the QoS Profile IE has been left FFS at the last RAN3 meeting.

Tdoc [4] has proposed for TS 38.413 to solve the FFS by encoding the 5QI mandatory for standardized case and optional for the non-standardized case.

We therefore propose to agree the Text Proposal below to mirror into TS 38.423 the change of the 5QI presence of tdoc [4].

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO

- 3GPP TS 23.501, Architecture for Next Generation System, SA2

- R3-180628, Correction of QoS Profile for TS 38.413, Nokia, Nokia Shanghai Bell

- R3-181668, Signalling 5QI in the QoS Profile of TS 38.413, Nokia, Nokia Shanghai Bell

# Text Proposal for TS 38.423

#### 9.2.2.1	QoS Flow Level QoS Parameters

This IE defines the QoS Parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | – |  |
| >Non Dynamic 5QI |  |  |  |  | – |  |
| >>Non dynamic 5QI Desciptor | M |  | 9.2.2.8 |  | – |  |
| >Dynamic 5QI |  |  |  |  | – |  |
| >>Dynamic 5QI Desciptor | M |  | 9.2.2.7 |  | – |  |
| Allocation and Retention Priority | M |  | 9.2.2.6 | Note: presence needs to be checked with latest SA2 status. | – |  |
| GBR QoS Flow Information | O |  | 9.2.2.2 | This IE shall be present for GBR QoS Flows only. | – |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, ...) | For details see TS 23.501 [7]. This IE applies to non-GBR bearers only and shall be ignored otherwise. | – |  |

#### 9.2.2.7	Dynamic 5QI Descriptor

This IE defines the QoS characteristics for a non-standardized or not pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5QI | O |  | INTEGER (0..255) | For details refer to TS 23.501 [7]. |
| Priority level | M |  | <reference> | For details refer to TS 23.501 [7]. |
| Packet Delay Budget | M |  | <reference> | For details refer to TS 23.501 [7]. |
| Packet Error Rate | M |  | <reference> | For details refer to TS 23.501 [7]. |
| Delay Critical | C-ifGBRflow |  | ENUMERATED (Delay critical, Non-delay critical) | For details refer to TS 23.501 [7]. |
| Averaging window | C-ifGBRflow |  | <reference> | For details refer to TS 23.501 [7]. |
| Maximum Data Burst Volume | O |  | <reference> | For details refer to TS 23.501 [7]. |

| Condition | Explanation |
| --- | --- |
| ifGBRflow | This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE. |

#### 9.2.2.8	Non dynamic 5QI Descriptor

This IE defines QoS characteristics for a standardized or pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5QI | M |  | INTEGER (0..255) | For details refer to TS 23.501 [7]. |
| Priority level | O |  | <reference> | For details refer to TS 23.501 [7]. When included overrides standardized or pre-configured value. |
| Averaging window | O |  | <reference> | This IE applies to GBR QoS Flows only. For details refer to TS 23.501 [7]. When included overrides standardized or pre-configured value. |
| Maximum Data Burst Volume | O |  | <reference> | For details refer to TS 23.501 [7]. When included overrides standardized or pre-configured value. |