3GPP TSG-RAN WG3 Meeting #99bis	R3-182146

# Sanya, P.R.China, 16th- 20th April  2018

Agenda Item:	10.1.1

Source:	Ericsson

Title:	Resolving the remaining FFS on QoS

Document for:	pCR

# Text Proposal for NGAP TS 38.423 v0.7.0

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.1	QoS Flow Level QoS Parameters

This IE defines the QoS Parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | – |  |
| >Non Dynamic 5QI |  |  |  |  | – |  |
| >>Non dynamic 5QI Desciptor | M |  | 9.2.2.8 |  |  |  |
| >Dynamic 5QI |  |  |  |  | – |  |
| >>Dynamic 5QI Desciptor | M |  | 9.2.2.7 |  | – |  |
| Allocation and Retention Priority | M |  | 9.2.2.6 | Note: presence needs to be checked with latest SA2 status. | – |  |
| GBR QoS Flow Information | O |  | 9.2.2.2 | This IE shall be present for GBR QoS Flows only. | – |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, ...) | For details see TS 23.501 [7]. This IE applies to non-GBR bearers only and shall be ignored otherwise. | – |  |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.2.2.7	Dynamic 5QI Descriptor

This IE defines the QoS characteristics for a non-standardized or not pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Priority level | M |  | <reference> | For details refer to TS 23.501 [7]. |
| Packet Delay Budget | M |  | <reference> | For details refer to TS 23.501 [7]. |
| Packet Error Rate | M |  | <reference> | For details refer to TS 23.501 [7]. |
| Delay Critical | C-ifGBRflow |  | ENUMERATED (Delay critical, Non-delay critical) | For details refer to TS 23.501 [7]. |
| Averaging window | C-ifGBRflow |  | <reference> | For details refer to TS 23.501 [7]. |
| Maximum Data Burst Volume | O |  | <reference> | For details refer to TS 23.501 [7]. |

| Condition | Explanation |
| --- | --- |
| ifGBRflow | This IE shall be present if the GBR QoS Flow Information IE is present in the QoS Flow Level QoS Parameters IE. |

#### 9.2.2.8	Non dynamic 5QI Descriptor

This IE defines QoS characteristics for a standardized or pre-configured 5QI for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| 5QI | M |  | INTEGER (0..255) | See details in TS 24.501 [7] |
| Priority level | O |  | <reference> | For details refer to TS 23.501 [7]. When included overrides standardized or pre-configured value. |
| Averaging window | O |  | <reference> | This IE applies to GBR QoS Flows only. For details refer to TS 23.501 [7]. When included overrides standardized or pre-configured value. |
| Maximum Data Burst Volume | O |  | <reference> | For details refer to TS 23.501 [7]. When included overrides standardized or pre-configured value. |

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>