3GPP TSG-RAN WG3 Meeting #99bis	R3-182148

# Sanya, P.R. China, 16th- 20th April  2018

Agenda Item:	10.1.1

Source:	Ericsson

Title:	Notification Control over NGAP

Document for:	pCR

# 1	Discussion

In TS 23.501, it is specified that the Notification control indicates whether notifications are requested from the RAN when the GFBR can no longer (or again) be fulfilled for a QoS Flow during the lifetime of the QoS Flow.

It is not clear from the Notification Cause if we only define “fulfilled, not fulfilled”, specially when there are more requirements to trigger the notification control to be added.

Proposal 1: RAN3 to agree on the notification cause as “GFBR fulfilled, GFBR not fulfilled”.

It is proposed to in addition to GFBR, that the PDB and PER could also trigger the notification control.

Proposal 2: RAN3 to agree on adding the notification causes such as “PDB fulfilled, PDB not fulfilled, PER fulfilled, PER not fulfilled”.

# 2	Proposals

Proposal 1: RAN3 to agree on the notification cause as “GFBR fulfilled, GFBR not fulfilled”.

Proposal 2: RAN3 to agree on adding the notification causes such as “PDB fulfilled, PDB not fulfilled, PER fulfilled, PER not fulfilled”.

# Text Proposal for NGAP TS 38.413 v0.7.0

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

AMF	Access and Mobility Management Function

EP	Elementary Procedure

GBR	Guaranteed Bit Rate

GFBR	Guaranteed Flow Bit Rate

GTP	GPRS Tunnelling Protocol

IE	Information Element

NAS	Non-Access Stratum

NCGI	NR Cell Global Identifier

NGAP	NG Application Protocol

NRPPa	NR Positioning Protocol Annex

NSCI	New Security Context Indicator

PDB	Packet Delay Budget

PER	Packet Error Rate

PLMN	Public Land Mobile Network

QoS	Quality of Service

SCTP	Stream Control Transmission Protocol

SMF	Session Management Function

S-NG-RAN node	Secondary NG-RAN node

S-NSSAI	Single Network Slice Selection Assistance Information

TEID	Tunnel Endpoint Identifier

UE	User Equipment

------------------------------------------

Skip to next change

------------------------------------------

### 8.2.4	PDU Session Resource Notify

#### 8.2.4.1	General

The purpose of the PDU Session Resource Notify procedure is to notify that the already established QoS flow(s) or PDU session(s) for a given UE are released or when the GFBR, PDB or PER can no longer (or again) be  fulfilled by the NG-RAN node for which a notification is issued. The procedure uses UE-associated signalling.

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: PDU session resource notify

The NG-RAN node initiates the procedure by sending a PDU SESSION RESOURCE NOTIFY message.

The PDU SESSION RESOURCE NOTIFY message shall contain the information of PDU sessions or QoS flows which are released or when the GFBR, PDB or PER can no longer (or again) be fulfilled by the NG-RAN node.

-	For each PDU session of which some QoS flows are released or when the GFBR, PDB or PER can no longer (or again) be fulfilled by the NG-RAN node, the PDU Session Resource Notify Transfer IE shall be included to report for each PDU session of which some QoS flow(s) are released or not fulfilled anymore or fulfilled again [FFS]:

1. 	The list of QoS flows which are released by the NG-RAN node, if any, shall be included in the QoS Flows Release Notify List IE.

2. 	The list of GBR QoS flows which the GFBR, PDB or PER can no longer (or again) be fulfilled  by the NG-RAN node, if any, shall be included in the QoS Flows Notify List IE.

-	For each PDU session  when GFBR, PDB or PER can no longer (or again) be fulfilled for all the QoS flows  by the NG-RAN node, the PDU Session Resource Notify Transfer IE shall be included to report the cause in the Notification Cause IE.

-	For each PDU session which is released by the NG-RAN node, the PDU Session Resource Notify Transfer IE shall be included to report the release cause in the Cause IE.

Upon reception of the PDU SESSION RESOURCE NOTIFY message, the AMF shall, for each PDU session indicated in the PDU Session ID IE, transfer transparently the PDU Session Resource Notify Transfer IE to each SMF associated with the concerned PDU session. [FFS] Upon reception of PDU Session Resource Notify Transfer IE, SMF normally initiate the appropriate release or modify procedure on the core network side for the PDU session(s) or QoS flow(s) identified in the PDU SESSION RESOURCE NOTIFY message [FFS].

Editor’s Note:	Further details are FFS.

#### 8.2.4.3	Abnormal Conditions

Editor’s Note:	Further details are FFS.

------------------------------------------

Skip to next change

------------------------------------------

#### 9.3.4.7	PDU Session Resource Notify Transfer

Editor’s Note: 	Further details FFS.

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE PDU Session Resource Notify Transfer | M |  |  |  | YES | reject |
| >QoS Flows Notify |  |  |  |  | - |  |
| >>QoS Flows Notify List |  | 1 |  |  | - |  |
| >>>QoS Flows Notify Item IEs |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>>>QoS Flow Indicator | M |  | <ref> |  | EACH | reject |
| >>>>Notification Cause | M |  | 9.3.x.y |  | - |  |
| >>>>Assistance Information [FFS] | O |  | <ref> |  | - |  |
| >>QoS Flows Release List [FFS] | O |  | QoS Flow List 9.3.1.13 |  | YES | ignore |
| >PDU Session Resource Release |  |  |  |  | - |  |
| >>Cause [FFS] | M |  | 9.3.1.2 |  | - |  |
| >PDU Session Resource Notify |  |  |  |  | - |  |
| >>Notification Cause | M |  | 9.3.x.y |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is FFS. |

------------------------------------------

Skip to next change

------------------------------------------

#### 9.3.x.y	Notification Cause

This IE contains the cause for sending the notification from NG-RAN node, refer to TS 23.501 [9].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Notification Cause | M |  | ENUMERATED (GFBR fulfilled, GFBR not fulfilled, PDB fulfilled, PDB not fulfilled, PER fulfilled, PER not fulfilled, …) |  |

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>