3GPP TSG-RAN WG3 #99bis	Tdoc R3-182240

Sanya, P. R. China, 16th- 20th April  2018

Agenda Item:	23.3

Source:	Ericsson

Title:	Error Indication

Document for:	pCR TS 38.463

# Introduction

As illustrated in TR 38.806 [1], during the study item it was agreed to define the following E1 interface management procedures:

- E1 setup: this procedure allows to setup the E1 interface. It includes the exchange of the parameters needed for interface operation.

- E1 reset: this procedure allows to reset the E1 interface. It is used to initialize the peer entity after node setup and after a failure event occurred. The E1 interface reset is initiated by either the CU-CP or the CU-UP.

- E1 error indication: this procedure allows to report detected errors in one incoming message. The E1 interface reset is initiated by either the CU-CP or the CU-UP.

In this contribution, we propose the content for the Error Indication procedure (same as for F1).

Proposal 1: 	RAN3 is kindly asked to agree with the TP in Annex I.

# Annex I: TP for 38.463

Start of Text Proposal 1 for TS 38.463

Table 2: Class 2 procedures

| Elementary Procedure | Message |
| --- | --- |
| Error Indication | ERROR INDICATION |

Start of Text Proposal 2 for TS 38.463

### 8.2.Y	Error Indication

#### 8.2.Y.1	General

The Error Indication procedure is initiated by a node in order to report detected errors in one incoming message, provided they cannot be reported by an appropriate failure message.

If the error situation arises due to reception of a message utilising UE associated signalling, then the Error Indication procedure uses UE associated signalling. Otherwise the procedure uses non-UE associated signalling.

#### 8.2.Y.2	Successful Operation

Figure 8.2.Y.2-1: Error Indication procedure, CU-CP originated. Successful operation.

Figure 8.2.Y.2-2: Error Indication procedure, CU-UP originated. Successful operation.

When the conditions defined in clause 10 are fulfilled, the Error Indication procedure is initiated by an ERROR INDICATION message sent from the receiving node.

The ERROR INDICATION message shall contain at least either the Cause IE or the Criticality Diagnostics IE. In case the Error Indication procedure is triggered by utilising UE associated signalling the CU-CP UE E1AP ID IE shall be included in the ERROR INDICATION message. If the CU-CP UE E1AP ID IE is not correct, the cause shall be set to appropriate value, e.g., "Unknown or already allocated CU-CP UE E1AP ID".

#### 8.2.Y.3	Abnormal Conditions

Not applicable.

Start of Text Proposal 3 for TS 38.463

#### 9.2.1.Y	ERROR INDICATION

This message is sent by both the CU-CP and the CU-UP and is used to indicate that some error has been detected in the node.

Direction: CU-CP  CU-UP and CU-UP  CU-CP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.x |  | YES | ignore |
| CU-CP UE E1AP ID | O |  | 9.3.1.y |  | YES | ignore |
| CU-UP UE E1AP ID | O |  | 9.3.1.z |  | YES | ignore |
| Cause | O |  | 9.3.1.a |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.b |  | YES | ignore |

End of Text Proposal for TS 38.463