3GPP TSG-RAN WG3 meeting #99-bis	R3-182369

Sanya, P.R.China, 16 – 20 April 2018

Agenda item:	31.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Problem statement for the AMBR in EN-DC

Document for:	Discussion

# 1	Introduction

At the RAN3#99bis, the problem of the AMBR has been reviewed. Based on the discussion, the problem was defined.

# 2	Problem statement

The problem concerns enabling a UE to utilize the available AMBR quota fully, where the data is split into hosting points and then further into radio bearers.

- Sharing AMBR between MN- and SN-terminated bearers:

- Fluctuations are due to application activity;

- Fluctuations happen in slow time scale;

- Sharing between MCG and SCG part of a split bearer:

- Fluctuations are due to radio conditions;

- Fluctuations may happen very fast;

Furthermore, RAN3’s understanding is that the purpose of the AMBR is to protect the radio interface from abusive use from particular UEs.

Existing solution for enforcing DL AMBR:

- the DL AMBR quota is assigned statically to a hosting node (which enforces it by sending DL data only up to the limit)
=> problem 1 exists;

- the hosting node dynamically directs the traffic to the radio part that offers better quality => problem 2 does not exist;

Existing solution for enforcing UL AMBR:

- the UL AMBR quota is assigned per radio part and enforced by control of the scheduling grants
=> problem 2 exists

- however, in CU-DU distribution, in the case of EN-DC, it is signalled to the CU only

# 3	Solutions

Solution A1: Existing method

- No changes to the principles of the existing solution.

- Only enhancement needed is the information on the UL AMBR quota to be transferred to the DU from the CU.

Expected changes:

- CR to F1AP: UL AMBR quota

Solution A2: Existing method, with event trigger reporting (overload condition indicator over CP or UP)

- UL AMBR quota transferred to DU (as A1)

- DU sends indication and/or current throughput info (F1 CP, possibly UP) when quota is close to being exceeded over time window, CU adjusts if possible

- CU may need to request changes of the quota from the MN (X2 CP, possibly UP)

- In order to enable the CU to be able to offer more AMBR quota to the DU:

  - SN sends indication and/or throughput info when overall quota is close to being exceeded; MN adjusts if possible; or

  - MN and SN exchange “consumed” UL throughput information and each node may use the AMBR quota not used by the other node (DL may be included too, to help solving problem 1 in DL).

Expected changes:

- CR to F1AP: UL AMBR quota;

- CR to F1AP or NR UP: quota limit reached and/or throughput information

- CR to X2AP and/or NR UP:

  - quota limit reached and/or throughput information; or

  - UL / DL “consumed” throughput information

Solution B1: UL enforcement at the PDCP level

- No changes in DL enforcement (the same as existing solution for enforcing DL AMBR).

- In UL, the AMBR eliminates overflowing traffic by discarding selected PDUs.

Expected changes:

- CR to stage-2: change of the UL enforcement from the scheduler to the PDCP

Solution B2: UL enforcement at the PDCP level, with enforcement at the scheduler

- No changes in DL enforcement (the same as existing solution for enforcing DL AMBR).

- In UL, the AMBR monitors traffic;

- If UL traffic exceeds the limit, the hosting node notifies the assisting node and/or the DU over CP or UP;
FFS: can the scheduler throttle single DRB?

- Optionally, MN and SN exchange “consumed” UL & DL throughput information and each node may use the AMBR quota not used by the other node

Expected changes:

- CR to stage-2: change of the UL enforcement from the scheduler to the PDCP

- CR to F1AP or NR UP: enforcement indication

- Optionally: CR to X2AP and/or NR UP:

  - quota limit reached and/or throughput information; or

  - UL / DL “consumed” throughput information