3GPP TSG-RAN WG3#99bis	R3-182393

Sanya, China, 16 – 20 April 2018

Agenda item:	10.1.1

Source:	Nokia, Nokia Shanghai Bell, Huawei

Title:	Handling of Notification Control in TS 38.413

Document for:	Discussion and Decision

# Description

The handling of notification was further clarified at last RAN2 meeting. LS was sent in tdoc [3]:

In RAN2’s understanding the gNB is allowed to  release a bearer/QoS flow for which a notification control to the CN is sent, and for which a response or action have not been received from the CN. This might be needed e.g. in overload conditions or in case of RLF. Concerns were expressed that not allowing the gNB to release a flow when notification control is enabled would leave radio resources under the control of 5GC.

It was thus clarified that upon receiving the Notification control for a given GBR flow in the PDU session setup message the gNB is free to decide between two behaviours:

- Notify when GBR flow is “not more fulfilled” and “fulfilled again”,

- Release the GBR flow.

The key point to retain is that gNB can decide to release the GBR flow even though it received the Notification Control IE. Therefore, the Notification Control IE is to be understood as an “enabling to Notify the CN” but not necessarily as a “request to Notify the CN”. The gNB behaviour can thus be described two folds:

- If gNB has NOT previously received the Notification Control IE in PDU Session setup and the QoS can no longer be fulfilled for a GBR QoS flow, then the gNB shall release the GBR flow.

- If the gNB has previously received the Notification Control IE in PDU session setup and the QoS can no longer be fulfilled for a GBR QoS flow, then the gNB shall either release the GBR flow or it can keep the GBR flow but then shall notify the 5GC.

Proposal 1: clarify the semantics of the Notification Control IE in view of last RAN2/SA2 decisions as proposed in the TP below for section 9.3.1.10 of TS 38.413.

Besides, when the gNB decides to release a QoS flow the 5GC should be informed to take appropriate actions like in LTE. In LTE the E-RAB Release Notification had been introduced as a class 2 message, therefore it seems natural that similar message is introduced.

Proposal 2: introduce a class 2 message from NG-RAN node to inform the 5GC of released QoS flows.

Besides, it was left open at last RAN3 whether to decouple the notification control reporting towards the SMF and the release notification for QoS flows.

While some form of decoupling is interesting, both are notifications towards the SMF and implementation should be optimized while keeping the two decoupled. For example, if the NG-RAN was in direct visibility of the SMF, the decoupling would be achieved by using two separate messages from NG-RAN node to SMF. However, given that AMF terminates the notification, and following our container approach, the AMF should not be aware of the decoupling and a real decoupling should instead be achieved by using one container towards AMF inside which the NG-RAN node should include two separate IEs to inform the SMF of for a given PDU session resources:

- Qos flows released by NG-RAN node,

- Qos flows fulfilled, not fulfilled by NG-RAN node.

These two separate IEs should obviously either be included alone, or together if both flows not fulfilled and released flows are to be reported. Therefore, even if separate they should not be in a choice structure but separate with presence optional.

This seems the right trade-off to implement the decoupling while optimizing the NGAP coding to the AMF.

Proposal 3: include one container in the PDU Session Resource Notify message including two separate optional IEs to report QoS flows released and/or QoS flows fulfilled/not fulfilled.

At the same time, for the reasons explained by RAN2 here-above (see [3]), it could happen that the NG-RAN node even releases all the RAN resources of a PDU session at a time. As we do for other messages, that PDU session resource level should then instead be visible to the AMF (like PDU session resources failed to setup, etc..).

Proposal 4: include a PDU Session Resource Released List in the PDU Session Resource Notify message.

# Proposal

It is proposed to agree the Text Proposal for TS 38.413 presented here-below which implements the four proposals above.

# References

- RP-172109, Revised Work Item on New Radio (NR) Access Technology, NTT DOCOMO

- 3GPP TS 23.501, Architecture for Next Generation System, SA2

- R2-181645, LS on QoS,

# 3	Text Proposal for TS 38.413

#### 8.2.4.2	Successful Operation

Figure 8.2.4.2-1: PDU session resource notify

The NG-RAN node initiates the procedure by sending a PDU SESSION RESOURCE NOTIFY message.

The PDU SESSION RESOURCE NOTIFY message shall contain the information of PDU sessions or QoS flows which are released or not fulfilled anymore or fulfilled again by the NG-RAN node.

-	For each PDU session of which some QoS flows are released or not fulfilled anymore or fulfilled again by the NG-RAN node, the PDU Session Resource Notify Transfer IE shall be included to report for each PDU session of which some QoS flow(s) are released or not fulfilled anymore or fulfilled again :

1. 	The list of QoS flows which are released by the NG-RAN node, if any, shall be included in the QoS Flows Release List IE.

2. 	The list of GBR QoS flows which are not fulfilled anymore or fulfilled again by the NG-RAN node, if any, shall be included in the QoS Flows Notify List IE together with the Notification Cause IE.

-	For each PDU session which is released by the NG-RAN node, the PDU Session Resource Release List IE shall be included to report including the release cause in the Cause IE.

Upon reception of the PDU SESSION RESOURCE NOTIFY message, the AMF shall, for each PDU session indicated in the PDU Session ID IE, transfer transparently the PDU Session Resource Notify Transfer IE to each SMF associated with the concerned PDU session.  Upon reception of PDU Session Resource Notify Transfer IE, SMF normally initiate the appropriate release or modify procedure on the core network side for the QoS flow(s) identified as not fulfilled anymore .

Editor’s Note:	Further details are FFS.

Next change

#### 9.3.1.10	GBR QoS Flow Information

Editor’s Note:	Further details FFS.

This IE indicates QoS parameters for a GBR QoS flow for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum Flow Bit Rate Downlink | M |  | Bit Rate 9.3.1.4 | Maximum Bit Rate in DL. Details in TS 23.501 [9]. | - | - |
| Maximum Flow Bit Rate Uplink | M |  | Bit Rate 9.3.1.4 | Maximum Bit Rate in UL. Details in TS 23.501 [9]. | - | - |
| Guaranteed Flow Bit Rate Downlink | M |  | Bit Rate 9.3.1.4 | Guaranteed Bit Rate (provided there is data to deliver) in DL. Details in TS 23.501 [9]. | - | - |
| Guaranteed Flow Bit Rate Uplink | M |  | Bit Rate 9.3.1.4 | Guaranteed Bit Rate (provided there is data to deliver). Details in TS 23.501 [9]. | - | - |
| Notification Control | O |  | ENUMERATED (notification enabled, ...) | Details in TS 23.501 [9]. | - | - |
| Maximum Packet Loss Rate Downlink | O |  | <ref> | Indicates the maximum rate for lost packets that can be tolerated in the downlink direction. Details in TS 23.501 [9]. | - | - |
| Maximum Packet Loss Rate Uplink | O |  | <ref> | Indicates the maximum rate for lost packets that can be tolerated in the uplink direction. Details in TS 23.501 [9]. | - | - |

Next change

#### 9.2.1.7	PDU SESSION RESOURCE NOTIFY

Editor’s Note:	Message structure and IEs need further checking and completion. Further details FFS.

This message is sent by the NG-RAN node to notify that the already established QoS flow(s) or PDU session(s) for a given UE are released or not fulfilled anymore or fulfilled again by the NG-RAN node.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| PDU Session Resource Notify List |  | 0..1 |  |  | YES | reject |
| >PDU Session Resource Notify Item IEs |  | 1..<maxnoofPDUSessions> |  |  | EACH | reject |
| >>PDU Session ID | M |  | <ref> |  | - |  |
| >>PDU Session Resource Notify Transfer | M |  | 9.3.4.7 |  | - |  |
| PDU Session Resource Released List | O |  | PDU Session List 9.3.1.11 |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions allowed towards one UE. Value is FFS. |

Next change

#### 9.3.4.7	PDU Session Resource Notify Transfer

Editor’s Note: 	Further details FFS.

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| QoS Flows Notify List |  | 0..1 |  |  | - |  |
| >QoS Flows Notify Item IEs |  | 1..<maxnoofQoSFlows> |  |  | - |  |
| >>QoS Flow Indicator | M |  | <ref> |  | EACH | reject |
| >>Notification Cause | M |  | ENUMERATED(fullfilled, not fulfilled, …) |  | - |  |
| >>Assistance Information [FFS] | O |  | <ref> |  | - |  |
| QoS Flows Release List | O |  | QoS Flow List 9.3.1.13 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofQoSFlows | Maximum no. of QoS flows allowed within one PDU session. Value is 64. |