3GPP TSG-RAN WG3 AdHoc	R3-101965

Beijing, China, June 29th  –  July 01st , 2010

Agenda Item:	7.3

Source: 	Motorola, Nokia Siemens Networks, NTT DOCOMO, Ericsson, Huawei

Title:  	Handling of neighboring info and choice of HO type

Document for:	Approval

# Introduction

During the RAN3 AH meeting on H(e)NB and Relaying, the choice of HO type and the ANR implications was discussed. This contribution summarizes the conclusions of such discussion.

# Handling of neighboring information and HO type

It is assumed that

- RN uses following information to determine whether it can initiate an X2 HO to the target cell

  - The “No X2” attribute and “No HO” attribute in NRT.

  - The information whether DeNB have an X2 connection with the neighboring eNB via eNB Configuration Update procedure, or X2 Setup procedure. It is FFS on how to carry this type of information in eNB Configuration Update procedure and X2 Setup procedure.

  - The information of target eNB’s GU Group, and the MME selected by the DeNB. It is FFS how RN knows the target eNB’s GU Group ID information and which MME is selected by the DeNB .

- It is FFS on how to ensure the NRT attribute in RN and DeNB are consistent..

# Proposal

RAN3 is kindly asked to acknowledge and confirm the assumptions made in clause 2 as a way forward when continuing the work on handling of neighboring info and choice of HO type for relay.

# References