3GPP TSG RAN WG3 Meeting NR ADHOC                   R3-180133

# Sophia Antipolis, France, 22nd– 26th Jan 2018

Agenda item:    10.10.8

Source: 	ZTE

Title: 	Discussion on load management over F1 interface

Document for:	Discussion and Approval

Introduction

During RAN3#98 meeting, it was agreed that load management should be postponed to the NR Adhoc meeting in January. In this paper, we focus on the load management related issues.

Discussion

In TS36.423 [1], the reporting of load measurements between eNBs can be achieved by Resource Status Reporting procedure. In CU-DU split case, the reporting of DU load measurements needs to be considered, which can help CU make RRM decision,e.g., mobility, PSCell/PCell/SCell management.

The aim of resource status reporting over F1 interface is that gNB-CU gets the load measurements of gNB-DU. So, the resource status reporting should be initiated by gNB-CU. In order to match the load management function over F1 interface, we change the name of load management signalling as Load Status Request, Load Status Response and Load Status Update.

From the above, the load management function includes the following steps:

- Load Status Request: This procedure allows the gNB-CU to initiate the requested load measurement in gNB-DU.

- Load Status Response: This procedure allows the gNB-DU to response that the requested load measurement in gNB-DU has been initiated.

- Load Status Update: This procedure allows gNB-DU to report the load measurement to gNB-CU.

In detail, the load status in the report could be hardware load status, the status of the F1 transport network Load and resource utilization ratio of each cell controlled by DU, etc.

Proposal1: It is proposed to introduce the load management function over F1 interface as above.

Conclusion

The following proposals are provided:

Proposal1: It is proposed that load management over F1 interface should be initiated by gNB-CU.

References

[1] TS 36.423 V14.4.0 X2 Application Protocol (X2AP).

- Text Proposal for TS 38.470

5.2.5	Load management function

This function allows gNB-CU to get the resource status from gNB-DU. The gNB-CU is responsible for the initiation of the load measurement, and the gNB-DU responses and reports the load measurement to gNB-CU which including the hardware load status, the status of the F1 transport network load and resource utilization ratio of each cell controlled by DU, etc.