3GPP TSG-RAN WG3 #NR Adhoc 1801   						   R3-180330

Sophia Antipolis, France, 22-26 January 2018

Agenda item:	10.10.7

Source:	CMCC

Title:	QoS management over F1

Document for:	Discussion and Decision

Introduction

At #97bis RAN3 meeting, QoS configuration and admission control issues are discussed. Three alternative solutions are proposed as indicated in [1],

1) CU signals QoS profile per DRB level, DU either accepts or rejects

2) CU signals QoS profile per-traffic flow, DU decides DRB QoS config and signals a list of admitted/failed DRBs

3) CU signals QoS profile per-traffic flow, DU decides DRB QoS config and signals a list of admitted/failed flows

In this contribution, we will give further analysis on this issue.

Discussion

In CU-DU architecture, SDAP and PDCP layers are located in the gNB-CU, where the SDAP layer maps QoS flow into DRB while the PDCP layer generates the PDCP PDUs of each DRB. PDCP PDUs are forwarded over the F1 interface to the gNB-DU for further processing. Considering the fact that the mapping between QoS flows and radio bearers is performed by the gNB-CU, the granularity of data delivery over F1 is radio bearer. Generally, the QoS flow can be invisible to the gNB-DU. So it is reasonable for gNB-CU to perform QoS configuration and provide DRB QoS profile to gNB-DU.

On the other hand, if the QoS configuration is done in gNB-DU as alternative 2 indicates, gNB-CU is not aware of the QoS configuration, thus only blind or very coarse mapping between QoS flow to DRB can be performed at gNB-CU. It may happen that CU maps a QoS flow to the DRB which does not tailored for implementing the QoS profile.

Proposal 1: gNB-CU performs QoS flow to DRB mapping and decides QoS configuration for DRB.

Another aspect we should consider is the admission control for the QoS Flow. The admission control for the QoS flow is a non-real time processing, and in general should be based on the stable decision principle, which means that we should not reject the QoS Flow based on the instantaneous capability. The gNB-CU can handle the admission control for the Qos flow by its own polices without the gNB-DU involved. The gNB-CU reports the admission result based on the resource requirement of each individual PDU session to the AMF. More specifically, after successful setup of each PDU session, the gNB-CU shall include the PDU Session Setup Response Transfer IE containing QoS Flows Setup List IE and QoS Flows Failed to Setup List IE. If the gNB-DU is involved in the QoS flow level Admission Control, the interaction between the gNB-CU and the gNB-DU is required. However, it is difficult to achieve a unified policy on the admission control between the gNB-CU and the gNB-DU. In conclusion, it is effective and simple for gNB-CU to handle the admission control for the QoS flow.

Proposal 2: gNB-CU signals DRB level QoS configurations to the gNB-DU over F1, and then the gNB-DU follows the DRB level QoS profile or rejects the DRB.

In summary, alternative 1 is preferred and the corresponding CR to TS 38.473 is provided in the annex.

Conclusion

Based on the discussion on QoS configuration and admission control, the following proposals are made:

Proposal 1: gNB-CU performs QoS flow to DRB mapping and decides QoS configuration for DRB

Proposal 2: gNB-CU signals DRB level QoS configurations to the gNB-DU over F1, and then the gNB-DU follows the DRB level QoS profile or rejects the DRB.

Reference

RAN3_97bis_Agenda_with_Tdocs_20171013_EOM

- Annex: Text Proposal for TS 38.473

---------------------------------------------------Start of the change--------------------------------------------

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| Target cell ID (FFS) | FFS |  | FFS |  | FFS | FFS |
| PSCell ID | O |  | NCGI 9.3.1.12 | PSCell Identifier in SgNB | YES | Ignore |
| CU to DU RRC Information | M |  | 9.3.1.25 |  | YES | reject |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.1.X.1 of TS 36.423 [9]. | YES | reject |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | 0 .. <maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NCGI 9.3.1.12 | SCell Identifier in SgNB | - | - |
| SRB to Be Setup List |  | 0..1 |  |  |  |  |
| >SRB to Be Setup Item IEs |  | 1 .. <maxnoofSRBs>] |  |  |  |  |
| >SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Setup List |  | 1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | 1 .. <maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>DRB QoS profile | M |  |  | Includes configurations for RLC/MAC/PHY parameters related to this DRB. | - |  |
| >>E-UTRAN QoS | O |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >> Tunnels to be setup List |  | 1 |  |  |  |  |
| >>>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofULTunnels> |  |  |  |  |
| >>>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - | - |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 64. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofULTunnels | Maximum no. of tunnels allowed towards one DRB, the maximum value is 2. |

------------------------------------------------Start of the next change----------------------------------------

- 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| PSCell ID | O |  | NCGI 9.3.1.12 | PSCell Identifier in SgNB | YES | Ignore |
| DRX Cycle | O |  | DRX Cycle 9.3.1.24 |  | YES | ignore |
| CU to DU RRC Information | O |  | 9.3.1.25 |  | YES | reject |
| Transmission Stop Indicator | O |  | 9.3.1.11 |  | YES | ignore |
| Resource Coordination Transfer Container | O |  | OCTET STRING | Includes the MeNB Resource Coordination Information IE as defined in subclause 9.1.X.1 of TS 36.423 [9]. | YES | reject |
| RRC-Container | O |  | 9.3.1.6 |  | YES | ignore |
| SCell To Be Setup List |  | 0..1 |  |  | YES | ignore |
| >SCell to Be Setup Item IEs |  | <0 .. maxnoofSCells> |  |  | EACH | ignore |
| >>SCell ID | M |  | NCGI 9.3.1.12 | SCell Identifier in SgNB | - | - |
| SRB to Be Setup List |  | 0..1 |  |  |  |  |
| >SRB to Be Setup Item IEs |  | <1..maxnoofSRBs> |  |  |  |  |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Setup List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Setup Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>DRB QoS profile | M |  |  | Includes configurations for RLC/MAC/PHY parameters related to this DRB. | - |  |
| >>E-UTRAN QoS | O |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >> Tunnels to be setup List |  | 1 |  |  |  |  |
| >>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofULTunnels> |  |  |  |  |
| >>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - | - |
| DRB to Be Modified List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Modified Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |
| >>DRB QoS profile | M |  |  | Includes configurations for RLC/MAC/PHY parameters related to this DRB. | - |  |
| >>E-UTRAN QoS | O |  | 9.3.1.19 | Shall be used for EN-DC case to convey E-RAB Level QoS Parameters |  |  |
| >> Tunnels to be setup List |  | 0..1 |  |  |  |  |
| >>>Tunnels to Be Setup Item IEs |  | 1 .. <maxnoofULTunnels> |  |  |  |  |
| >>>>UL GTP Tunnel Endpoint | M |  | GTP Tunnel Endpoint 9.3.2.1 | gNB-CU endpoint of the F1 transport bearer. For delivery of UL PDUs. | - | - |
| SRB TO Be Released List |  | 0..1 |  |  |  |  |
| >SRB To Be Released Item IEs |  | <1..maxnoofSRBs> |  |  |  |  |
| >>SRB ID | M |  | 9.3.1.7 |  |  |  |
| DRB to Be Released List |  | 0..1 |  |  | YES | reject |
| >DRB to Be Released Item IEs |  | <1 .. maxnoofDRBs> |  |  | EACH | reject |
| >>DRB ID | M |  | 9.3.1.8 |  | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofSCells | Maximum no. of SCells allowed towards one UE, the maximum value is 64. |
| maxnoofSRBs | Maximum no. of SRB allowed towards one UE, the maximum value is 8. |
| maxnoofDRBs | Maximum no. of DRB allowed towards one UE, the maximum value is 64. |
| maxnoofDLTunnels | Maximum no. of tunnels allowed towards one DRB, the maximum value is 2. |

-----------------------------------------------End of the change --------------------------------------------