3GPP TSG-RAN WG3 #AH-1801	R3-180382

# Sophia Antipolis, France, 22nd – 26th  January 2018

Agenda Item:	10.1.1

Source:	Ericsson

Title:	Delay critical GBR QoS flows – stage 3 solution

Document for:	pCR

# Text Proposal for NGAP TS 38.423 v0.5.0

<<<<<<<<<<<<<<<<<<<< Begin Text Proposal >>>>>>>>>>>>>>>>>>>>

### 9.2.2	GBR QoS Flow Information

This IE indicates the maximum and guaranteed bit rates of a GBR QoS flows for downlink and uplink.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Maximum Flow Bit Rate Downlink | M |  | 9.2.3 | Maximum Bit Rate in DL. Details in TS 23.501 [7]. | – | – |
| Maximum Flow Bit Rate Uplink | M |  | 9.2.3 | Maximum Bit Rate in UL. Details in TS 23.501 [7]. | – | – |
| Guaranteed Flow Bit Rate Downlink | M |  | 9.2.3 | Guaranteed Bit Rate (provided that there is data to deliver) in DL. Details in TS 23.501 [7]. | – | – |
| Guaranteed Flow Bit Rate Uplink | M |  | 9.2.3 | Guaranteed Bit Rate (provided that there is data to deliver). Details in TS 23.501 [7]. | – | – |
| Delay Critical GBR | O |  | ENUMERATED {True, ...} | Indicates if the flow is Delay Critical GBR. Details in TS 23.501 [7]. Editor Note: may not needed if special QoS values are defined for Delay Critical GBRs. | – | – |
| Maximum Data Burst Volume | O |  | <ref> | For details refer to TS 23.501 [7]. | - | - |

<<<<<<<<<<<<<<<<<<<< End Text Proposal  >>>>>>>>>>>>>>>>>>>>