3GPP TSG-RAN WG3 #AH-1801	R3-180383

# Sophia Antipolis, France, 22nd – 26th  January 2018

Agenda Item:	10.1.2

Source:	Ericsson

Title:	Further Discussion on QoS setting for default DRBs

Document for:	Discussion, Decision

# Introduction

At the past RAN3 meeting, the need for a reference QoS profile for the default DRBs settings have been discussed but not concluded.

This paper discusses further on this issue.

# Discussion

RAN3 has clarified that the default QoS profile/ default QoS rule is provided to UE via NAS which UE will use. Hence it is not aware in RAN. For the sake of clarify, we do not use default QoS in the below discussion, instead we use reference QoS ( refQoS in short.)

One proposal is to let 5GC to indicate the reference QoS profile among the NAS-level QoS profiles configured to RAN at the PDU session set-up which can be used by RAN to establish default DRB tailored to the PDU session characteristics. The reason given is that “gNB is in fact unaware of the nature of the PDU session.”

In our opinion, during PDU session setup, 5GC provides enough information of the nature of the PDU session, which includes:

- PDU session ID;

- QoS flows included in this PDU session, as well as QoS level QoS parameters;

We should highlight the fact that the QFI is mandatory present and the QoS is either configured in RAN over NGAP or it is a standardized 5QI. RAN shall not accept incorrect QFIs, so there should be no case that QFI accepted by RAN is unknown.

Observation 1: RAN always know the QFIs being accepted.

We believe in 5G, PDU session and QoS flows are introduced to bring flexibility so there will be no more one-one QoS flow and DRB mapping. The principle is that RAN is responsible for the DRB mapping. Thus RAN is solely responsible to decide the QoS profile for the DRBs. Only RAN has the ability to consider the information from the 5GC and the knowledge about the RAN resource and decide a QoS profile for the DRB settings.

Observation 2: RAN does not need extra information from 5GC for the QoS settings on the DRBs.

RAN3 has concluded that “RAN decides the QoS mapping and respective resource configuration for DRBs (no challenge)”

Is it then useful to have the refQoS as recommendation from CN? It is a recommendation from CN point of view, if used, it may lead to that RAN choose a QoS profile which cannot best cope the RAN resource situation. Considering the fact that when different QoS flows are setup, RAN may accept some of the QoS flows and reject some other. If the refQoS profile is associated with a certain QoS flow (as it was proposed), it could be that this QoS flow is actually rejected by RAN. In our opinion, by providing such a reference QoS may cause confusion.

Observation 3:  The QoS flow with refQoS associated might be rejected by RAN. In our opinion, by providing such a refQoS may cause confusion.

Proposal 1: RAN3 to agree that the QoS profile setting on DRBs is totally a RAN decision. There is no need for any additional QoS profile signalled from CN.

# Conclusion

Proposal 1: RAN3 to agree that the QoS profile setting on DRBs is totally a RAN decision. There is no need for any additional QoS profile signalled from CN.

# References

R3-173930	Discussion on QoS setting for default DRBs, Ericsson