3GPP TSG RAN WG3 NR AH 18-07	R3-184088

Montreal, Canada, 2nd – 6th July 2018

Agenda Item:	31.3.5

Source:	Ericsson

Title:	Latest findings on UL AMBR

Document for:	Discussion, Decision

# Introduction

At the last RAN3 meeting, the monitoring and enforcement of UL AMBR was discussed, and a way-forward document was provided in R3-183522 [1]. Some companies supported the proposed way-forward, but the document was not agreed.

Following the discussion at the meeting, we carried out more internal studies. We identified that the solution proposed in R3-183522 may introduce a significant amount of network signalling, which may overload the control plane interfaces and delay other more critical procedures. Furthermore, the advantages of introducing soft UL AMBR portions are questionable.

In conclusion, we propose to follow a simple approach in Rel-15 where only hard UL AMBR thresholds are exchanged over the network interfaces. The corresponding CR for F1AP is provided in R3-184089 [2].

# Discussion

As discussed in R3-183336 [3], two important requirements for the solution for UL AMBR enforcement are:

- Allowing fast and flexible shifting of UL throughput from one radio leg to another, for UEs that are in dual connectivity and for UEs that in future releases may be in multi connectivity;

- Avoid signalling intensive solutions that can overload the network interfaces, thus risking delaying more critical procedures and disrupting network behavior.

A solution that tries to accommodate both the two requirements above was outlined in R3-183522 and is summarized in the following:

- The UE UL AMBR can be split into portions assigned to MN and SN;

- The gNB-CU assigns portions of UL AMBR to the gNB-DU(s);

- The portions of UL AMBR can be soft (i.e., can be breached) or hard (i.e., cannot be breached);

- Enforcement of UE UL AMBR for EN-DC and MR-DC occurs at MAC;

- If a breach of UL AMBR portion occurs and if the threshold is soft, the node hosting MAC reports the breach to MN (via the gNB-CU in case of corresponding gNB-DU) together with throughput information (details of the reporting are FFS);

- The MN responds with a command (eventually passed to gNB-DU via gNB-CU) to either enforce the current limit or to configure a new limit (details are FFS);

- This request / response mechanism above is supported over the F1-C and the X2-C interfaces (support over Xn and related details are FFS).

The solution above may seem attractive in high level, but when digging into the details one can see that is incomplete and it under-estimates the amount of signalling required over the network interfaces. The main problems arise from the fact that:

- Radio conditions vary dynamically and in the short time range. Therefore, it is very difficult for the node hosting MAC to decide when to report a breach in the UL AMBR.

  - If the node hosting MAC reports immediately after detecting a breach, there is a risk that breach reporting will be very frequent. It may also happen that after sending a breach report the UL radio conditions have already changed, so that the report will not be up-to-date when reaching the receiving node, which may lead to wrong or unnecessary re-configurations.

  - If the node hosting MAC waits too long before reporting a breach, the UE may UL more traffic than what is allowed for a relatively long time. This may violate the conditions of the UE subscription and it may affect other UEs.

- The node receiving a breach report may not be able to decide the action to take e.g., because it does not know the UL status for the UE over other radio legs. The only solution is to generate additional signalling that will overload the network interfaces. To explain better this issue, we provide an example in the following, taking the CU-DU split architecture as a reference.

Once the gNB-DU1 reports a breach of UL AMBR in step 5, the gNB-CU may have no information about the UL status in gNB-DU2. Therefore, it may not be able to select an action (e.g., enforce current UL AMBR portion or update the UL AMBR portion).

To obtain the UL status in gNB-DU2 a new procedure may be needed over F1 (step 6). This will cause extra signalling over the F1 interfaces. The situation would be even worse in EN-DC, where new procedure and extra signalling would be needed also on the X2 interface.

Overall, combining issue 1 and 2, it becomes clear that there is a big risk of generating a lot of extra-signalling over the network interfaces that can overload and destabilize the network. Therefore, we conclude that for Rel-15 it is better to rely on a simple solution to monitor and enforce UL AMBR that consists of using only hard UL AMBR portions. If this will become a serious limitation, solutions may be considered in future releases.

Proposal 1: 	To avoid the risk of overloading the network interfaces, define only hard UE UL AMBR portions (at least in Rel-15).

Proposal 2: 	Agree with the CR in R3-184089 to introduce a hard UE UL AMBR portion in the UE Context Setup Request and UE Context Modification Request messages.

# Conclusion

In this contribution, we analysed more in details the solutions prosed for enforcing UE UL AMBR.

Proposal 1: 	To avoid the risk of overloading the network interfaces, define only hard UE UL AMBR portions (at least in Rel-15).

Proposal 2: 	Agree with the CR in R3-184089 to introduce a hard UE UL AMBR portion in the UE Context Setup Request and UE Context Modification Request messages.

# References

[1] R3-183522, “Summary of offline discussion on UL UE AMBR”

[2] R3-184089, “Introduction of UL AMBR on F1”, Ericsson

[3] R3-183336, “Summary and way forward of discussions on UL AMBR”, Ericsson