3GPP TSG-RAN WG3 Meeting # Ad-hoc	R3-170007

Spokane, Washington, USA, Jan. 17th – 19th, 2017

Agenda item:		10.3.2

Source:	LG Electronics Inc.

Title:	Text Proposal for NG Interface

Document for:		Discussion

1. Text Proposal

----------------Start of the First Change---------------

7.2.4	NG Interface Procedures

To support the functions listed in Section 7.2.3 the NG interface should support the following procedures. The procedures are classified in different categories.

Interface management procedures:

-	NG Setup: To establish an NG interface between the New RAN and NGC nodes

-	Configuration Updates from New RAN and NGC nodes: To update configuration of the interface from the New RAN or NGC (FFS)

-	NG Reset: To reset the NG interface

-	Error Indication: To report detected errors in one incoming message

UE Context Management Procedures:

-	Initial Context Setup: To establish the initial UE context both in New RAN and in NGC

-	UE Context Release: To remove UE context information

-	UE Context Modification: To modify an already established UE context (FFS)

UE Mobility Management Procedures:

-	Handover Preparation: Needed to prepare resource allocation from the source RAN to the NGC for a UE handing over

-	Handover Resource Allocation: Needed to reserve resources at the target RAN for a UE handing over

-	Path Switch Request: Needed to request the switch of a UP connection to a different UP GW at the NGC

-	Handover Cancel: Needed to cancel an ongoing handover preparation or an already prepared handover

-	Handover Notification: Needed to indicate to the NGC that handover has been successfully completed

Transport of NAS Messages Procedures:

-	Initial UE Message: To transport the first NAS PDU sent when no UE signaling connection is established from the UE to the NGC (FFS)

-	DL NAS Transport: To transport NAS PDUs in DL

-	UL NAS Transport: To transport NAS PDUs in UL

-	Non Delivery NAS Indication: To indicate that the New RAN node did not deliver a NAS PDU to the UE (FFS)

-	NAS Rerouting: In order to route a NAS request to a different CN node (FFS)

Paging Procedures:

-	Paging: To provide a paging instruction from the NGC to the New RAN

PDU Session Management Procedures:

-	PDU Session Setup: Needed to establish a PDU Session and assign resources to it according to the QoS and other configuration parameters assigned to the flow(s) configured within the session

-	PDU Session Modify: Needed to modify a previously configured PDU session, related QoS flows and respective New RAN resources

-	PDU Session Release: Needed to release a previously configured PDU Session and related New RAN resources

-	PDU Session Modification Indication: Needed to switch a PDU Session from one termination point at the New RAN to a different termination point.

----------------End of the First Change---------------

----------------Start of the Second Change---------------

7.2.4.1	NG Interface Procedures Descriptions

This section describes NG procedures. The NG interface can terminate in an eLTE eNB or in a gNB. Therefore, NG procedures apply to both an eLTE eNB and a gNB.

7.2.4.1.1	Interface Management Procedures

NG Setup Procedure

Figure 7.2.4.1.1-1: Example of NG Setup procedure

This procedure is used to setup the NG interface. The procedure enables exchange of configuration parameters between the RAN and the NGC.

7.2.4.1.2	UE Context Management Procedures

Initial Context Setup

Figure 7.2.4.1.2-1: Example of Initial Context Setup procedure

This procedure is used to establish a context at the New RAN for a given UE. In this procedure, UE context related information held in the NGC are signalled to the New RAN.

UE Context Release

Figure 7.2.4.1.2-2: Example of UE Context Release Request procedure

Figure 7.2.4.1.2-3: Example of UE Context Release procedure

This procedure consists of two sequences of messages: one initiated by the New RAN and requesting the NGC to initiate a UE context release; the other initiated directly by the NGC.

This procedure is used to remove an already established UE context at any time.

7.2.4.1.3	Transport of NAS Messages Procedures

UL NAS Transport and DL NAS Transport

Figure 7.2.4.1.3-1: Example of Uplink NAS Transport procedure

Figure 7.2.4.1.3-2: Example of Downlink NAS Transport procedure

These two procedures are needed in order to allow an immediate and independent transmission of NAS PDUs between New RAN and NGC.

7.2.4.1.x	UE Mobility Management Procedures

Handover Preparation

Figure 7.2.4.1.x-1: Example of Handover Preparation procedure

This procedure is to request the preparation of resources at the target side via the NGC.

Handover Resource Allocation

Figure 7.2.4.1.x-2: Example of Handover Resource Allocation procedure

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target gNB for the handover of a UE.

Path Switch Request

Figure 7.2.4.1.x-3: Example of Path Switch Request procedure

The purpose of the Path Switch Request procedure is to request the switch of a downlink GTP tunnel towards a new GTP tunnel endpoint.

Handover Cancellation

Figure 7.2.4.1.x-4: Example of Handover Cancel

The purpose of the Handover Cancel procedure is to enable a source gNB to cancel an ongoing handover preparation or an already prepared handover.

Handover Notification

Figure 7.2.4.1.x-5: Example of Handover Notification procedure

----------------End of the Second Change---------------