3GPP TSG RAN WG3 Meeting #Adhoc	                                        R3-170075

Spokane, Washington, USA, 17th – 19th January 2017

Source:	CATT

Title:	Consideration on RAN architecture for CU/DU split

Agenda Item:	10.7.1

Document for:	Discussion and Decision

Introduction

At RAN3#94 meeting, it is clarified that the specification aspect of the interface between CU and DU should be studied.

In this contribution, we make some analysis on this topic and give some suggestions on the definitions of CU/DU and RAN internal architecture.

Discussion

2.1 Definition of CU and DU

Currently, 8 CU/DU function split options are captured in RAN3 TR[1] which includes high layer split options and low layer split options. In different CU/DU split option, the functions of CU and DU may be different. However, the DU/CU in different options have the same characters as follows:

CU:

- One CU can connect and manage one or multiple DUs

- CUs provide the control plane interface i.e. NG-c to CN

- CUs provide the user plane interface i.e. NG-u to CN (DU may directly have user plane interface to CN. This would depend on functionality split for DU and CU. For example, if all UP functions are handled at DU i.e. option 1,however,high layer split options only consider option 2 and option 3,option 1 seems to be ruled out)

- Xn interface terminated in CU.

- CU provide functions of RRC layer and higher UP functions

DU:

For DU, we would like to clarify the relationship between TRP and DU.TRPs are essential RAN elements that constitute one major aspect of recent LTE features e.g. CoMP and are expected to further play a key role in the increase of access point density. Therefore it is important to clarify TRP and DU mapping: we could either consider a DU is a remote site hosting one or multiple TRPs or simply have a one to one DU – TRP mapping. The former case would call for a complex hierarchical RAN model with CU/DU/TRP levels. Hence to keep RAN model simple we propose adopting the latter mapping. On the other hand, the same TRP can operate on multiple carriers or bands. Hence similarly, we suggest that the same DU can operate one multiple carriers or bands. At the moment, we don’t see the need or use case for a DU connected/controlled to/by multiple concurrent CUs and we suggest a DU only connects to one CU.

In summary, we foresee the following characters for DU:

- A DU is mapped onto one TRP

- A DU only connects to one CU

- A DU provide low layer functions

Based on the above analysis, we propose the following definition of CU and DU

CU:The central unit which provides high layer functions of gNB and terminate the NG interface to CN and Xn interface to other gNBs

DU:The distribute unit in gNB which directly connects to CU through NGi interface and provide low layer function of gNB

Proposal 1:It is proposed to capture the above definition of CU and DU into the TR.

2.2 RAN internal architecture

As described in 2.1, one CU could connect and control multiple DUs while DU only need to connect to one CU.So,we propose the following RAN internal architecture:

Figure 1 5G NR Architecture

Proposal 2: it’s proposed to capture the above RAN internal architecture into TR.

Conclusion

Based on the discussion in section 2 the followings are proposed:

Proposal 1: It is proposed to capture the above definition on CU and DU into TR.

Proposal 2: It is proposed to capture the above RAN internal architecture into TR.

Reference

[1] R3-162236  NFV Terminology  Intel