3GPP TSG-RAN WG3 Meeting #Ad Hoc	R3-170182

Spokane, Washington, USA, 17 – 19, January, 2017

Agenda Item:	   10.3.2

Source: 	   Ericsson

Title: 	   Clarifications on Initial UE Access

Document for:	Discussion and Decision

# Introduction

In TR38.801 a description of the Initial UE Access was added. In this contribution corrections to such procedure’s description is proposed.

# Initial UE Access Procedure

The following description has been captured for the Initial UE Access procedure:

### 10.2.4	Initial UE Access

Following a UE connection request, this is initiated by the New RAN to establish the necessary overall initial UE context in the New RAN node in case of an Idle-to Active transition.

Figure 10.2.4-1: Initial UE Access over NG

-	At step 1 following the reception of the first UL NAS message to be forwarded to NGC, the selection of CN entity is performed.

Editor’s note: The details of the selection of the CN entity is pending SA2 decision

-	At step 2, the New RAN uses the INITIAL UE MESSAGE to deliver the NAS PDU to the CN entity selected in the previous step

Editor’s note: It is still FFS whether INITIAL UE MESSAGE is used for this.

-	At step 3, the NGC initiates the UE associated signaling connection setup. The New RAN configures the UE accordingly.

The description above is not entirely accurate because the Initial UE Access procedure is not meant to create a UE context at serving RAN but, by means of the INITIAL UE MESSAGE, it initiates a signaling connection between the serving RAN node and the CN. Indeed, to create a UE context at the RAN node the Initial Context Setup procedure is used.

For the above reasons the text quoted above should be modified to say that, following a UE connection request, the Initial UE Access is initiated by the New RAN to establish a signalling connection between New RAN and NGC in case of an Idle-to Active transition. The first message from the NGC, following the INITIAL UE MESSAGE, completes the establishment of a signalling connection. With the first reply message the NGC may initiate a UE context setup.

# Conclusions

This contribution has explained that the text for the Initial UE Access procedure captured in TR38.801 is not accurate and should be corrected.

In order to correct TR38.801 it is proposed to agree to the TP in R3-170183